| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0323 |
| --- | --- |

| Title: | Non-CE3: Unification of shifting for MIP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Shuai Wan, Yanzhuo Ma, Jinkun Guo, Fuzheng Yang Yuanfang Yu Yang Liu Ming Li Haixin Wang | Email: | jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn yzma@mail.xidian.edu.cn guojinkun@mail.nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com myron.li@oppo.com haixinwang_xd@163.com |
| Source: | Northwestern Polytechnical University, Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This document proposes modification of matrix-based intra prediction (MIP). In current VVC, the weight shift may be equal to 8 or 9. This contribution unifies weight shift into 8 in MIP prediction calculation.

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time comparison over VTM5.0:

For AI configuration: 0.00%, -0.05%, and -0.02%, with 100% EncT, 100% DecT;

For RA configuration: 0.00%, -0.06%, and -0.09%, with 100% EncT, 100% DecT;

Introduction

In VTM5.0, matrix-based intra prediction (MIP) is applied to derive the intra luma predicted samples as：

predMip[ x ][ y ] = ( (i = 02 * boundarySize - 1mWeight i  y*incH*predC+x*incW *p i ) + 
	( vBias[ y * incH * predC + x * incW ]  << sB ) + oW ) >> sW                          (1-1)

The weight shift sW depends on the different MipSizeId and modeId and is provided by a look-up table.

Table 1 sW look-up table

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| 1 | 8 | 8 | 8 | 9 | 8 | 8 | 8 | 8 | 9 | 8 |  |  |  |  |  |  |  |  |
| 2 | 8 | 8 | 8 | 8 | 8 | 8 |  |  |  |  |  |  |  |  |  |  |  |  |

Proposed method

In this contribution, the weight shift is unified to 8 in all MIP modes so that the look-up table for sW can be cancelled.

In this contribution, the intra luma predicted samples are calculated by

predMip[ x ][ y ] = ( (i = 02 * boundarySize - 1mWeight i  y*incH*predC+x*incW *p i ) + 
	( vBias[ y * incH * predC + x * incW ]  << sB ) + oW ) >> 8                           (2-1)

The reduced weight matrix and bias vector are shown in Table 2 to Table 5.

Specifically, for the case where weight shift equals to 9, the weight matrix and bias vector are reduced to half compared to the weighted matrix and bias vector in VVC.

Table 2 the updated weight matrix when MipSizeId=1 and MIP mode is 3 or 12

| 109 | -28 | 4 | -2 | 205 | -38 | 6 | -2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 224 | 47 | -23 | 3 | 11 | -8 | 1 | -2 |
| -1 | 238 | 38 | -17 | -3 | -1 | 0 | -2 |
| 0 | -8 | 233 | 30 | 2 | -2 | 0 | -2 |
| -19 | 1 | -2 | 0 | 130 | 179 | -39 | 4 |
| 61 | -20 | 0 | -1 | 231 | -13 | -6 | -1 |
| 202 | 27 | -21 | 2 | 60 | -17 | 1 | -3 |
| 53 | 201 | 8 | -6 | 1 | -2 | 0 | -3 |
| 2 | -1 | -1 | 0 | -31 | 133 | 187 | -37 |
| -13 | 1 | -2 | -1 | 54 | 233 | -14 | -6 |
| 29 | -13 | -2 | -1 | 209 | 48 | -18 | -2 |
| 150 | 9 | -8 | 2 | 114 | -11 | -2 | -4 |
| -1 | -2 | -1 | -1 | 12 | -48 | 145 | 148 |
| 1 | 0 | -3 | -1 | -15 | 51 | 232 | -15 |
| -8 | 0 | -3 | 0 | 20 | 214 | 35 | -7 |
| 13 | -3 | -1 | 0 | 151 | 96 | -7 | -1 |

Table 3 the updated bias vector when MipSizeId=1 and MIP mode is 3 or 12

| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 1 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4 the updated weight matrix when MipSizeId=1 and MIP mode is 8 or 17

| -46 | 11 | 6 | 4 | 104 | 130 | 36 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -13 | -11 | 7 | 6 | 44 | 153 | 57 | 17 |
| -8 | 9 | -8 | 2 | 15 | 163 | 67 | 19 |
| -6 | 5 | 15 | -10 | 1 | 162 | 71 | 20 |
| 6 | -2 | 0 | 0 | -7 | 136 | 113 | 13 |
| 1 | 5 | -2 | -3 | 3 | 115 | 124 | 17 |
| 0 | 4 | 2 | -5 | 5 | 107 | 131 | 18 |
| -1 | 5 | 2 | -4 | 7 | 104 | 129 | 20 |
| 2 | 0 | 2 | -2 | 3 | 46 | 170 | 39 |
| 2 | 3 | 0 | -3 | 3 | 56 | 164 | 37 |
| 1 | 6 | 1 | -4 | 3 | 61 | 159 | 37 |
| 0 | 6 | 3 | -5 | 4 | 68 | 151 | 37 |
| 3 | 0 | 2 | -3 | 4 | 19 | 108 | 126 |
| 2 | 5 | 0 | -4 | 4 | 29 | 156 | 70 |
| 1 | 6 | 2 | -4 | 5 | 35 | 163 | 57 |
| 1 | 7 | 2 | -4 | 7 | 41 | 157 | 55 |

Table 5 the updated bias vector when MipSizeId=1 and MIP mode is 8 or 17

| -4 | -5 | -6 | -6 | -5 | -7 | -9 | -9 | -6 | -8 | -10 | -11 | -5 | -9 | -11 | -11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Experimental results

Experiments were performed based on VTM 5.0 software [1] under JVET common test condition [2].

Table 2 show experimental results of the proposed method compared to VTM 5.0.

Table 6 Results of the proposed method over VTM5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.03% | 99% | 98% |
| Class A2 | 0.00% | -0.08% | -0.03% | 100% | 101% |
| Class B | 0.01% | -0.07% | -0.02% | 99% | 100% |
| Class C | -0.01% | 0.00% | 0.10% | 100% | 99% |
| Class E | -0.01% | -0.09% | -0.14% | 99% | 101% |
| Overall | 0.00% | -0.05% | -0.02% | 100% | 100% |
| Class D | 0.00% | 0.03% | -0.19% | 100% | 100% |
| Class F (optional) | 0.00% | 0.06% | 0.15% | 101% | 101% |
|  | Random Access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.15% | 0.19% | 100% | 100% |
| Class A2 | 0.03% | 0.05% | -0.03% | 100% | 99% |
| Class B | 0.01% | -0.01% | -0.23% | 100% | 100% |
| Class C | -0.04% | -0.13% | -0.18% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.06% | -0.09% | 100% | 100% |
| Class D | 0.01% | 0.14% | 0.26% | 100% | 100% |
| Class F (optional) | -0.02% | 0.01% | 0.22% | 100% | 100% |

Conclusions

This contribution reports the unification of MIP. The average luma BD-rates and encoding/decoding time are almost the same while MIP calculation is unified. We propose to study this method in CE.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, June 2019.

[3] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, June 2019.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).