| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0324 |
| --- | --- |

| Title: | Non-CE3: MIP Simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Shuai Wan, Yanzhuo Ma, Jinkun Guo, Fuzheng Yang Yuanfang Yu Yang Liu Ming Li | Email: | jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn yzma@mail.xidian.edu.cn guojinkun@mail.nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com myron.li@oppo.com |
| Source: | Northwestern Polytechnical University, Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This document proposes to restrict matrix-based intra prediction (MIP) according to the ratio of width W and height H. Specifically, MIP will only be applied to blocks with W/H<4 or H/W<4. The calculation of MIP can be simplified.

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time comparison over VTM5.0:

- For AI configuration: 0.05%, -0.01%, and 0.00%, with 100% EncT, 100% DecT;

- For RA configuration: 0.05%, 0.00%, and 0.04%, with 99% EncT, 100% DecT;

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time comparison over 8-bit MIP in JVET-O0084:

- For AI configuration: xxx%, xxx%, and xxx%, with xxx% EncT, xxx% DecT;

- For RA configuration: xxx%, xxx%, and xxx%, with xxx% EncT, xxx% DecT.

Introduction

In VTM5.0, matrix-based intra prediction (MIP) is applied to intra luma predicted samples for all blocks with W/H<8 or H/W<8, as Table 1 shows.

Table 1 block restriction of MIP in VTM5.0

| (4x4) | (4x8) | (4x16) | (4x32) | (4x64) |
| --- | --- | --- | --- | --- |
| (8x4) | (8x8) | (8x16) | (8x32) | (8x64) |
| (16x4) | (16x8) | (16x16) | (16x32) | (16x64) |
| (32x4) | (32x8) | (32x16) | (32x32) | (32x64) |
| (64x4) | (64x8) | (64x16) | (64x32) | (64x64) |

There will be four sizes of reduced prediction block in MIP which are determined in Fig.1.

- For current block with max(W,H)≤8, the reduced prediction block with size 4×4 will be generated.

- For the other block:

  - if W=16 and H=4, the size of reduced prediction block will be 8×4;

  - if W=4 and H=16, the size of reduced prediction block will be 4×8;

  - for the others, the reduced prediction block with size 8×8 will be generated.

Fig.1 MIP classification

For blocks with size 16×4 or 4×16, the matrix will leave out every row that corresponds to an odd entry along the horizontal axis of the downsampled block in order to keep the output size as 32.

Proposed method

In this contribution, MIP is applied to intra luma predicted samples for all blocks with W/H<4 or H/W<4 as Table 2 shows.

Table 2 block restriction of MIP in proposed method

| (4x4) | (4x8) | (4x16) | (4x32) | (4x64) |
| --- | --- | --- | --- | --- |
| (8x4) | (8x8) | (8x16) | (8x32) | (8x64) |
| (16x4) | (16x8) | (16x16) | (16x32) | (16x64) |
| (32x4) | (32x8) | (32x16) | (32x32) | (32x64) |
| (64x4) | (64x8) | (64x16) | (64x32) | (64x64) |

The matrix leaving out procedure will not be needed any more since blocks with W=16 and H=4 or W=4 and H=16 will not be applied to MIP. Hence, the MIP prediction calculation progress will be unified and the algorithm will also be pruned as shown in Fig.2. It will also be speeded up because more blocks will not be applied to MIP.

Fig.2 MIP classification in proposed method

Experimental results

The proposed method is integrated into both VTM5.0 [1] and 8bit-MIP [2] and tested under the CTC [3] against VTM-5.0. Results are reported in the following subsections.

Table 4 shows the experimental results of the proposed method compared to VTM 5.0.

Table 4. Results of the proposed method over VTM5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.02% | 0.01% | 99% | 99% |
| Class A2 | 0.05% | -0.06% | -0.04% | 100% | 101% |
| Class B | 0.06% | 0.00% | -0.02% | 100% | 100% |
| Class C | 0.05% | 0.04% | 0.06% | 100% | 101% |
| Class E | 0.04% | -0.01% | 0.01% | 99% | 100% |
| Overall | 0.05% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.05% | 0.17% | 0.00% | 100% | 100% |
| Class F (optional) | 0.02% | 0.02% | 0.08% | 100% | 101% |
|  | Random Access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.13% | 0.23% | 99% | 100% |
| Class A2 | 0.07% | 0.04% | 0.08% | 99% | 100% |
| Class B | 0.05% | 0.03% | -0.06% | 99% | 100% |
| Class C | 0.02% | 0.03% | -0.03% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.00% | 0.04% | 99% | 100% |
| Class D | 0.06% | 0.31% | 0.31% | 99% | 100% |
| Class F (optional) | 0.02% | -0.05% | 0.04% | 99% | 100% |

Table 5 shows the experimental results of the proposed method compared to 8bit-MIP.

Table 5. Results of the proposed method over 8bit-MIP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-O0084 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.03% | -0.08% | 0.02% | 100% | 101% |
| Class E | -0.07% | -0.15% | -0.13% | 98% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | 0.00% | -0.06% | 100% | 100% |
| Class F (optional) | 0.01% | 0.05% | -0.06% | 100% | 100% |
|  | Random Access Main10 |  |  |  |  |
|  | Over JVET-O0084 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | -0.12% | 0.02% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.11% | 0.03% | 0.17% | 99% | 100% |
| Class F (optional) | 0.01% | 0.02% | 0.03% | 100% | 100% |

Conclusions

This contribution reports the simplification of MIP. The average luma BD-rates and encoding/decoding time are almost the same while the calculation and algorithm are simplified. We propose to study this method in CE.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-5.0

- J. Pfaff, T. Hinz, P. Helle, P. Mekle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand, “8-bit implementation and simplification of MIP”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O0084, Gothenburg, SE, 3–12 July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, June 2019.

[3] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, June 2019.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).