| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0325 |
| --- | --- |

| Title: | CE8-related: Simple BVD coding scheme without adding more context |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yucheng Sun Fangdong Chen Li Wang | Tel: Email: | +86 18105755698 sunyucheng@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution proposes a simple BVD coding scheme reusing the context of MVD coding. In method 1, zero BVD is disabled because of the redundancy caused by ABVP list construction modification adopted in last meeting, which reuse the merge list for ABVP mode. In method 2, contains method 1 and the binarization process [1] for abs_mvd_minus2 syntax, which is coded by Exp-Golomb order K equal to 1 (EG1), is changed to use EG3. From experiment results, following BD rate changes are observed over VTM-5.0 with IBC:

Method 1,(won’t parse abs_mvd_greater0_flag[0] when abs_mvd_greater0_flag[1] = 0)

- AI: -0.63% (CTC), -0.12% (Class F), -0.43% (TGM)

- RA: 0.01% (CTC), -0.12% (Class F), -0.18% (TGM)

Method 2,(method 1 + EG3 coding for abs_mvd_minus2)

- AI: -0.64% (CTC), -0.26% (Class F), -0.74% (TGM)

- RA: 0.02% (CTC), -0.20% (Class F), -0.41% (TGM)

Proposed method

This contribution proposes two coding methods of BVD when current coding block is coded by IBC mode.

Method 1: remove zero BVD

Method 1 disabled zero BVD in VTM5.0. In last meeting, ABVP candidate will take the first two merge candidate, which will cause redundancy when the encoder choose ABVP mode with zero BVD which just the same meaning when choose the first two candidates in IBC merge mode. Because of the multiple reference frames this problems isn’t so obvious in regular inter prediction, which means in regular inter mode the MVP usually different from the merge candidates. When disabled zero BVD, the situation that both abs_mvd_greater0_flag[0] syntax and abs_mvd_greater0_flag[1] equal to 0 won’t happen. Two optional modification can be applied:

Original parsing process:

abs_mvd_greater0_flag[ 0 ]

abs_mvd_greater0_flag[ 1 ]

Optional modification 1:

abs_mvd_greater0_flag[ 0 ]

if( !abs_mvd_greater0_flag[ 0 ] )

abs_mvd_greater0_flag[ 1 ]

Optional modification 2:

abs_mvd_greater0_flag[ 1 ]

if( !abs_mvd_greater0_flag[ 1 ] )

abs_mvd_greater0_flag[ 0 ]

After analysis current SCC sequence, modification 2 is better than modification 1. Because the distribution of BVD has more zero value in y-axis than x-axis, which means more abs_mvd_greater0_flag[0] can skip parsing process under modification 2.

Method 2: method 1 + binarization modification

Method 2 contains method 1 and binarization process modification for abs_mvd_minus2 syntax. When prediction mode of current block is IBC mode, binarization process for abs_mvd_minus2 syntax is signaled by Exp-Golomb order K equal to 3. However, when prediction mode of current block is inter mode, binarization process for abs_mvd_minus2 syntax is signaled by Exp-Golomb order K equal to 1 as same as anchor.

Draft modification

|  | Descriptor |
| --- | --- |
| if( general_merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| bvd_coding( x0, y0, 0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_amvr_enabled_flag && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ) ) { |  |
| amvr_precision_flag[ x0 ][ y0 ] | ae(v) |
| } |  |

|  | Descriptor |
| --- | --- |
| bvd_coding( x0, y0, refList ,cpIdx ) { |  |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( !abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

When abs_mvd_greater0_flag[ compIdx ] is not present, it is inferred to be equal to 1.

Experimental Results

The proposed methods have been implemented on top of VTM-5.0 and tested under CE8 test conditions [2].

Table 1. Performance of method 1 in AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.04% | -0.07% | 0.09% | 139% | 98% |
| Class A2 | -0.78% | -1.06% | -0.95% | 144% | 98% |
| Class B | -0.78% | -0.69% | -0.86% | 153% | 98% |
| Class C | -0.28% | -0.29% | -0.25% | 155% | 103% |
| Class E | -1.27% | -1.23% | -1.33% | 158% | 102% |
| Overall | -0.63% | -0.65% | -0.66% | 150% | 100% |
| Class D | -0.55% | -0.46% | -0.62% | 148% | 101% |
| Class F | -0.12% | -0.11% | -0.08% | 99% | 98% |
| Class SCC | -0.43% | -0.38% | -0.41% | 99% | 98% |

Table 2. Performance of method 1 in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.11% | -0.11% | 0.22% | 104% | 99% |
| Class A2 | -0.06% | -0.38% | -0.11% | 104% | 99% |
| Class B | -0.14% | -0.35% | -0.52% | 107% | 99% |
| Class C | 0.17% | -0.18% | -0.10% | 112% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.26% | -0.18% | 107% | 100% |
| Class D | 0.14% | -0.09% | 0.11% | 112% | 101% |
| Class F | -0.12% | -0.05% | 0.06% | 100% | 99% |
| Class SCC | -0.18% | -0.20% | -0.28% | 100% | 99% |

Table 3. Performance of test method 1 in LD

|  | Y | U |  | V |  | EncT |  | DecT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.45% |  | -0.16% |  | 117% |  | 97% |  |
| Class C | 0.10% | 0.36% |  | -0.38% |  | 125% |  | 96% |  |
| Class E | 0.15% | -0.71% |  | 0.46% |  | 113% |  | 98% |  |
| Overall | 0.08% | 0.13% |  | -0.08% |  | 119% |  | 97% |  |
| Class D | 0.07% | 0.49% |  | 0.92% |  | 126% |  | 101% |  |
| Class F | -0.08% | -0.29% |  | 0.42% |  | 101% |  | 101% |  |
| Class SCC | -0.14% | -0.16% |  | -0.13% |  | 101% |  | 101% |  |

Table 4. Performance of method 2 in AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.04% | -0.01% | 0.01% | 140% | 99% |
| Class A2 | -0.79% | -1.06% | -1.00% | 144% | 98% |
| Class B | -0.78% | -0.67% | -0.89% | 155% | 99% |
| Class C | -0.30% | -0.24% | -0.19% | 156% | 104% |
| Class E | -1.30% | -1.26% | -1.35% | 159% | 104% |
| Overall | -0.64% | -0.63% | -0.68% | 151% | 101% |
| Class D | -0.56% | -0.50% | -0.54% | 149% | 106% |
| Class F | -0.26% | -0.32% | -0.37% | 100% | 100% |
| Class SCC | -0.74% | -0.70% | -0.71% | 100% | 99% |

Table 5. Performance of method 2 in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.10% | -0.20% | 0.16% | 104% | 98% |
| Class A2 | -0.05% | -0.30% | -0.07% | 104% | 98% |
| Class B | -0.12% | -0.28% | -0.75% | 107% | 99% |
| Class C | 0.18% | -0.22% | -0.25% | 112% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.25% | -0.30% | 107% | 100% |
| Class D | 0.10% | -0.20% | -0.17% | 112% | 101% |
| Class F | -0.20% | -0.12% | -0.15% | 100% | 100% |
| Class SCC | -0.41% | -0.37% | -0.40% | 100% | 100% |

Table 6. Performance of method 2 in LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.26% | -0.23% | 117% | 94% |
| Class C | 0.13% | 0.04% | 0.06% | 125% | 93% |
| Class E | 0.09% | -0.10% | 0.86% | 113% | 96% |
| Overall | 0.08% | 0.10% | 0.14% | 119% | 94% |
| Class D | 0.08% | -0.09% | 0.58% | 126% | 100% |
| Class F | -0.06% | 0.16% | 0.75% | 101% | 100% |
| Class SCC 1080p | -0.26% | -0.13% | -0.18% | 101% | 99% |

Conclusion

In this contribution, two method of block vector coding for IBC mode are tested. It is recommended to adopt the proposed BVD coding for IBC mode.

Reference

- Junghak Nam, Jaehyun Lim, “CE8-1.2: Block vector coding for IBC,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N0458-v2, 14th Meeting: Geneva, CH, 19–27 March 2019.

- Xiaozhong Xu, Yung-Hsuan Chao, Yu-Chen Sun, Jizheng Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” , Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1028-v4, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).