| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0327-v2 |
| --- | --- |

| Title: | CE8-related: Default filling method for IBC Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurui Ye, Yucheng Sun, Fangdong Chen, Li Wang | Tel: Email: | +86 159-9011-0726 yeshurui@hikvision.com sunyucheng@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution presents a default filling method for IBC mode in order to replace invalid zero block vectors. The reported BD rate changes from the VTM5.0 with IBC enable are as follows:

Method 1(Filling the merge list with (-2*cbWidth, 0) block vector):

- 0.05%/0.06%/0.11% gain in AI/RA/LDB configuration for class TGM, separately.

Method 2(Filling the merge list with a default block vector list):

- 0.09%/0.16%/0.17% gain in AI/RA/LDB configuration for class TGM, separately.

Introduction

In VTM5.0, if the number of available IBC merge candidates is less than 6, default zero block vectors are added until the merge list is fully filled. However, zero block vectors are invalid for IBC mode.

Technical description

Method 1

If the merge list is still not complete, the default block vector of (-2*cbWidth, 0) is used to fill the merge list, where, cbWidth is the coding block width in luma samples, and block vector in integer pixel accuracy.

Hence, the text is modified as follows.

- When numCurrCand is less than MaxNumMergeCand, the following applies until numCurrCand is equal to MaxNumMergeCand:

  - mvCandList[ numCurrCand ][ 0 ] is set equal to 0(-2 * cbWidth << 4).

  - mvCandList[ numCurrCand ][ 1 ] is set equal to 0.

  - numCurrCand is increased by 1.

Method 2

The default block vectors are added in the order of: (-2*cbWidth, 0), (0, -cbHeight), (-4*cbWidth, 0), (0, -2*cbHeight), (-8*cbWidth, 0), (0, -4*cbHeight), where cbWidth and cbHeight are the coding block width and height in luma samples, respectively.

Hence, the text is modified as follows.

- When numCurrCand is less than MaxNumMergeCand, for each candidate in DefaultCandList[DefaultIdx] with index = 1…MaxNumMergeCand, the following applies until numCurrCand is equal to MaxNumMergeCand:

  - mvCandList[ numCurrCand ][ 0 ] is set equal to 0 DefaultCandList[DefaultIdx++][0].

  - mvCandList[ numCurrCand ][ 1 ] is set equal to 0 DefaultCandList[DefaultIdx++][1].

  - numCurrCand is increased by 1.

- Table 8-15 –Default candidates

| DefaultIdx | DefaultCandList[DefaultIdx][0] | DefaultCandList[DefaultIdx][1] |
| --- | --- | --- |
| 1 | -2 * cbWidth << 4 | 0 |
| 2 | 0 | -1 * cbHeight << 4 |
| 3 | -4 * cbWidth << 4 | 0 |
| 4 | 0 | -2 * cbHeight << 4 |
| 5 | -8 * cbWidth << 4 | 0 |
| 6 | 0 | -4 * cbHeight << 4 |

- When numCurrCand is less than MaxNumMergeCand, the following applies until numCurrCand is equal to MaxNumMergeCand:

  - mvCandList[ numCurrCand ][ 0 ] is set equal to 0(-2 * cbWidth << 4) , 0, (-4 * cbWidth << 4), 0, (-8 * cbWidth << 4), in the order.

  - mvCandList[ numCurrCand ][ 1 ] is set equal to 0, (-cbHidth << 4), 0, (-2*cbHidth << 4), 0, (-4*cbHidth << 4) , in the order.

  - numCurrCand is increased by 1.

Simulation results

The simulation is based on VTM5.0 software. The anchor is generated by using CE8 test conditions [1]. Results are shown in the following tables:

Method 1: Filling the merge list with (-2*cbWidth, 0) block vector

Table 1. Filling the merge list with (-2*cbWidth, 0) block vector

|  | Over VTM5.0 (AI configuration) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.03% | -0.01% | 154% | 100% |
| Class A2 | -0.78% | -1.05% | -0.98% | 154% | 100% |
| Class B | -0.78% | -0.71% | -0.83% | 166% | 101% |
| Class C | -0.29% | -0.28% | -0.25% | 166% | 104% |
| Class E | -1.30% | -1.18% | -1.35% | 174% | 103% |
| Overall | -0.63% | -0.63% | -0.67% | 163% | 102% |
| Class D | -0.52% | -0.49% | -0.57% | 159% | 107% |
| Class F | -0.04% | 0.03% | -0.02% | 103% | 100% |
| TGM | -0.05% | -0.03% | -0.03% | 101% | 101% |
|  | Over VTM5.0 (RA configuration) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.05% | 0.11% | 111% | 100% |
| Class A2 | -0.06% | -0.13% | -0.01% | 111% | 100% |
| Class B | -0.15% | -0.24% | -0.66% | 114% | 100% |
| Class C | 0.10% | -0.35% | -0.11% | 122% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.21% | -0.23% | 115% | 100% |
| Class D | 0.12% | -0.36% | 0.06% | 122% | 102% |
| Class F | -0.09% | -0.18% | 0.11% | 106% | 100% |
| TGM | -0.06% | -0.03% | -0.08% | 103% | 100% |
|  | Over VTM5.0 (LDB configuration) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.31% | 0.18% | 131% | 100% |
| Class C | 0.14% | 0.29% | 0.06% | 142% | 99% |
| Class E | 0.14% | 0.35% | 0.11% | 123% | 100% |
| Overall | 0.08% | 0.31% | 0.12% | 133% | 100% |
| Class D | 0.07% | -0.27% | 0.76% | 142% | 101% |
| Class F | -0.17% | -0.14% | -0.56% | 111% | 103% |
| TGM | -0.11% | -0.17% | -0.12% | 106% | 107% |

Method 2: Filling the merge list with a default block vector list

Table 2. Filling the merge list with a default block vector list

|  | Over VTM5.0 (AI configuration) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.06% | -0.04% | 147% | 100% |
| Class A2 | -0.80% | -1.01% | -0.99% | 150% | 99% |
| Class B | -0.80% | -0.68% | -0.86% | 161% | 101% |
| Class C | -0.31% | -0.27% | -0.24% | 162% | 105% |
| Class E | -1.30% | -1.27% | -1.28% | 166% | 104% |
| Overall | -0.65% | -0.62% | -0.68% | 158% | 102% |
| Class D | -0.58% | -0.49% | -0.64% | 154% | 105% |
| Class F | -0.07% | 0.05% | -0.03% | 102% | 101% |
| TGM | -0.09% | -0.05% | -0.05% | 101% | 100% |
|  | Over VTM5.0 (RA configuration) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.17% | 0.07% | 108% | 100% |
| Class A2 | -0.08% | -0.22% | -0.12% | 108% | 100% |
| Class B | -0.16% | -0.30% | -0.71% | 111% | 100% |
| Class C | 0.10% | -0.21% | -0.24% | 118% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.24% | -0.31% | 112% | 101% |
| Class D | 0.00% | -0.29% | -0.04% | 118% | 102% |
| Class F | -0.13% | -0.16% | -0.13% | 104% | 100% |
| TGM | -0.16% | -0.21% | -0.27% | 102% | 100% |
|  | Over VTM5.0 (LDB configuration) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.02% | -0.21% | 125% | 100% |
| Class C | 0.08% | 0.16% | -0.12% | 133% | 100% |
| Class E | -0.01% | 0.38% | 0.28% | 118% | 99% |
| Overall | 0.02% | 0.16% | -0.06% | 126% | 100% |
| Class D | 0.12% | -0.25% | -0.16% | 135% | 102% |
| Class F | -0.22% | -0.40% | -0.53% | 106% | 102% |
| TGM | -0.17% | -0.30% | -0.27% | 104% | 107% |

Conclusions

This contribution presents a default filling method for IBC mode in order to replace invalid zero block vectors. “Method1: Filling the merge list with (-2*cbWidth, 0) block vector” shows 0.05% gain in AI configuration, 0.06% gain in RA configuration and 0.11% gain in LB configuration in screen content coding materials. “Method2: Filling the merge list with a default block vector list” shows 0.09% gain in AI configuration, 0.16% gain in RA configuration and 0.17% gain in LB configuration.

References

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, JVET-N1028_v4, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, Geneva, March 2019.

Patent rights declaration(s)

Hikvision may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).