| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0329 |
| --- | --- |

| Title: | Non-CE4: Simplification of motion vector storage process for triangle merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee, Sung-Chang Lim, Jinho Lee, Jungwon Kang, Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-6138 {hanilee, sclim, jinosoul, jungwon, hykim5}@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents simplified methods on motion vector storage process for triangle merge mode. In method 1, when a 4x4 subblock is type-2 and two triangular partitions have motion information in the same reference list, the reference picture mapping process is removed. Instead, one of two triangular partition motion information is assigned based on triangular split direction information. In method 2, type- 2 is removed so that a 4x4 subblock can be either type-0 or type-1. Compared with VTM-5.0, the method 1 incurs an average 0.00% and 0.06% luma BD-rate changes for RA and LDB, respectively. The method 2 incurs an average 0.XX% and 0.XX% luma BD-rate changes for RA and LDB, respectively.

Introduction

In VVC WD 5 [1], triangular prediction can be used to enable triangular shape prediction units for inter prediction. If triangular mode is selected, it is split diagonally and each resulting triangular partition (A or B) is predicted using each own uni-directional motion information. motion information means motion vector and reference index. Besides, two motion information corresponding to each triangular partition are populated to create 4x4 motion information that are used for spatial and temporal neighbouring motion information for Merge/AMVP.

More specifically, for each 4x4 subblock, type value, 0, 1 or 2, is derived as shown in Figure 1. If type value is equal to either 0 or 1, one of uni-directional triangular partition motion information is assigned to 4x4 subblock. If type value is equal to 2, a bi-predictive motion information could be assigned to 4x4 subblock and type 2 motion information is derived as follows.

- If two triangular partitions have different direction motion information, for example, when A has L0 motion information and B has L1 motion information, the bi-predictive motion information is derived directly by combining two different direction motion information.

- Otherwise (two triangular partitions have same direction motion information), for example, the case that partition A has L0 motion information and partition B has L0 motion information and vice versa. the bi-predictive motion information is derived through the reference picture mapping process for searching a pair reference picture having same POC in opposite reference picture list. If both of two triangular partitions do not have their pair reference picture in opposite reference picture list, the uni-directional motion information of a partition A is assigned to 4x4 subblock.

| 2; 2; 0; 0; 0; 0; 0; 0. 1; 1; 2; 2; 0; 0; 0; 0. 1; 1; 1; 1; 2; 2; 0; 0. 1; 1; 1; 1; 1; 1; 2; 2. | 0; 0; 0; 0; 0; 0; 2; 2. 0; 0; 0; 0; 2; 2; 1; 1. 0; 0; 2; 2; 1; 1; 1; 1. 2; 2; 1; 1; 1; 1; 1; 1. |
| --- | --- |

(a) triangleDir = 0                           (b) triangleDir =1

Figure 1. 4x4 subblock motion type (CU: 32x16)

Proposed Methods

Proposed Method 1

When a 4x4 subblock is type-2 and two triangular partitions have same direction motion information, for example, the case that partition A has L0 motion information and partition B has L0 motion information and vice versa, the reference picture mapping process for searching a pair reference picture having same POC in opposite reference picture list is removed. Instead, one of two triangular partition motion information is assigned based on triangular split direction information as below

- if triangular split direction, triangleDir, is equal to 0,

  - MVA and RefIdxA is assigned to 4x4 subblock

- otherwise (triangular split direction, triangleDir, is equal to 1),

  - MVB and RefIdxB is assigned to 4x4 subblock

Proposed Method 2

In method 2, a type-2 is removed so that a 4x4 subblock can be either type-0 or type-1 as shown in Figure 2. The type variable is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx >= yIdx )  ?  0  :  1

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx < minSb )  ?  0  :  1

| 0; 0; 0; 0; 0; 0; 0; 0. 1; 1; 0; 0; 0; 0; 0; 0. 1; 1; 1; 1; 0; 0; 0; 0. 1; 1; 1; 1; 1; 1; 0; 0. | 0; 0; 0; 0; 0; 0; 1; 1. 0; 0; 0; 0; 1; 1; 1; 1. 0; 0; 1; 1; 1; 1; 1; 1. 1; 1; 1; 1; 1; 1; 1; 1. |
| --- | --- |

(a) triangleDir = 0                          (b) triangleDir =1

Figure 2. 4x4 subblock motion type in method 2 (CU: 32x16)

Experimental Results

The proposed methods were implemented on VTM-5.0 software [2]. Simulations were performed following the JVET CTC [3]. The experiments were conducted on the platform of Intel Xeon E5-2690 v2 (3.0GHz) and 64GB RAM with GCC 7.3.1 compiler under CentOS Linux.

The experimental results compared to VTM-5.0 anchor are summarized in Table 1 and Table 2. More detailed results can be found in the attached spreadsheets.

Table 1. Results of proposed method 1 compared to VTM-5.0

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.08% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.02% | 0.04% | -0.02% | 100% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.13% | -0.10% | 100% | 100% | 0.07% | 0.21% | -0.03% | 100% | 100% |
| Class C | -0.02% | -0.14% | -0.10% | 101% | 100% | 0.06% | 0.10% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  | 0.02% | 0.14% | -0.43% | 100% | 100% |
| Overall | 0.00% | 0.01% | -0.05% | 100% | 100% | 0.06% | 0.16% | -0.10% | 100% | 100% |
| Class D | 0.01% | 0.18% | 0.15% | 100% | 99% | 0.01% | 0.00% | 0.43% | 100% | 100% |
| Class F | -0.01% | 0.03% | 0.00% | 100% | 100% | 0.05% | 0.31% | -0.05% | 100% | 99% |

Table 2. Results of proposed method 2 compared to VTM-5.0

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.01% | -0.13% | -0.15% | 100% | 100% | 0.16% | 0.12% | 0.21% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.05% | -0.13% | 0.35% | 100% | 99% | 0.16% | -0.21% | 1.03% | 100% | 100% |
| Class F | -0.01% | 0.05% | 0.01% | 100% | 100% | 0.01% | 0.04% | -0.22% | 100% | 100% |

Conclusions

In method 1, when a 4x4 subblock is type 2 and two triangular partitions have same directional motion information, the reference picture mapping process for searching a pair reference picture having same POC in opposite reference picture list is removed. Instead, one of two triangular partition motion information is assigned based on triangular split direction information. In method 2, type-2 is removed so that a 4x4 subblock can have either type-0 or type-1. Simulation results show minor coding performance impact with the proposed methods. It is recommended to consider adoption of either method 1 or method 2 into VVC WD6 and VTM-6.0.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: VVC WD5 Changes

The following shows the proposed draft text changes based on VVC WD5 v8 when using the proposed methods:

Method 1

Motion vector storing process for triangle merge mode

…

The variable minSb is set equal to min( numSbX, numSbY ) – 1.

The variable cbRatio is derived as follows:

cbRatio = ( cbWidth > cbHeight ) ? ( cbWidth / cbHeight ) : ( cbHeight / cbWidth )			(8-814)

The variable refIdxTempA is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with X set equal to predListFlagA and refIdxN set equal to refIdxA as inputs.

The variable refIdxTempB is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with the X set equal to predListFlagB and refIdxN set equal to refIdxB as inputs.

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xIdx and yIdx are derived as follows:

xIdx = ( cbWidth > cbHeight )  ?  ( xSbIdx / cbRatio ) : xSbIdx	(8-815)

yIdx = ( cbWidth > cbHeight )  ?  ySbIdx  :  ( ySbIdx / cbRatio )	(8-816)

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-817)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-818)

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0	(8-819)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1	(8-820)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(8-821)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(8-822)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(8-823)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(8-824)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(8-825)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(8-826)

- Otherwise, if sType is equal to 1, the following applies:

predFlagL0 = ( predListFlagB = = 0 )  ?  1  :  0	(8-827)

predFlagL1 = ( predListFlagB = = 0 )  ?  0  :  1	(8-828)

refIdxL0 = ( predListFlagB = = 0 )  ?  refIdxB : −1	(8-829)

refIdxL1 = ( predListFlagB = = 0 )  ?  −1  :  refIdxB	(8-830)

mvL0[ 0 ] = ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0	(8-831)

mvL0[ 1 ] = ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0	(8-832)

mvL1[ 0 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]	(8-833)

mvL1[ 1 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]	(8-834)

- Otherwise (sType is equal to 2), the following applies:

  - If predListFlagA + predListFlagB is equal to 1,

predFlagL0 = 1		(8-835)

predFlagL1 = 1		(8-836)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA  :  refIdxB	(8-837)

refIdxL1 = ( predListFlagA = = 0 )  ?  refIdxB  :  refIdxA	(8-838)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  mvB[ 0 ]	(8-839)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  mvB[ 1 ]	(8-840)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  mvB[ 0 ]  :  mvA[ 0 ]	(8-841)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  mvB[ 1 ]  :  mvA[ 1 ]	(8-842)

  - If predListFlagA + predListFlagB is equal to 0, the following applies:

predFlagL0 = 1		(8-843)

predFlagL1 = 0

refIdxL0 = triangleDir == 0 ? refIdxA : refIdxB

refIdxL1 = −1

mvL0[0] = triangleDir == 0 ? mvA[0] : mvB[0]

mvL0[1] = triangleDir == 0 ? mvA[1] : mvB[1]

mvL1[0] = 0

mvL1[1] = 0

predFlagL1 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-844)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-845)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-846)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-847)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-848)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-849)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-850)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

  - If predListFlagA + predListFlagB is equal to 2, the following applies:

predFlagL0 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-851)

predFlagL0 = 0

predFlagL1 = 1		(8-852)

refIdxL0 = −1

refIdxL1 = triangleDir == 0 ? refIdxA : refIdxB

mvL0[0] = 0

mvL0[1] = 0

mvL1[0] = triangleDir == 0 ? mvA[0] : mvB[0]

mvL1[1] = triangleDir == 0 ? mvA[1] : mvB[1]

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-853)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-854)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-855)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-856)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-857)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-858)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

….

Reference picture mapping process for triangle merge mode

Input to this process are:

- a variable X representing a reference list being equal to 0 or 1,

- a reference index refIdxN.

Output of this process is:

- a reference index refIdxTemp.

The variable refPicPoc is derived as follows:

refPicPoc = ( X = = 0 )  ?  RefPicList[ 0 ][ refIdxN ]  :  RefPicList[ 1 ][ refIdxN ]	(8-865)

The reference picture list refPicListTemp is derived as follows:

refPicListTemp = ( X = = 0 )  ?  RefPicList[ 1 ]  :  RefPicList[ 0 ]	(8-866)

The variable refIdxTemp is derived as follows:

- The variable mapStop is set equal to FALSE.

- For the variable refIdxm with m = 0..NumRefIdxActive[ 1 ] − 1, the following applies until mapStop is equal to FALSE:

refIdxTemp = ( refPicListTemp[ refIdxm ] = = refPicPoc )  ?  refIdxm  :  −1	(8-867)

mapStop = ( refIdxTemp != −1 )  ?  TRUE  :  FALSE	(8-868)

Method 2

Motion vector storing process for triangle merge mode

…

The variable minSb is set equal to min( numSbX, numSbY ) – 1.

The variable cbRatio is derived as follows:

cbRatio = ( cbWidth > cbHeight ) ? ( cbWidth / cbHeight ) : ( cbHeight / cbWidth )			(8-814)

The variable refIdxTempA is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with X set equal to predListFlagA and refIdxN set equal to refIdxA as inputs.

The variable refIdxTempB is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with the X set equal to predListFlagB and refIdxN set equal to refIdxB as inputs.

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xIdx and yIdx are derived as follows:

xIdx = ( cbWidth > cbHeight )  ?  ( xSbIdx / cbRatio ) : xSbIdx	(8-815)

yIdx = ( cbWidth > cbHeight )  ?  ySbIdx  :  ( ySbIdx / cbRatio )	(8-816)

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx >= yIdx )  ?  0  :  1 )	(8-817)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-818)

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0	(8-819)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1	(8-820)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(8-821)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(8-822)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(8-823)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(8-824)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(8-825)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(8-826)

- Otherwise, if sType is equal to 1, the following applies:

predFlagL0 = ( predListFlagB = = 0 )  ?  1  :  0	(8-827)

predFlagL1 = ( predListFlagB = = 0 )  ?  0  :  1	(8-828)

refIdxL0 = ( predListFlagB = = 0 )  ?  refIdxB : −1	(8-829)

refIdxL1 = ( predListFlagB = = 0 )  ?  −1  :  refIdxB	(8-830)

mvL0[ 0 ] = ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0	(8-831)

mvL0[ 1 ] = ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0	(8-832)

mvL1[ 0 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]	(8-833)

mvL1[ 1 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]	(8-834)

- Otherwise (sType is equal to 2), the following applies:

  - If predListFlagA + predListFlagB is equal to 1,

predFlagL0 = 1		(8-835)

predFlagL1 = 1		(8-836)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA  :  refIdxB	(8-837)

refIdxL1 = ( predListFlagA = = 0 )  ?  refIdxB  :  refIdxA	(8-838)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  mvB[ 0 ]	(8-839)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  mvB[ 1 ]	(8-840)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  mvB[ 0 ]  :  mvA[ 0 ]	(8-841)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  mvB[ 1 ]  :  mvA[ 1 ]	(8-842)

  - If predListFlagA + predListFlagB is equal to 0, the following applies:

predFlagL0 = 1		(8-843)

predFlagL1 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-844)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-845)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-846)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-847)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-848)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-849)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-850)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

  - If predListFlagA + predListFlagB is equal to 2, the following applies:

predFlagL0 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-851)

predFlagL1 = 1		(8-852)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-853)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-854)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-855)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-856)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-857)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-858)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

….

Reference picture mapping process for triangle merge mode

Input to this process are:

- a variable X representing a reference list being equal to 0 or 1,

- a reference index refIdxN.

Output of this process is:

- a reference index refIdxTemp.

The variable refPicPoc is derived as follows:

refPicPoc = ( X = = 0 )  ?  RefPicList[ 0 ][ refIdxN ]  :  RefPicList[ 1 ][ refIdxN ]	(8-865)

The reference picture list refPicListTemp is derived as follows:

refPicListTemp = ( X = = 0 )  ?  RefPicList[ 1 ]  :  RefPicList[ 0 ]	(8-866)

The variable refIdxTemp is derived as follows:

- The variable mapStop is set equal to FALSE.

- For the variable refIdxm with m = 0..NumRefIdxActive[ 1 ] − 1, the following applies until mapStop is equal to FALSE:

refIdxTemp = ( refPicListTemp[ refIdxm ] = = refPicPoc )  ?  refIdxm  :  −1	(8-867)

mapStop = ( refIdxTemp != −1 )  ?  TRUE  :  FALSE	(8-868)