| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0331 |
| --- | --- |

| Title: | Non-CE8: On IBC BVP Candidate List Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee, Sung-Chang Lim, Jinho Lee, Jungwon Kang, Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-6138 {hanilee, sclim, jinosoul, jungwon, hykim5}@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

In this contribution, it was proposed to fix an issue in IBC BVP candidate list derivation. The issue is that when the maximum number of merge candidates is set to 1, the second candidate could be unavailable in AMPV mode. The proposed changes have no impact under CTC test conditions (maximum number of merge candidates > 1). Compared to VTM-5.0 with IBC and MaxNumMergeCand = 1, the proposed method 1 reportedly shows that there are no BD-rate changes. The proposed method 2 reportedly shows that average luma BD-rate change is -0.01%, -0.02% and -0.05% for AI, RA and LDB, respectively. For class F, it was observed that average luma BD-rate change is -0.32%, -0.20% and -0.10% for AI, RA and LDB, respectively. For TGM sequences, it was observed that average luma BD-rate change is -0.91%, -0.46% and -0.20% for AI, RA and LDB, respectively.

Introduction

In VVC WD 5 [1], the block vector predictors for merge mode and AMVP mode in IBC share a common predictor list, which consist of the following elements:

- 1) 2 spatial neighboring position (A1, B1)

- 2) 5 HMVP candidates (when numCurrCand is less than MaxNumMergeCand)

- 3) Default zero vectors (when numCurrCand is less than MaxNumMergeCand)

For merge mode, up to MaxNumMergeCand entries of this list will be used. for AMVP mode, the first 2 entries of this list will be used.

Problem Statements

In the common test conditions, the current block vector predictor (BVP) list construction design does not cause any problems. However, if maximum number of merge candidate, MaxNumMergeCand, is set to 1, the current BVP list construction can cause a problem in AMVP mode. More specifically, when MaxNumMergeCand is set equal to 1, because only one candidate can be derived from the BVP list, the problem can occur in AMPV mode which requires two candidates. For example, when MaxNumMereCand is set to 1 and either A1 or B1 spatial candidate is unavailable, the number of candidate derived from the BVP candidate list is 1.

In the current VTM-5.0, when the number of candidate derived from the BVP candidate list is equal to 1, the zero vector is inadvertently assigned as a second candidate because the variable, highlighted in Figure 1, has been initialized to (0, 0). For the above reason, when MaxNumMergeCand is set to 1, the problem does not occur in the VTM-5.0 software. However, the current draft text does not define the second candidate of AMVP mode in this situation.

void PU:: fillIBCMvpCand() { MergeCtx mergeCtx; PU::getIBCMergeCandidates(pu, mergeCtx, AMVP_MAX_NUM_CANDS - 1); int candIdx = 0; while (pInfo->numCand < AMVP_MAX_NUM_CANDS) { pInfo->mvCand[pInfo->numCand] = mergeCtx.mvFieldNeighbours[(candIdx << 1) + 0].mv;; pInfo->numCand++; candIdx++; } }

Figure 1. IBC BVP list construction in AMVP mode (VTM-5.0)

Proposed Methods

In this contribution, two methods are proposed to fix the above problem which might be occurred in the non-CTC.

Proposed Method 1

For AMVP mode, it is proposed to assign zero vector as a second candidate when the number of candidates derived from the BVP candidate list is one and the second candidate is not available.

Proposed Method 2

In the current draft text, since the maximum number of BVP candidate is defined as the maximum number of merge candidates, MaxNumMergeCand, in both merge and AMVP mode, this issue is occurred.

Therefore, it is proposed to define separately the maximum number of BVP candidates (e.g., MaxNumBVPCand) for merge and AMPV mode as follows.

- IBC Merge(SKIP) Mode: MaxNumBVPCand = MaxNumMergeCand

- IBC AMVP Mode: MaxNumBVPCand = 2

These proposed changes have no impact under CTC test conditions

Working Draft Text Changes

Proposed Method 1

8.6.2.2	Derivation process for IBC luma motion vector prediction

…

The luma motion vector mvL is derived by the following ordered steps:

- The derivation process for spatial motion vector candidates from neighbouring coding units as specified in clause 8.6.2.3 is invoked with the luma coding block location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, and the luma coding block height cbHeight set equal to smrWidth and smrHeight as inputs, and the outputs being the availability flags availableFlagA1, availableFlagB1 and the motion vectors mvA1 and mvB1.

- The motion vector candidate list, mvCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mvCandList [ i++ ] = mvA1		(8-902)
if( availableFlagB1 )
	mvCandList [ i++ ] = mvB1

- The variable numCurrCand is set equal to the number of merging candidates in the mvCandList.

- When numCurrCand is less than MaxNumMergeCand and smrNumHmvpIbcCand is greater than 0, the derivation process of IBC history-based motion vector candidates as specified in 8.6.2.4 is invoked with mvCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], and numCurrCand as inputs, and modified mvCandList and numCurrCand as outputs.

- When numCurrCand is less than MaxNumMergeCand, the following applies until numCurrCand is equal to MaxNumMergeCand:

    - mvCandList[ numCurrCand ][ 0 ] is set equal to 0.

    - mvCandList[ numCurrCand ][ 1 ] is set equal to 0.

    - numCurrCand is increased by 1.

- When general_merge_flag[ xCb ][ yCb ] is equal to 0 and numCurrCand is less than 2, the the following applies until numCurrCand is equal to 2:

    - mvCandList[ numCurrCand ][ 0 ] is set equal to 0.

    - mvCandList[ numCurrCand ][ 1 ] is set equal to 0.

    - numCurrCand is increased by 1.

- The variable mvIdx is derived as follows:

mvIdx = general_merge_flag[ xCb ][ yCb ] ? merge_idx[ xCb ][ yCb ] : mvp_l0_flag[ xCb ][ yCb ]	(8-903)

- The following assignments are made:

mvL[ 0 ] = mvCandList[ mvIdx ][ 0 ]		(8-904)

mvL[ 1 ] = mvCandList[ mvIdx ][ 1 ]		(8-905)

Proposed Method 2

8.6.2.2	Derivation process for IBC luma motion vector prediction

…

The luma motion vector mvL is derived by the following ordered steps:

- The derivation process for spatial motion vector candidates from neighbouring coding units as specified in clause 8.6.2.3 is invoked with the luma coding block location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, and the luma coding block height cbHeight set equal to smrWidth and smrHeight as inputs, and the outputs being the availability flags availableFlagA1, availableFlagB1 and the motion vectors mvA1 and mvB1.

- The motion vector candidate list, mvCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mvCandList [ i++ ] = mvA1		(8-902)
if( availableFlagB1 )
	mvCandList [ i++ ] = mvB1

- The variable numCurrCand is set equal to the number of merging candidates in the mvCandList.

- If general_merge_flag[ xCb ][ yCb ] is equal to 1, the variable MaxNumBVPCand is set equal to MaxNumMergeCand. Otheriwse (general_merge_flag[ xCb ][ yCb ] is equal to 0), the variable MaxNumBVPCand is set equal to 2.

- When numCurrCand is less than MaxNumMergeCand MaxNumBVPCand and smrNumHmvpIbcCand is greater than 0, the derivation process of IBC history-based motion vector candidates as specified in 8.6.2.4 is invoked with mvCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], and numCurrCand as inputs, and modified mvCandList and numCurrCand as outputs.

- When numCurrCand is less than MaxNumMergeCand MaxNumBVPCand, the following applies until numCurrCand is equal to MaxNumMergeCand MaxNumBVPCand:

    - mvCandList[ numCurrCand ][ 0 ] is set equal to 0.

    - mvCandList[ numCurrCand ][ 1 ] is set equal to 0.

    - numCurrCand is increased by 1.

- The variable mvIdx is derived as follows:

mvIdx = general_merge_flag[ xCb ][ yCb ] ? merge_idx[ xCb ][ yCb ] : mvp_l0_flag[ xCb ][ yCb ]	(8-903)

- The following assignments are made:

mvL[ 0 ] = mvCandList[ mvIdx ][ 0 ]		(8-904)

mvL[ 1 ] = mvCandList[ mvIdx ][ 1 ]		(8-905

Experimental Results

The proposed methods were implemented on VTM-5.0 software [2]. Simulations were performed under the common test condition [3]. The experiments were conducted on the platform of Intel Xeon E5-2690 v2 (3.0GHz) and 64GB RAM with GCC 7.3.1 compiler under CentOS Linux.

The experimental results compared to VTM-5.0 anchor are summarized in Table 1 and Table 2. More detailed results can be found in the attached spreadsheets. In the anchor and tests, the maximum number of merge candidates, MaxNumMergeCand, was set to 1. Also, IBC was enabled and MaxNumTriangleMergeCand was set to 0.

Table 1. Results of proposed method 1 compared to VTM-5.0 (non-CTC)

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| TGM | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 2. Results of proposed method 2 compared to VTM-5.0 (non-CTC)

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.01% | 100% | 100% | 0.00% | 0.03% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | 0.03% | 0.02% | 100% | 100% | 0.01% | 0.04% | 0.06% | 100% | 100% |  |  |  |  |  |
| Class B | -0.02% | -0.09% | -0.01% | 100% | 100% | -0.02% | 0.12% | -0.17% | 100% | 100% | 0.00% | 0.32% | 0.30% | 100% | 100% |
| Class C | -0.01% | -0.01% | -0.08% | 100% | 100% | -0.05% | 0.11% | -0.01% | 100% | 100% | -0.08% | 0.02% | -0.05% | 100% | 100% |
| Class E | -0.03% | -0.04% | 0.15% | 100% | 100% |  |  |  |  |  | -0.10% | 0.41% | 1.10% | 100% | 100% |
| Overall | -0.01% | -0.02% | 0.01% | 100% | 100% | -0.02% | 0.08% | -0.05% | 100% | 100% | -0.05% | 0.24% | 0.39% | 100% | 100% |
| Class D | -0.02% | 0.00% | 0.02% | 100% | 100% | -0.01% | -0.05% | 0.06% | 100% | 100% | -0.04% | 0.10% | -0.52% | 100% | 100% |
| Class F | -0.32% | -0.24% | -0.13% | 99% | 100% | -0.20% | -0.24% | -0.15% | 100% | 100% | -0.10% | 0.46% | -0.03% | 100% | 100% |
| TGM | -0.91% | -0.89% | -0.88% | 99% | 100% | -0.46% | -0.46% | -0.47% | 100% | 100% | -0.20% | -0.24% | -0.24% | 100% | 100% |

Conclusions

In this contribution, it was proposed to fix an issue in IBC BVP list derivation. The issue is that when the maximum number of merge candidates is set to 1, the second candidate could be unavailable in AMPV mode. Compared to VTM-5.0 with IBC and MaxNumMergeCand = 1, the proposed method 1 reportedly shows that there are no BD-rate changes. The proposed method 2 reportedly shows that average luma BD-rate change is -0.01%, -0.02% and -0.05% for AI, RA and LDB, respectively. For class F, it was observed that average luma BD-rate change is -0.32%, -0.20% and -0.10% for AI, RA and LDB, respectively. For TGM sequences, it was observed that average luma BD-rate change is -0.91%, -0.46% and -0.20% for AI, RA and LDB, respectively. It is recommended to consider adoption of the proposed method into VVC WD6 and VTM-6.0 to fix the issue under non-CTC.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).