| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0332 |
| --- | --- |

| Title: | AHG8: Signaling and Filtering for Reference Picture Resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

Proposed are the updates of JVET-N0052, which focus on signaling aspects for reference picture resampling (a.k.a. adaptive resolution change) and filter designs.

The key features of the proposed RPR scheme includes the followings:

- Support of adaptive (decoded) picture resolution change within a coded video sequence.

- Support of constant reference picture resolution for simplification of motion compensation process.

- Support of constant output picture resolution for guided display resolution

- Support of adaptive resampling mode (option1: with additional filtering, option2: without additional filtering)

In order to support the above features, the following high-level syntax modifications are proposed:

- Minimum/Maximum picture resolution (in DPS)

- A flag indicating that RPR is enabled in a coded video sequence (in SPS)

- Decoded picture resolutions in a table (in SPS)

- A flag indicating that any reference picture has the same spatial resolution, and the constant reference picture size. (in SPS)

- A flag indicating that any output picture has the same spatial resolution, and the constant output picture size. (in SPS)

- An index indicating the decoded picture size among the candidates signaled in SPS (in PPS)

- A flag indicating that motion vector scaling for temporal motion vector prediction is disabled (in PPS)

- Filter mode selection (in PPS)

Introduction

In Geneva meeting, it was agreed in principle to use the following (at least as an initial approach) for adaptive resolution change (ARC):

- Having the resolution of the stored reference picture being different from that of the current picture.

- A range of a factor of 4 in each dimension was suggested.

Technically, ARC can be implemented as reference picture resampling (RPR). Implementing reference picture resampling has two major aspects: the signaling of the resampling information in the bitstream and the resampling filters design. This document proposes both aspects, the syntax elements and the improved design of resampling filters.

Overview of Tencent’s RPR/ARC implementation

Fig. 1. Encoder modification for RPR

Fig. 2. Decoder modification for RPR

Figures 1 & 2 illustrate Tencent’s RPR en-/decoder implementation, respectively. In our implementation, it is possible to change the picture width and height, on a per picture granularity irrespective of the picture type. At the encoder, the input image data is down-sampled to the selected picture size for the current picture encoding. After the first input picture is encoded as intra-picture, the decoded picture is stored in the decoded picture buffer (DPB). When the consequent picture is down-sampled with a different sampling ratio and encoded as inter-picture, the reference picture(s) in the DPB is/are up-/down-scaled according the spatial ratio between the picture size of the reference and the current picture size. At the decoder, the decoded picture is stored in the DPB without resampling. However, the reference picture in the DPB is up-/down-scaled in relation to the spatial ratio between the currently decoded picture and the reference, when used for motion compensation. The decoded picture is up-sampled to the original picture size or the desired output picture size when bumped out for display. In motion estimation/compensation process, motion vectors are scaled in relation to picture size ratio as well as picture order count difference.

The key features of the proposed RPR scheme includes the followings:

- Support of adaptive (decoded) picture resolution change within a coded video sequence.

- Support of constant reference picture resolution for simplification of motion compensation process.

- Support of constant output picture resolution for guided display resolution

- Support of adaptive resampling mode (option1: with additional filtering, option2: without additional filtering)

Signaling of RPR

In order to support the desired features for RPR/ARC, described in Section 2, the following high-level syntax modifications are proposed.

Summary of syntax modifications:

- Minimum/Maximum picture resolution (in DPS)

- To facilitate cap exchange/negotiation

- A flag indicating that RPR is enabled in a coded video sequence (in SPS)

- Decoded picture resolutions in a table (in SPS)

- A list of decoded picture sizes, which may be used by one or more pictures in the coded video sequence.

- A flag indicating that any reference picture has the same spatial resolution, and the constant reference picture size. (in SPS)

- If the flag value is 1, any decoded picture in the coded video sequence is up-scaled by re-sampling process, so that any reference picture stored in DPB has the same picture size with the reference picture size, signalled in SPS.

- A flag indicating that any output picture has the same spatial resolution, and the constant output picture size. (in SPS)

- If the flag value is 1, any output picture in the coded video sequence is up-scaled by re-sampling process, so that any outputted picture has the same picture size with the output picture size, signalled in SPS.

- An index indicating the decoded picture size among the candidates signaled in SPS (in PPS)

- To facilitate cap exchange/negotiation

- A flag indicating that motion vector scaling for temporal motion vector prediction is disabled (in PPS)

- If the flag value is 1, any temporal motion vector prediction is processed without motion vector scaling. – further study in CE is requested.

- Filter mode selection (in PPS)

- Details about filter mode selection will be discussed in Section 4.

The resulting syntax could look as follows:

- Decoder parameter set RBSP syntax

| dec_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| max_pic_width_in_luma_samples | ue(v) |
| max_pic_height_in_luma_samples | ue(v) |
| ... |  |
| } |  |

max_pic_width_in_luma_samples specifies the maximum width of decoded pictures in units of luma samples in the bitstream. max_pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. The value of max_pic_width_in_luma_samples[ i ] cannot be greater than the value of max_pic_width_in_luma_samples.

max_pic_height_in_luma_samples specifies the maximum height of decoded pictures in units of luma samples. max_pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. The value of max_pic_height_in_luma_samples[ i ] cannot be greater than the value of max_pic_height_in_luma_samples.

- Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| reference_pic_resampling_flag | u(1) |
| if(reference_pic_resampling_flag) { |  |
| num_dec_pic_size_in_luma_samples_minus1 | ue(v) |
| for( i = 0; i <= num_dec_pic_size_in_luma_samples_minus1; i++ ) { |  |
| dec_pic_width_in_luma_samples[ i ] | ue(v) |
| dec_pic_height_in_luma_samples[ i ] | ue(v) |
| } |  |
| constant_ref_pic_size_flag | u(1) |
| if(constant_ref_pic_size_flag) |  |
| { |  |
| reference_pic_width_in_luma_samples | ue(v) |
| reference_pic_height_in_luma_samples | ue(v) |
| } |  |
| constant_output_pic_size_flag | u(1) |
| if(constant_output_pic_size_flag) |  |
| { |  |
| output_pic_width_in_luma_samples | ue(v) |
| output_pic_height_in_luma_samples | ue(v) |
| } |  |
| } |  |
| else { |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| } |  |
| ... |  |
| } |  |

reference_pic_resampling_flag equal to 1 specifies that the decoded picture size of a coded picture associated with the SPS may or may not change within the coded video sequence. reference_pic_resampling_flag equal to 0 specifies that the decoded picture size of a coded picture associated with the SPS shall not change within the coded video sequence. When the value of reference_pic_resampling_flag is equal to 1, one or more decoded picture sizes (dec_pic_width_in_luma_samples[ i ], dec_pic_height_in_luma_samples[ i ]), which may be indicated and used by a coded picture within the coded video sequence, are present, and a constant reference picture size (reference_pic_width_in_luma_samples, reference_pic_height_in_luma_samples) and a constant output picture size output_pic_width_in_luma_samples, output_pic_height_in_luma_samples) are present, conditioned on the values of constant_ref_pic_size_present_flag and constant_output_pic_size_present_flag, respectively.

constant_ref_pic_size_flag equal 1 specifies that reference_pic_width_in_luma_samples and reference_pic_height_in_luma_samples are present. constant_ref_pic_size_flag equal 0 specifies that eference_pic_width_in_luma_samples and reference_pic_height_in_luma_samples are not present.

reference_pic_width_in_luma_samples specifies the width of the reference picture in units of luma samples. reference_pic_width_in_luma_samples shall not be equal to 0. When not present, the value of reference_pic_width_in_luma_samples is inferred to be equal to dec_pic_width_in_luma_samples[ i ].

reference_pic_height_in_luma_samples specifies the height of the reference picture in units of luma samples. reference_pic_height_in_luma_samples shall not be equal to 0. When not present, the value of reference_pic_height_in_luma_samples is inferred to be equal to dec_pic_height_in_luma_samples[ i ].

NOTE1 – The size of the reference picture, stored in DPB, shall be equal to the values of reference_pic_width_in_luma_samples and reference_pic_height_in_luma_samples, when the value of constant_pic_size_present_flag is equal to 1. In this case, any additional resampling process is not performed for motion compensation.

constant_output_pic_size_flag equal 1 specifies that output_pic_width_in_luma_samples and output_pic_height_in_luma_samples are present. constant_output_pic_size_flag equal 0 specifies that output_pic_width_in_luma_samples and output_pic_height_in_luma_samples are not present.

output_pic_width_in_luma_samples specifies the width of the output picture in units of luma samples. output_pic_width_in_luma_samples shall not be equal to 0. When not present, the value of output_pic_width _in_luma_samples is inferred to be equal to dec_pic_width_in_luma_samples[ i ].

output_pic_height_in_luma_samples specifies the height of the output picture in units of luma samples. output_pic_height_in_luma_samples shall not be equal to 0. When not present, the value of output_pic_height_in_luma_samples is inferred to be equal to dec_pic_height_in_luma_samples[ i ].

NOTE2 – The size of the output picture shall be equal to the values of output_pic_width_in_luma_samples and output_pic_height_in_luma_samples, when the value of constant_output_pic.

num_dec_pic_size_in_luma_samples_minus1 plus 1 specifies the number of the decoded picture size (dec_pic_width_in_luma_samples[ i ], dec_pic_height_in_luma_samples[ i ] ) in units of luma samples in the coded video sequence.

dec_pic_width_in_luma_samples[ i ] specifies the i-th width of the decoded picture sizes in units of luma samples in the coded video sequence. dec_pic_width_in_luma_samples[ i ] shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

dec_pic_height_in_luma_samples[ i ] specifies the i-th height of the decoded picture sizes in units of luma samples in the coded video sequence. dec_pic_height_in_luma_samples[ i ] shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

NOTE3 – The i-th decoded picture size (dec_pic_width_in_luma_samples[ i ], dec_pic_height_in_luma_samples[ i ] ) may be equal to the decoded picture size of the decoded picture in the coded video sequence.

It is a requirement of bitstream conformance that the value of constant_output_pic_size_flag is equal to 1, when the value of constant_ref_pic_size_flag is equal to 1.

It is a requirement of bitstream conformance that the values of output_pic_width_in_luma_samples and output_pic_height_in_luma_samples are greater than or equal to the values of dec_pic_width_in_luma_samples[ i ] dec_pic_height_in_luma_samples[ i ], respectively.

- Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if(reference_pic_resampling_flag) { |  |
| dec_pic_size_idx | ue(v) |
| disabling_motion_vector_scaling_flag | u(1) |
| resampling_mode | u(2) |
| if(constant_ref_pic_size_flag) { |  |
| update_ref_pic_size_flag | u(1) |
| if(update_ref_pic_size_flag) |  |
| { |  |
| updated_ref_pic_width_in_luma_samples | ue(v) |
| updated_ref_pic_height_in_luma_samples | ue(v) |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |

dec_pic_size_idx specifies that the width of the decoded picture shall be equal to pic_width_in_luma_samples[ dec_pic_size_idx ] and the height of the decoded picture shall be equal to pic_height _in_luma_samples[ dec_pic_size_idx ].

disabling_motion_vector_scaling_flag equal 1 specifies that a reference motion vector is used without scaling process dependent on POC values or spatial resolutions for temporal motion vector prediction. disabling_motion_vector_scaling_flag equal 0 specifies that a reference motion vector is used with or without scaling process dependent on POC values or spatial resolutions for temporal motion vector prediction.

resampling_mode equal to 0 specifies that the resampling operation is not processed for block-level. resampling_mode equal to 1 specifies that the resampling operation is processed for block-level, before the interpolation operation for motion compensation. Other values are reserved.

update_ref_pic_size_flag equal 1 specifies that updated_ref_pic_width_in_luma_samples and updated_ref_pic_height_in_luma_samples are present. update_ref_pic_size_flag equal 0 specifies that updated_ref_pic_width_in_luma_samples and updated_ref_pic_height_in_luma_samples are not present.

updated_ref_pic_width_in_luma_samples specifies the width of the reference picture in units of luma samples. updated_ref_pic_width_in_luma_samples shall not be equal to 0. When not present, the value of updated_ref_pic_width_in_luma_samples is inferred to be equal to reference_pic_width_in_luma_samples.

updated_ref_pic_height_in_luma_samples specifies the height of the reference picture in units of luma samples. updated_ref_pic_height_in_luma_samples shall not be equal to 0. When not present, the value of updated_ref_pic_height_in_luma_samples is inferred to be equal to reference_pic_height_in_luma_samples.

NOTE4 –When update_ref_pic_size_flag is equal 1, all reference pictures stored in DPB and the current decoded picture are resampled to the reference pictures with the updated reference picture size (updated_ref_pic_width_in_luma_samples, updated_ref_pic_height_in_luma_samples).

Discussions on Filter design and Reference Picture Management

Four different filter design and reference picture management methods for RPR are possible, in principle. In this Section, four methods are compared to each other, highlighting its pros and cons in terms of reference picture management, computational complexity and memory bandwidth. As a conclusion, we propose a hybrid design of two promising approaches.

Method-1: Pre-generating multiple reference pictures with different resolutions, by up-/down-sampling process

This method 1 needs two sets of filters (or poly-phase filter banks) for up-/down-sampling of a decoded picture to multiple reference pictures. Once a coded picture is decoded with the current decoded picture size, the decoded picture is resampled to generate multiple reference pictures with different spatial resolutions, of which the number is equal to the number of decoded picture sizes used in the current coded video sequence.

The advantages of the method 1:

- No low-level tool changes. (No changes on interpolation process for motion compensation)

- Resampling with any arbitrary ratio is theoretically possible, depending on filter design.

- No memory bandwidth increment for interpolation of motion compensation

The disadvantages of the method 1:

- Substantial increment of DPB buffer size (N-times) to store multiple reference pictures with different resolutions

- N-times picture-level resampling process per picture, where N is equal to the number of decoded picture sizes used in the coded video sequence

- Memory bandwidth increment for picture-level resampling process

It is notified that the method-1 is almost identical to the picture-based resampling described in JVET-N0118.

Method-2: Cascaded(or combined) filtering process with one reference picture

This method 2 needs two sets of filters (or poly-phase filter banks) for up-/down-sampling of a reference block. Once a coded picture is decoded with the current decoded picture size, the decoded picture is stored in DPB, without resampling. When the decoded picture is used as a reference picture for motion compensation, each reference block is firstly resampled to the target resolution and secondly interpolated for motion compensation. In terms of filter design, both the resampling filters and the interpolation filter can be combined into a set of filter bank. Then, only one filtering (combining resampling and interpolation) process is needed for the method 2.

The advantages of the method 2:

- No increment of DPB buffer size

- Resampling with any arbitrary ratio is theoretically possible, depending on filter design.

- One-time block-level resampling process for each motion-compensated block.

The disadvantages of the method 2:

- Memory bandwidth increment for block-level resampling + motion compensation process

- Low-level tool changes on block-level resampling + motion compensation process

It is notified that the method-2 is almost identical to the method proposed by JVET-N0052 and the block-based resampling described in JVET-N0118.

Method-3: Reusing the motion compensation interpolator for resampling

This method 3 does not need additional filters for up-/down-sampling. Instead, the resampling process is realized in the interpolation process for motion compensation. Once a coded picture is decoded with the current decoded picture size, the decoded picture is stored in DPB, without resampling. When the decoded picture is used as a reference picture for motion compensation, each reference block is interpolated for motion compensation. Then, the interpolated pixels are sampled with the sampling ratio determined by the spatial resolution difference between the current picture and the reference picture.

The advantages of the method 3:

- No increment of DPB buffer size

- No memory bandwidth increment

- No additional resampling process

The disadvantages of the method 3:

- Resampling ratio is limited (up-sampling ratio not less than 1/8, down-sampling ratio not larger than 2)

- Quality of resampling may be relatively poor

- Low-level tool changes on interpolation for motion compensation

It is notified that the method-3 is almost identical to the method proposed by JVET-N0279.

Method-4: Resampling the decoded picture to the reference picture with a constant picture size

This method 2 needs two sets of filters (or poly-phase filter banks) for up-/down-sampling of the reference pictures. Once a coded picture is decoded with the current decoded picture size, the decoded picture is resampled with the scaling ratio determined by the spatial resolution difference between the decoded picture size and the constant reference picture size, and stored in DPB, as a reference picture. When the reference picture is used for motion compensation, if the reference picture size is equal to the currently decoded picture size (in most cases, yes), no resampling process is needed for motion compensation. Otherwise, method 2 or method 3 needs to be used. In case that the picture size is infrequently changed, the reference picture size is updated (signalled in PPS). Then, all reference pictures stored in DPB are rescaled according to the scaling ratio determined by the updated reference picture size and stored in DPB. If the picture resolution is changed two or three times within a coded video sequence. The resampling process per picture is needed three or four times, only. Another benefit of the method 4 can reduce the DPB buffer size, by intentionally reducing the reference picture stored in DPB.

The advantages of the method 4:

- No low-level tool changes. (No changes on interpolation process for motion compensation)

- Resampling with any arbitrary ratio is theoretically possible, depending on filter design

- No DPB buffer size increment (Moreover, it can reduce the DPB buffer size intentionally)

- No memory bandwidth increment for interpolation of motion compensation

- Only a few number of picture-level resampling process, when the picture size is infrequently changed

The disadvantages of the method 4:

- Inappropriate for cases that the picture size is frequently changed

As captured by the above comparison, each method has pros and cons in terms of computational complexity, memory bandwidth, DPB size, low-level tool changes and so on. Focusing on DPB size and memory bandwidth issue significantly, the methods 3 and 4 are preferred. It is asserted that the method 3 is good for cases that the picture size is frequently changed, for example a picture with a higher temporal ID may have a different picture size that a picture with temporal ID equal to 0. Whereas, we assert that the method 4 is good for cases that the picture size is infrequently changed, for example the picture resolution is changed at a scene change point, within a coded video sequence. A CE to study the comparison of those methods is recommended.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).