| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0337 |
| --- | --- |

| Title: | AHG8/AHG17: Layered Random Access point (LRA) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution proposes a new random access point (RAP) picture type, layered random access (LRA), which is enabled by the reference picture resampling (RPR) and the layer concept. The proposed LRA picture has two variants, LRA with inter-prediction and LRA without inter-prediction, which are beneficial for coding gain improvement and latency reduction, respectively.

Introduction

As random access point (RAP) pictures, instantaneous decoding refresh (IDR) and clean random access (CRA) pictures have been employed in HEVC and VVC. The CRA picture provides a better coding efficiency than an IDR picture, by enabling an open group of picture (GOP) structure which allows a following picture to reference a leading picture for motion compensated prediction. However, any conventional RAP types, including IDR and CRA, are basically coded as an intra coded picture (I-picture). Therefore, inserting RAP pictures frequently sacrifices the coding efficiency considerably. External decoding refresh (EDR) and dependent random access (DRA) were recently introduced. While those RAP picture types show significant coding gains, it is asserted that any special handling or assist from system-level is required to support the random access feature with those RAP types.

In this contribution, we propose a new RAP picture type, layered random access (LRA), which is enabled by the reference picture resampling (RPR) and the layer concept (with inter-layer prediction), which were agreed in principle to VVC. The proposed LRA picture has two variants, LRA with inter-prediction and LRA without inter-prediction. In this proposal, these two variants are called LRA-D (dirty) and LRA-C (clean), respectively, for a reason. The proposed LRA-D and LRA-C are useful in terms of coding efficiency improvement and latency reduction, respectively.

Layered random access (LRA) with inter-prediction

An LRA access unit (AU) consists of two (layered) coded pictures, including a base layer picture and one (or more) enhancement layer picture. The spatial resolution of the base layer picture can be lower than or equal to that of the enhancement layer picture. The base layer picture is coded as an I-picture, while the enhancement layer picture is coded as P-/B-picture. The enhancement layer picture in the LRA access unit can reference decoded pictures stored in the decoded picture buffer (DPB), including the base layer picture of the same AU and the previously decoded pictures of other AUs (in case of LRA-D).

In most cases, the bitrate of the base layer picture in the LRA AU could be much lower than that of the IDR/CRA picture, because the base layer picture is coded in lower spatial resolution or with higher quantization parameter. Also, the bitrate of the enhancement layer picture of LRA-D could be much lower than that of the IDR/CRA picture, because the enhancement layer picture of LRA-D is coded as P-/B-picture by referencing the previous picture and/or the base layer picture of the same AU for motion compensated prediction and motion vector prediction. It is asserted that the total bitrate of the base layer picture and the enhancement layer picture in an LRA-D AU is considerably lower than that of the single IDR/CRA picture, in most cases. Fig. 1 shows an example of the prediction structure with an LRA-D AU. When a random access does not occur at an LRA-D AU, the base and enhancement layer pictures of the LRA-D AU can be decoded, sequentially.

Figure 1. An example of sequence structure of LRA with inter-prediction

One disadvantage of the LRA-D is that the enhancement layer picture of LRA-D cannot be correctly decoded, when the random access may occur at the LRA-D AU, because the enhancement layer picture has decoding dependency from the previous reference pictures, which are not present in the current DPB due to the random access. In this case, the decoded base layer picture is re-sampled (with the RPR), and the resampled picture replaces the enhance layer picture. Fig. 2 shows an example that a random access occurs at an LRA-D AU. The re-sampled based layer picture of the LRA-D AU could be stored in the DPB and used as a reference picture for motion compensated prediction of the sub-sequent pictures. Probably, a certain mismatch between the resampled base layer picture and the decoded enhancement layer picture of LRA-D may appear in the current decoded picture, and propagate to the subsequent pictures. Since the reconstructed picture is not clean, when a random access occurs, the LRA with inter-prediction is called LRA-D (dirty). The drift can be cleaned up by another subsequent IRAP picture (IDR/CRA).

Figure 1. An example of sequence structure of LRA with inter-prediction, when a random access occurs.

Layered random access (LRA) without inter-prediction

Same as the above case, an LRA-C access unit (AU) consists of two (layered) coded pictures, including a base layer picture and one (or more) enhancement layer picture. The different is that the enhancement layer picture of LRA-C cannot reference the previously decoded pictures of other AUs for motion compensation. Only inter-prediction from the base layer picture to the enhancement layer picture is allowed. The advantage of LRA-C is not the coding gain benefit. It can reduce a latency, by discarding an enhancement layer picture, when starting with an LRA AU. As usual, the first IRAP picture at full resolution results in increased latency. Adaptive downsizing of the first IRAP picture bits may enable an adaptive control of latency for real-time streaming. As a disadvantage, discarding the enhancement layer picture may cause a drift of mismatch until another IRAP picture refreshes. However, as long as the enhancement layer picture is not discarded, a clean random access is supported.

Conclusion

In spite of the dirty random access, we assert that the proposed LRA has remarkable benefits in terms of coding gain and latency. In order to measure the benefits quantitatively with object means, we suggest further studies of the proposed method in a CE or AhG.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).