| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0338-v2 |
| --- | --- |

| Title: | AHG12: Improvements on Slice, Tile and Brick signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent America |  |  |

_____________________________

# Abstract

In this contribution, we propose the following modifications on slice, tile or brick in the current VVC WD.

Proposal-1: Signaling sh_slice_id instead of slice_address, if the value of rect_slice_flag is equal to 1

Proposal-2: Adding an implicit derviation rule of the values of top_left_brick_idx[ i ] and bottom_right_brick_idx_delta[ i ], for the last rectangular slice.

Proposal-3: Replacing num_brick_rows_minus1[ i ] with num_brick_rows_minus2[ i ]

Proposal-4: Adding single_brick_in_pic_flag to enable one tile with multiple bricks in a picture

# Proposal-1

In the latest VVC WD (JVET-N1001-v8), the syntax element slice_address is interpreted as the brick ID with rect_slice_flag equal to 0, or as the slice ID with rect_slice_flag equal to 1. The bit length is also different for each case. It is asserted that the mixed semantics degrade a readability.

Also, the constraint that the slices of a picture shall be in increasing order of their slice_address values cannot be conformed, if the value of srect_slice_flag equal to 1, because the slice ID does not have such a constraint.

Therefore, we propose to adopt one of the following options:

- Option-1: If rect_slice_flag is equal to 1, sh_slice_id is signaled instread of slice_address.

- Option-2: Removal of the constraint “The slices of a picture shall be in increasing order of their slice_address values.”

- Option-3: Addition of the constraint “The slices of a picture shall be in increasing order of their slice_id values.”.

- Option-4: Loosening the constraint “If signalled_slice_id_flag is equal to 0, the slices of a picture shall be in increasing order of their slice_address values.”

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ !rect_slice_flag && NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( rect_slice_flag ) |  |
| sh_slice_id | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |

Proposed specification text (Option-1)

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If rect_slice_flag is equal to 0, the following applies:

- The slice address is the brick ID as specified by Equation 6-8.

- The length of slice_address is Ceil( Log2 ( NumBricksInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumBricksInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the slice ID of the slice.

- The length of slice_address is signalled_slice_id_length_minus1 + 1 bits.

- If signalled_slice_id_flag is equal to 0, the value of slice_address shall be in the range of 0 to num_slices_in_pic_minus1, inclusive. Otherwise, the value of slice_address shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) − 1, inclusive.

sh_slice_id specifies the slice ID of the slice. When not present, the value of sh_slice_id is inferred to be equal to 0.

- The length of slice_id is signalled_slice_id_length_minus1 + 1 bits.

- If signalled_slice_id_flag is equal to 0, the value of sh_slice_id shall be in the range of 0 to num_slices_in_pic_minus1, inclusive. Otherwise, the value of sh_slice_id shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- The value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- The slices of a picture shall be in increasing order of their slice_address values.

- The shapes of the slices of a picture shall be such that each brick, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded brick(s).

Proposed specification text (Option-4)

- slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If rect_slice_flag is equal to 0, the following applies:

- The slice address is the brick ID as specified by Equation 6-8.

- The length of slice_address is Ceil( Log2 ( NumBricksInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumBricksInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the slice ID of the slice.

- The length of slice_address is signalled_slice_id_length_minus1 + 1 bits.

- If signalled_slice_id_flag is equal to 0, the value of slice_address shall be in the range of 0 to num_slices_in_pic_minus1, inclusive. Otherwise, the value of slice_address shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- The value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- If signalled_slice_id_flag is equal to 0, the slices of a picture shall be in increasing order of their slice_address values.

- The shapes of the slices of a picture shall be such that each brick, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded brick(s).

# Proposal-2

A bit-efficient signaling of the syntax elements top_left_brick_idx[ i ] and bottom_right_brick_idx_delta[ i ] is proposed. This proposal-2 is inspired by JVET-N0496, which proposed the implicit derivation rule of top_left_tile_idx[ i ] without explicit signaling. The proposal-2 in this contribution suggests an implicit derivation rule of the values of top_left_brick_idx[ i ] and bottom_right_brick_idx_delta[ i ], only for the last rectangular slice. In the case of the last rectangular slice, the derivation rule is relatively simple, as shown in the proposed specification text.

Proposed specification text

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 && i != num_slices_in_pic_minus1 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| if( i != num_slices_in_pic_minus1 ) |  |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |

top_left_brick_idx[ i ] specifies the brick index of the brick located at the top-left corner of the i-th slice. The value of top_left_brick_idx[ i ] shall not be equal to the value of top_left_brick_idx[ j ] for any i not equal to j. When not present, the value of top_left_brick_idx[ i ] is inferred to be equal to i. The length of the top_left_brick_idx[ i ] syntax element is Ceil( Log2( NumBricksInPic ) bits.

bottom_right_brick_idx_delta[ i ] specifies the difference between the brick index of the brick located at the bottom-right corner of the i-th slice and top_left_brick_idx[ i ]. When single_brick_per_slice_flag is equal to 1, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to 0. The length of the bottom_right_brick_idx_delta[ i ] syntax element is Ceil( Log2( NumBricksInPic − top_left_brick_idx[ i ] ) ) bits.

It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile.

The variable NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows:

if( i == 0 )
	for( j = 0; j < NumBricksInPic; j++ )
		BricksToSliceMap[ j ] = num_slices_in_pic_minus1
if( i == num_slices_in_pic_minus1 ) {
   j = 0;
	while( BricksToSliceMap[ j ] == num_slices_in_pic_minus1 )		j++
		topLeftBkIdx = j
		botRightBkIdx = NumBricksInPic – 1
}
else {
		topLeftBkIdx = top_left_brick_idx[ i ]
		botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]
}

NumBricksInSlice[ i ] = 0
botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]
for( j = 0; j < NumBricksInPic; j++) {
		if( BrickColBd[ j ]  >=  BrickColBd[ top_left_brick_idx[ i ] topLeftBkIdx ]  &&
					BrickColBd[ j ]  <=  BrickColBd[ botRightBkIdx ]  &&
					BrickRowBd[ j ]  >=  BrickRowBd[ top_left_brick_idx[ i ] topLeftBkIdx ]  &&	(7-36)
					BrickRowBd[ j ] <= BrickRowBd [ botRightBkIdx ] ) {
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

# Proposal-3

For bit-efficiency, it is proposed to replace num_brick_rows_minus1[ i ] with num_brick_rows_minus2[ i ], because the value of num_brick_rows_minus1shall be greater than 1, if brick_split_flag[ i ] is equal to 1.

Proposed specification text

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus12[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus12 [ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

num_brick_rows_minus12[ i ] plus 1 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus12[ i ] shall be in the range of 1 to RowHeight[ i ] − 1, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_rows_minus1[ i ] is inferred to be equal to 0. Otherwise, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus12[ i ] is inferred as specified in 6.5.1.

# Proposal-4

In the latest VVC WD (JVET-N1001-v8), the syntax element single_tile_in_pic_flag, signaled in PPS, indicates whether there is only one tile in a picture or there is more than one tile in each picture. If the value of single_tile_in_pic_flag is equal to 0, splitting the tile into bricks is not allowed, because the syntax elements brick_splitting_present_flag and brick_split_flag[ i ] are not present. If brick_split_flag[ i ] is not present, the value of each brick_split_flag[ i ] is inferred to be equal to 0 and no tiles of pictures referring to the PPS are divided into two or more bricks. In order to enable one tile with multiple bricks in a picture, we propose to add the syntax element single_brick_in_pic_flag, which specifies that there is only one brick in each picture, or there is more than one brick in each picture. One possible scenario of having one tile with multiple bricks in a picture is that the picture is a sub-picture extracted from a picture having multiple tiles with multiple bricks. Also, the brick signaling has a minor benefit compared to the tile signaling, in terms of bit saving, when a picture is partitioned into bricks in a rectangular slice.

Proposed specification text

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| single_brick_in_pic_flag | u(1) |
| if( !single_brick_in_pic_flag ) { |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

single_brick_in_pic_flag equal to 1 specifies that there is only one brick in each picture referring to the PPS. single_brick_in_pic_flag equal to 0 specifies that there is more than one brick in each picture referring to the PPS.

It is a requirement of bitstream conformance that the value of single_brick_in_pic_flag shall be the same for all PPSs that are activated within a CVS.

single_tile_in_pic_flag equal to 1 specifies that there is only one tile in each picture referring to the PPS. single_tile_in_pic_flag equal to 0 specifies that there is more than one tile in each picture referring to the PPS. When not present, the value of single_tile_in_pic_flag is inferred to be equal to 1.

NOTE – In absence of further brick splitting within a tile, the whole tile is referred to as a brick. When a picture contains only a single tile without further brick splitting, it is referred to as a single brick.

It is a requirement of bitstream conformance that the value of single_tile_in_pic_flag shall be the same for all PPSs that are activated within a CVS.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019.