| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0339 |
| --- | --- |

| Title: | AHG17: On Virtual boundary signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent America |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to specify the indication of the i-th vitual boundary X-(or Y-) position as the difference value between the i-th vitual boundary X-(or Y-) and the (i-1)-th vitual boundary X-(or Y-), respectively. Thus, we propose to replace the syntax elements pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ] with pps_virtual_boundaries_pos_x_delta[ i ] and pps_virtual_boundaries_pos_y_delta[ i ]. It is asserted that signaling delta value of the virtual boundary position (x, y) is always possible and save bits.

Introduction

In the last Geneva meeting, the feature disabling loop filter across virtual boundaries was adopted [1]. Disabling loop filter across discontinuous boundaries is helpful to improve the subjective visual quality by alleviating undesired filtering effect.

In this contribution, it is proposed to specify the indication of the i-th vitual boundary X-(or Y-) position as the difference value between the i-th vitual boundary X-(or Y-) and the (i-1)-th vitual boundary X-(or Y-), respectively. Thus, the syntax elements pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ] are replaced by pps_virtual_boundaries_pos_x_delta[ i ] and pps_virtual_boundaries_pos_y_delta[ i ]. It is asserted that coding delta value of those syntax elements is always possible because the values of virtual boundary x, y can be in ascending order, and it results in bit saving. The proposed changes to the specification text are highlighted below.

Proposed specification text

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x_delta[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y_delta[ i ] | u(v) |
| } |  |
| … |  |
| } |  |

pps_num_ver_virtual_boundaries specifies the number of pps_virtual_boundaries_pos_x[ i ] syntax elements that are present in the PPS. When pps_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0.

pps_virtual_boundaries_pos_x_delta[ i ] is used to compute the value of PpsVirtualBoundariesPosX[ i ], which specifies the location of the i-th vertical virtual boundary in units of luma samples. The number of bits used to represent pps_virtual_boundaries_pos_x_delta[ i ] is Ceil( Log2( pic_width_in_luma_samples ) − 3 ). pps_virtual_boundaries_pos_x_delta[ i ] shall be in the range of 1 to Ceil( (pic_width_in_luma_samples - PpsVirtualBoundariesPosX[ i - 1 ])  ÷ 8 ) − 12, inclusive.

The location of the vertical virtual boundary PpsVirtualBoundariesPosX[ i ] is derived as follows:

if( i == 0 )
PpsVirtualBoundariesPosX[ i ] = pps_virtual_boundaries_pos_x_delta[ i ] * 8

else
PpsVirtualBoundariesPosX[ i ] = pps_virtual_boundaries_pos_x_delta[ i ] * 8 
                            + PpsVirtualBoundariesPosX[ i - 1 ] 	(7-37)

The distance between any two vertical virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

pps_num_hor_virtual_boundaries specifies the number of pps_virtual_boundaries_pos_y[ i ] syntax elements that are present in the PPS. When pps_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0.

pps_virtual_boundaries_pos_y_delta [ i ] is used to compute the value of PpsVirtualBoundariesPosY[ i ], which specifies the location of the i-th horizontal virtual boundary in units of luma samples. The number of bits used to represent pps_virtual_boundaries_pos_y_delta[ i ] is Ceil( Log2( pic_height_in_luma_samples ) − 3 ). pps_virtual_boundaries_pos_y_delta[ i ] shall be in the range of 1 to Ceil( (pic_height_in_luma_samples- PpsVirtualBoundariesPosY[ i - 1 ]) ÷ 8 ) − 12, inclusive.

The location of the horizontal virtual boundary PpsVirtualBoundariesPosY[ i ] is derived as follows:

if( i == 0 )
PpsVirtualBoundariesPosY[ i ] = pps_virtual_boundaries_pos_y_delta[ i ] * 8

else
PpsVirtualBoundariesPosY[ i ] = pps_virtual_boundaries_pos_y_delta[ i ] * 8 
                            + PpsVirtualBoundariesPosY[ i - 1 ]	(7-38)

The distance between any two horizontal virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019.