| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0341-v1 |
| --- | --- |

| Title: | Non-CE3: Unification of intra interpolation filter selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Jungwon Kang Hahyun Lee Sung-Chang Lim Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr jungwon@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents unification method of intra interpolation filter selection. In VTM5.0, an interpolation filter is selected differently according to an intra mode which is MRL, ISP, or regular. For regular mode, MDIS condition is used to select the interpolation filter, while a different condition from the MDIS is used for ISP mode. The proposed method removes the filter selection condition for ISP mode and unifies the filter selection process with regular(method1) or MRL(method2). It is reported that the average luma BD-rate for AI case of method1 and method2 are 0.00% and 0.00% compared to VTM 5.0, respectively.

- Algorithm description

In VTM5.0, two intra interpolation filters (cubic or Gaussian) are used for angular prediction. Basically, one filter is selected by MDIS condition which is based on current block size and intra prediction mode. However, when an intra mode is ISP a different condition from the MDIS is used to select the interpolation filter as represented in Table 1. The condition for ISP is satisfied when block width is less than or equal to 8 and intra prediction mode is greater than or equal to INTRA_ANGULAR34, or block height is less than or equal to 8 and intra prediction mode is less than INTRA_ANGULAR34.

Table 1. Interpolation filter selection in VTM5.0

| Intra angular mode | MRL | ISP | Regular |
| --- | --- | --- | --- |
| Integer sample copy modes | Cubic (fractional sample position 0) |  |  |
| Other angular modes | Cubic | Cubic (Mode ≥ 34 && W ≤ 8) or (Mode < 34 && H ≤ 8) | Cubic or Gaussian (MDIS condition) |
|  |  | Gaussian (otherwise) |  |

In this contribution, the condition for filter selection of ISP is removed to simplify and unify the filter selection process. After removing the condition, the filter selection process for ISP can be unified with the process of regular or MRL. Method1 unifies the filter selection process with regular and method2 unifies it with MRL as represented in Table2 and Table3, respectively.

Table 2. Interpolation filter selection in the method1

| Intra prediction mode | MRL | Regular/ISP |
| --- | --- | --- |
| Integer sample copy modes | Cubic (fractional sample position 0) |  |
| Other angular modes | Cubic | Cubic or Gaussian (MDIS condition) |

Table 3. Interpolation filter selection in the method2

| Intra prediction mode | MRL/ISP | Regular |
| --- | --- | --- |
| Integer sample copy modes | Cubic (fractional sample position 0) |  |
| Other angular modes | Cubic | Cubic or Gaussian (MDIS condition) |

Experimental results

The experiments were performed based on VTM 5.0 software [1] with VTM 5.0 configuration file under JVET common test condition [2].

Table 4 and Table 5 show experimental results of the proposed method1 and method2 compared to VTM 5.0, respectively.

Table 4. Results of the proposed method1 over VTM 5.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.04% | 100% | 100% |
| Class A2 | 0.00% | -0.07% | -0.03% | 99% | 100% |
| Class B | 0.00% | -0.06% | 0.02% | 100% | 100% |
| Class C | 0.01% | -0.09% | 0.11% | 100% | 100% |
| Class E | 0.00% | -0.06% | -0.09% | 100% | 100% |
| Overall | 0.00% | -0.06% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.06% | -0.03% | 99% | 100% |
| Class F | 0.01% | 0.07% | 0.09% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.21% | 0.08% | 100% | 100% |
| Class A2 | 0.02% | 0.09% | 0.01% | 100% | 100% |
| Class B | 0.02% | -0.04% | -0.20% | 100% | 100% |
| Class C | -0.02% | -0.20% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.09% | -0.08% | 100% | 100% |
| Class D | 0.03% | -0.07% | 0.16% | 100% | 100% |
| Class F | 0.00% | 0.01% | 0.09% | 100% | 100% |

Table 5. Results of the proposed method2 over VTM 5.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | 0.00% | 99% | 100% |
| Class B | 0.00% | -0.06% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.03% | 100% | 100% |
| Class E | -0.01% | 0.00% | -0.06% | 100% | 100% |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.01% | 0.08% | 0.01% | 100% | 100% |
| Class F | -0.01% | -0.02% | -0.03% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.13% | 100% | 100% |
| Class A2 | 0.00% | -0.06% | 0.06% | 100% | 100% |
| Class B | 0.02% | -0.01% | -0.09% | 100% | 100% |
| Class C | -0.02% | -0.23% | -0.19% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.09% | -0.04% | 100% | 100% |
| Class D | 0.07% | -0.07% | 0.17% | 101% | 100% |
| Class F | 0.00% | 0.17% | 0.11% | 101% | 100% |

Draft text (Based on JVET-N1001-v8)

Method1

8.4.5.2.12	Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

…

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0.

- prefFilterFlag is equal to 1

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is greater than or equal to INTRA_ANGULAR34 and nTbW is less than or equal to 8

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is less than INTRA_ANGULAR34 and nTbH is less than or equal to 8.

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-9.

- The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- minDistVerHor is greater than intraHorVerDistThres[ nTbS ] and refFilterFlag is equal to 0

- Otherwise, filterFlag is set equal to 0.

Method2

8.4.5.2.12	Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

…

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0.

- prefFilterFlag is equal to 1

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is greater than or equal to INTRA_ANGULAR34 and nTbW is less than or equal to 8

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is less than INTRA_ANGULAR34 and nTbH is less than or equal to 8.

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-9.

- The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- minDistVerHor is greater than intraHorVerDistThres[ nTbS ] and refFilterFlag is equal to 0

- Otherwise, filterFlag is set equal to 0.

Conclusions

This contribution reports unification method of intra interpolation filter selection. The proposed methods remove the filter selection process for ISP mode and unifies with the one for MRL or regular. It can unify the filter selection process and reduce complexity. We suggest the proposed methods to be adopted into VTM 6.0.

Reference

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).