| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0342-v2 |
| --- | --- |

| Title: | Non-CE8: Unification of intra prediction mode of BDPCM block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Jungwon Kang Hahyun Lee Sung-Chang Lim Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr jungwon@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents unification of intra prediction mode of BDPCM block. In VTM5.0, a planar mode is stored for BDPCM block even though BDPCM is performed using horizontal or vertical intra prediction mode. The proposed modification logically stores a horizontal or vertical intra prediction mode when BDPCM is performed by horizontal or vertical direction, respectively. It is reported that the average luma BD-rate for class F and TGM in AI case are -0.12% and -0.18% compared to VTM 5.0, respectively.

- Algorithm description

In VTM5.0, BDPCM mode predicting residual signal horizontally or vertically was adopted. When the BDPCM is enabled horizontal or vertical intra prediction is performed for prediction block, but stored mode for the BDPCM block is different from the one for prediction. As represented in Table 1, Planar mode is stored for the BDPCM block.

Table 1. Intra prediction mode for BDPCM in VTM 5.0

| Intra prediction | BDPCM | Stored mode |
| --- | --- | --- |
| Horizontal mode | Horizontal direction | Planar mode |
| Vertical mode | Vertical direction |  |

In this contribution, the stored intra prediction mode is modified by horizontal or vertical intra prediction mode. When the BDPCM is performed by horizontal or vertical direction, a horizontal or intra prediction mode is stored, respectively. By the modification, intra prediction mode for intra prediction and storing is unified as represented in Table 2.

Table 2. Intra prediction mode for BDPCM in the proposed method

| Intra prediction / Stored mode | BDPCM |
| --- | --- |
| Horizontal mode | Horizontal direction |
| Vertical mode | Vertical direction |

Experimental results

The experiments were performed based on VTM 5.0 software [1] with VTM 5.0 configuration file under JVET common test condition [2].

Table 3 shows experimental results of the proposed method compared to VTM 5.0. In CTC, since the BDPCM is enabled in class F and TGM sequences, experimental results for regular sequences are same as anchor.

Table 3. Results of the proposed method over VTM 5.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.12% | 0.04% | -0.08% | 101% | 100% |
| TGM | -0.18% | -0.06% | -0.07% | 101% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.09% | 0.08% | 0.10% | 101% | 100% |
| TGM | -0.18% | -0.02% | -0.13% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.02% | -0.21% | -0.44% | 100% | 100% |
| TGM | -0.04% | 0.01% | 0.04% | 100% | 101% |

Draft text (Based on JVET-N1001-v8)

8.4.1	General decoding process for coding units coded in intra prediction mode

…

- The variable predModeIntra is derived as follows:

- If BdpcmFlag[ xCb ][ yCb ] is equal to 1, predModeIntra is derived as follows:

predModeIntra = BdpcmDir[ xCb ][ yCb ]  ?  50  :  18	(8-7)

- Otherwise, predModeIntra is set equal to IntraPredModeY[ xCb ][ yCb ].

- The general decoding process for intra blocks as specified in clause 8.4.5.1 is invoked with the sample location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before in-loop filtering.

8.4.3	Derivation process for luma intra prediction mode

…

IntraPredModeY[ xCb ][ yCb ] is derived as follows:

- If BdpcmFlag[ xCb ][ yCb ] is equal to 1, IntraPredModeY[ xCb ][ yCb ] is derived as follows:

IntraPredModeY[ xCb ][ yCb ] = BdpcmDir[ xCb ][ yCb ] ? INTRA_ANGULAR50 : INTRA_ANGULAR18

- Otherwise if BdpcmFlag[ xCb ][ yCb ] is equal to 1 or intra_luma_not_planar_flag[ xCb ][ yCb ] is equal to 0, IntraPredModeY[ xCb ][ yCb ] is set equal to INTRA_PLANAR.

- Otherwise (intra_luma_not_planar_flag[ xCb ][ yCb ] is equal to 1), the following ordered steps apply:

Conclusions

This contribution reports a modification of stored intra prediction mode for BDPCM block. By the modification intra prediction mode for intra prediction and storing of the BDPCM block is unified. The experimental results reportedly show that the average luma BD-rates of the proposed method is -0.12% and -0.18% for class F and TGM in AI case compared to VTM 5.0, respectively. We suggest the proposed methods to be adopted into VTM 6.0.

Reference

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).