| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0343 |
| --- | --- |

| Title: | CE3-related: Simplification on CCLM process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dae-Yeon Kim Ju-Ock Lee 11/12/13F, V&S Tower, 26 Samseong-ro 85-gil, Gangnam-gu, Seoul, 06194, KOREA Sung-Chang Lim Jinho Lee Jungwon Kang 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-10-3060-3361 brad.kim@chipsnmedia.com gretta.lee@chipsnmedia.com sclim@etri.re.kr jinosoul@etri.re.kr jungwon@etri.re.kr |
| Source: | Chips&Media and Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

In WD 5.0, CCLM process including the neighbouring sample position derivation, the luma down-sampling, and the linear model parameter derivation has many conditional branches, which are not desirable in terms of hardware and software implementations. To simplify these processes of the CCLM, it is proposed that unavailable neighbouring samples are substituted by the available sample values using reference sample generation process, which is already defined in WD. Additionally, fixed neighbouring sample positions for linear model parameter derivation are proposed. Compared to VTM 5.0, it is reported that the proposed method #1 shows 0.00%/0.00%/0.01% and 0.xx%/0.xx%/0.xx% BD-rate impact on Y/Cb/Cr components for AI and RA cases, respectively. And, the proposed method #2 shows 0.xx%/0.xx%/0.xx% and 0.xx%/0.xx%/0.xx% BD-rate impact on Y/Cb/Cr components for AI and RA cases, respectively.

Introduction

The CCLM is very efficient tool which can significantly improve the coding efficiency of 2.1%/19.1%/19.2% BD-rate impact on Y/Cb/Cr components for AI case according to the results from JVET AHG report: Tool reporting procedure (AHG13) [1].

However, the neighbouring sample position derivation, the luma down-sampling, and the linear model parameter derivation processes in the CCLM process have many conditional branches to use only available neighbouring samples as in WD 5.0 [2]. Thus, it is not desirable in terms of hardware and software implementations. More specifically, the neighbouring sample position derivation process is dependent on the availabilities of top neighbouring samples and left neighbouring samples. And, for the luma down-sampling process of the luma samples, the CCLM defines various luma down-sampling filter shapes which are selected depending on the position of samples and its neighbouring sample availabilities. Furthermore, both the number and positions of neighbouring samples to be used in the linear model parameter derivation are not fixed because they are also dependent on their availabilities. Therefore, the current CCLM design would be difficult to allow parallel processing such as SIMD in software implementation and consumes a lot of cycles and logics in hardware implementation.

Proposed methods

In this contribution, it is proposed that unavailable neighbouring samples are substituted by the available sample values by using the reference sample generation process, which is already defined in WD, to simplify the processes for the neighbouring sample position derivation, the luma down-sampling, and the linear model parameter derivation in the CCLM process. And, as all the luma neighbouring samples can be available by the above-mentioned reference sample generation process, fixed neighbouring sample positions for linear model parameter derivation are also proposed.

Fig. 1 shows a comparison of processing flows between the CCLM in WD 5.0 and the proposed CCLM. By adding the chroma and luma reference sample generation process, three processes such as the luma down-sampling process, the neighbouring sample position derivation, and the linear model parameter derivation, can be simplified.

Fig. 1. Processing flow comparison between CCLM in WD 5.0 and the proposed CCLM

Proposed usage of reference sample generation process

The proposed method reuses reference sample availability marking process in WD 5.0 without any change to generate the chroma neighbouring sample availability information. And, to make all of the luma and chroma neighbouring samples available, the neighbour samples of the chroma block and that of the corresponding luma block are generated using the chroma neighbouring sample availability information. By adding the reference sample generation process, maximum three luma neighbouring sample lines in both left and top sides are generated to remove the availability checking operations in the CCLM processes. To generate multiple luma neighbouring sample lines, the reference sample generation process is also employed as defined for the generation of multiple luma neighbouring sample lines for multi-reference line intra prediction (MRL).

Simplified neighbouring sample position derivation:

At the previous JVET meeting, JVET-N0271: CCLM derived with four neighbouring samples which uses at most four selected samples to derive the linear parameter model was adopted [3]. However, the number and the positions of the selected samples are dependent on the neighbouring sample availabilities. As all the luma and chroma neighbouring samples are available by using the reference sample generation process, the number and the positions of selected samples can be fixed. Thus, the neighbouring sample position derivation process can be simplified.

Simplified luma down-sampling:

The luma down-sampling process of the CCLM have many conditional branches and various luma down-sampling filter shapes. However, as all the luma neighbouring samples are available by using the reference sample generation process, the process can be also simplified significantly. By using the proposed method, most of the conditional branches and the various filter shapes can be removed so that unified luma down-sampling filter shapes are used. Fig. 2, Fig. 3, and Fig. 4 show the comparison of luma down-sampling filter shapes for inner block samples (C, TI, LI, and I), top neighbouring samples (TN), and left neighbouring samples (LN) in the WD 5.0 and the proposed method, respectively.

(a) Various luma down-sampling filter shapes for inner samples (C, TI, LI, and I) in WD 5.0

(b) Unified luma down-sampling filter shapes for inner samples (C, TI, LI, and I) in proposed method

Fig 2. Comparison of luma down-sampling filter shapes for inner samples

(a) Various luma down-sampling filter shapes for top neighbouring samples (TN) in WD 5.0

(b) Unified luma down-sampling filter shapes for top neighbouring samples (TN) in proposed method

Fig 3. Comparison of luma down-sampling filter shapes for top neighbouring samples

(a) Various luma down-sampling filter shapes for left neighbouring samples (LN) in WD 5.0

(b) Unified luma down-sampling filter shapes for left neighbouring samples (LN) in proposed method

Fig 4. Comparison of luma down-sampling filter shapes for left neighbouring samples

Simplified linear model parameter derivation:

The number of selected samples depend on the neighbouring sample availabilities in WD 5.0, thus additional operation and conditional branch are needed according to the number of selected samples. On the other hand, in the proposed method, always four selected samples are used, thus the additional operation and conditional branch can be removed.

Proposed fixed neighbouring sample position derivation

In WD 5.0, for the neighbouring sample position derivation, the selected sample positions can be different and depend on the length of neighbouring sample availability. On the contrast, by using the proposed usage of reference sample generation process, as all the luma neighbouring samples are available, the number of the selected samples is always equal to 4 and their positions can be fixed as following pseudo code:

If predModeIntra = = INTRA_LT_CCLM startPosT = (nTbW >> 2) startPosL = (nTbH >> 2) pickStepT = (nTbW) >> 1 pickStepL = (nTbH) >> 1 cntT = cntL = 2 else if predModeIntra = = INTRA_L_CCLM startPosL = (nTbH + Min(nTbW, nTbH)) >> 3 pickStepL = (nTbH + Min(nTbW, nTbH)) >> 2 cntL = 4 else if predModeIntra = = INTRA_T_CCLM startPosT = (nTbW + Min(nTbW, nTbH)) >> 3 pickStepT = (nTbW + Min(nTbW, nTbH)) >> 2 cntT = 4 pickPosL[ pos ] = (startPosL + pos * pickStepL), with pos = 0..cntL − 1. pickPosT[ pos ] = (startPosT + pos * pickStepT), with pos = 0..cntT − 1.

Table 1 shows comparison of lengths of top neighbouring samples and left neighbouring samples in the WD 5.0 and the proposed fixed neighbouring sample position derivation according to the CCLM modes.

Table 1. Comparison of lengths of top neighbouring samples and left neighbouring samples in WD 5.0 and proposed fixed neighbouring sample position derivation (*: variable numbers depending on neighbouring sample availability)

|  | INTRA_LT_CCLM |  | INTRA_T_CCLM | INTRA_L_CCLM |
| --- | --- | --- | --- | --- |
|  | Length of top neighbouring samples | Length of left neighbouring samples | Length of top neighbouring samples | Length of left neighbouring samples |
| WD 5.0 | 0 / tuWidth | 0 / tuHeight | tuWidth ~ tuWidth + Min(numAvailTR*, tuHeight) | tuHeight ~ tuHeight + Min(numAvailLB*, tuWidth) |
| Proposed fixed neighbouring sample position derivation | tuWidth | tuHeight | tuWidth + Min(tuWidth, tuHeight) | tuHeight + Min(tuWidth, tuHeight) |

Fig. 5 and Fig. 6 show examples of selected sample positions for INTRA_T_CCLM and INTRA_L_CCLM in the WD 5.0 and the proposed fixed neighbouring sample position derivation, respectively, in case that the chroma block size is equal to 8x4.

(a), (b) Selected sample positions depending on neighbouring sample availability in WD 5.0

(c) Fixed selected sample positions in proposed method

Fig. 5. Comparison of selected sample positions (gray-colored) for INTRA_T_CCLM

(a), (b), and (c) Selected sample positions depending on neighbouring sample availability in WD 5.0

(d) Fixed selected sample positions in proposed method

Fig. 6. Comparison of selected sample positions (gray-colored) for INTRA_L_CCLM

Experimental results

The experiments of the proposed methods were performed based on VTM-5.0 software [4] with VTM configuration files under JVET common test condition [5]. And, the experiments were conducted on the platform of Intel Xeon E5-2690 v2 (3.0GHz) and 64GB RAM with GCC 7.3.1 compiler under CentOS Linux. And, Table 2 describes the proposed CCLM methods.

Table 2. Description of the proposed CCLM methods

| Proposed Method | Description |
| --- | --- |
| #1 | Proposed usage of reference sample generation process (as specified in clause 2.1) |
| #2 | Proposed usage of reference sample generation process (as specified in clause 2.1) + Proposed fixed neighbouring sample position derivation (as specified in clause 2.2) |

The experimental results of the proposed methods compared to VTM 5.0 anchor are summarized in Table 3 to Table 4. The proposed method #1 shows 0.00%/0.00%/0.01% and 0.xx%/0.xx%/0.xx% BD-rate impact on Y/Cb/Cr components for AI and RA cases, respectively, as shown in Table 3. And, the proposed method #2 shows 0.xx%/0.xx%/0.xx% and 0.xx%/0.xx%/0.xx% BD-rate impact on Y/Cb/Cr components for AI and RA cases, respectively, as shown in Table 4. And, the detailed results for tests can be found in accompanied Excel files.

Table 3. Experimental result of the proposed method #1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.01% | 100% | 101% |
| Class A2 | 0.00% | 0.01% | -0.01% | 99% | 101% |
| Class B | 0.00% | 0.00% | 0.01% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.05% | 100% | 101% |
| Class E | 0.00% | -0.03% | -0.04% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 101% |
| Class D | -0.01% | 0.10% | 0.06% | 100% | 101% |
| Class F | 0.00% | -0.01% | -0.02% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | 92% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | 91% |
| Class B | 0.01% | -0.13% | 0.02% | 100% | 100% |
| Class C | -0.03% | -0.09% | -0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | 97% |
| Class D | 0.04% | -0.03% | 0.41% | 100% | 100% |
| Class F | -0.01% | -0.01% | -0.02% | 100% | 100% |

Table 4. Experimental result of the proposed method #2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | 88% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | 76% |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | 99% |
| Class C | 0.01% | 0.03% | 0.12% | 101% | 100% |
| Class E | -0.01% | -0.03% | 0.00% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | 93% |
| Class D | 0.00% | 0.07% | -0.13% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | 97% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | 73% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | 63% |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | 99% |
| Class C | -0.03% | -0.11% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | 86% |
| Class D | 0.03% | 0.19% | 0.16% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 91% | 100% |

Conclusion

In this contribution, to simplify the CCLM process, it is proposed that unavailable neighbouring samples are substituted by the available sample values using reference sample generation process. And, fixed neighbouring sample positions for linear model parameter derivation are proposed. From the experimental results, it is observed that the proposed method has almost no impact on BD-rate. Therefore, it is recommended to adopt this proposal to clean up the current CCLM design.

References

- W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, S. Liu, and D. Luo, “JVET AHG report: Tool reporting procedure (AHG13),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0013, Mar. 2019

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019

- J. Huo, Y. Ma, S. Wan, Y. Yu, M. Wang, K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, J. Li, S. Wang, and W. Gao, “CE3-1.5: CCLM derived from four neighbouring samples,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0271, Mar. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Chips&Media may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).