| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET- O0345-v2 |
| --- | --- |

| Title: | Non-CE3: MIP simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chang-Hao Yau1, Ching-Chieh Lin2 and Chun-Lung Lin3 2870 Zanker Road, Suite 140, San Jose, CA 95134, USA | Tel: Email: | +886-3-5914490 changhao.yau@itri.com1 JackLin@itri.com2 chunlung@itri.com3 |
| Source: | ITRI International |  |  |

_____________________________

- Abstract

This document proposed two simplifications for MIP. The first method proposed not to use MIP on the 4X4 block. The reduction of the stored MIP parameters is 1.8 Kilobyte (22.73% of the total stored MIP parameters). The results of the first method are the 0.02% luma loss for AI and 0.01% luma loss for RA configuration.

The second method porposes unsing one-step downsampling for the boundary for all block sizes. The reference samples for upsampling use the original reference samples. The second method shows the 0.01% luma gain for AI and RA configurations and saves 4% decoding time for AI configuration.

The results of the combination of the two methods are reported without luma loss for AI and with 0.01% luma loss for RA configuration.

Introduction

The Matrix-based Intra Prediction(MIP)[1] was adopted in the last meeting. There are three stages in the current MIP. The first stage is the downsampling of the reference samples from the boundary to prepare reference samples for the second stage and the third stage. The second stage is the generation of the reduced prediction samples by matrix-vector multiplication. The third stage is the upsampling by using the 1D linear interpolation to generate the final prediction.

The matrices and offset vectors in the second stage are from three sets S0, S1, S2. The set S0 used for 4x4 block consists of 18 matrices (16x4) and 18 offset vectors (16x1). The set S1 consists of 10 matrices(16x8) and 10 offset vectors (16x1). Matrices and offset vectors of S1 are used for 4×8, 8×4 and 8×8 blocks. Finally, the set S2 consists of 6 matrices(64x8) and 6 offset vectors(64x1). Matrices and offset vectors of S2 are used for the rest of the blocks. The total memory size for storage is 7.92 Kilobyte as shown in Table 1.

Table 1. Statistics of the parameters of MIP

| Sets | Numbers of parameters | Memory size (Kilobyte) | Percentage |
| --- | --- | --- | --- |
| S0 | 1440 | 1.80 | 22.73 |
| S1 | 1440 | 1.80 | 22.73 |
| S2 | 3456 | 4.32 | 54.55 |
| Total | 6336 | 7.92 |  |

The downsampling process is done by using averaging from reference samples. When the maximum width of the block size and height of the block size is larger than 8, one of the boundaries needs to downsample twice. Given an example for a 16×16 block as shown in Figure 1, the first downsampling is used to prepare the reference samples for upsampling. The reference samples from the second downsampling process are for matrix-vector multiplication.

Figure 1. An example of MIP processing for a 16X16 block in the current design

Proposed methods

Method 1: removal of MIP to 4X4 block

Because the matrices and offset vectors in S0 are only for 4X4 block and need 1.8 Kilobyte(22.73% of the total stored MIP parameters), the proposed method 1 is the removal of MIP to 4X4 block. The advantage is the reduction in storage and the improvement of decoding throughput.

Method 2: unification of the downsampling process

In the current design, the top and left boundary of the block have different downsampling processes. The proposed method 2 tries to unify the downsampling process. The reference samples for upsampling use the original reference samples. The reference samples for matrix-vector multiplication is using the downsampling processes directly from original reference samples as shown in Figure 2. Therefore, the boundaries need to downsample only once. The advantage of this method is the reduction of the number of conditional checks which may simplify the hardware design for the downsampling process in MIP.

Figure 2. An example of MIP processing for a 16X16 block in the method 2

Experimental results

The proposed modifications were implemented based on VTM5.0 reference software. According to the CE3 description in JVET-N1023 [2] and the common test conditions in JVET-N1010 [3], the proposed methods were evaluated under intra-only (AI) and random-access (RA) configurations with VTM5.0 configuration.

Test1

Table 2 and table 3 show the results of proposed method 1 for AI and RA respectively. The AI results for Test1 show 0.02% (Y) loss in terms of average BD-rate and the encoder run-time is not changed compared to VTM5.0 anchor.

Table 2. Results of the Test1 for all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.01% | -0.01% | 99% | 100% |
| Class B | 0.01% | 0.00% | 0.04% | 100% | 100% |
| Class C | 0.06% | 0.02% | -0.01% | 100% | 100% |
| Class E | 0.03% | -0.10% | -0.08% | 100% | 100% |
| Overall | 0.02% | -0.02% | -0.01% | 100% | 100% |
| Class D | 0.11% | 0.25% | 0.00% | 100% | 99% |
| Class F (optional) | 0.04% | 0.04% | 0.21% | 99% | 100% |

Table 3. Results of the Test1 for random access (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | 0.10% | 100% | 100% |
| Class A2 | 0.03% | 0.05% | 0.05% | 100% | 100% |
| Class B | 0.00% | 0.01% | -0.20% | 100% | 100% |
| Class C | 0.00% | -0.24% | 0.01% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.09% | -0.04% | 100% | 100% |
| Class D | 0.06% | 0.11% | 0.03% | 99% | 100% |
| Class F (optional) | 0.04% | -0.04% | -0.04% | 100% | 100% |

Test2

Table 4 and Table 5 show the experimental results of the proposed method 2 for AI and RA respectively. The simulation results (Test2) for AI show 0.01% coding gain in terms of average BD-rate of Y. In addition, the decoder run-time for Test2 can reduce 4% compared to VTM5.0.

Table 4. Results of the Test2 for all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.11% | -0.07% | 100% | 95% |
| Class A2 | -0.01% | -0.09% | -0.11% | 100% | 98% |
| Class B | -0.02% | -0.08% | 0.03% | 101% | 96% |
| Class C | 0.00% | -0.08% | 0.10% | 100% | 97% |
| Class E | -0.01% | -0.07% | -0.03% | 101% | 96% |
| Overall | -0.01% | -0.09% | 0.00% | 101% | 96% |
| Class D | 0.01% | 0.12% | -0.20% | 102% | 101% |
| Class F (optional) | 0.00% | 0.05% | 0.04% | 101% | 97% |

Table 5. Results of the Test2 for random access (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.24% | 0.01% | 99% | 100% |
| Class A2 | 0.00% | 0.07% | 0.04% | 99% | 100% |
| Class B | -0.01% | -0.04% | -0.27% | 99% | 101% |
| Class C | 0.00% | -0.08% | -0.30% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.16% | 99% | 101% |
| Class D | -0.01% | 0.09% | 0.27% | 99% | 101% |
| Class F (optional) | 0.03% | -0.04% | 0.18% | 99% | 101% |

Test3

Table 6 and Table 7 show the experimental results of the combination of method 1 and 2 for AI and RA respectively. The simulation results (Test3) for AI show x.xx% coding gain in terms of average BD-rate of Y.

Table 6. Results of the Test3 for all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.08% | -0.07% | 99% | 100% |
| Class A2 | -0.02% | -0.10% | -0.05% | 99% | 100% |
| Class B | -0.01% | -0.08% | -0.10% | 99% | 100% |
| Class C | 0.05% | -0.05% | 0.03% | 99% | 100% |
| Class E | 0.01% | -0.03% | -0.06% | 99% | 100% |
| Overall | 0.00% | -0.07% | -0.05% | 99% | 100% |
| Class D | 0.12% | 0.14% | 0.15% | 99% | 100% |
| Class F (optional) | 0.02% | 0.09% | 0.20% | 99% | 100% |

Table 7. Results of the Test3 for random access (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.27% | -0.06% | 100% | 100% |
| Class A2 | 0.02% | -0.05% | -0.04% | 100% | 100% |
| Class B | 0.01% | -0.06% | -0.08% | 100% | 100% |
| Class C | 0.02% | -0.20% | -0.20% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.14% | -0.10% | 100% | 100% |
| Class D | 0.08% | 0.03% | 0.07% | 99% | 100% |
| Class F (optional) | 0.04% | -0.06% | 0.11% | 100% | 100% |

Conclusions

In this contribution, the removal of MIP to 4X4 block is proposed for the reduction of the storage of the parameters of the matrices and offset vectors for MIP. The change in coding efficiency and run-time is negligible. Meanwhile, for all block sizes, it is proposed to downsample the boundaries only once for the preparation of matrix-vector multiplication. Then, the reference samples for upsampling use the original reference samples. The decoder run-time saves 4% compared to VTM5.0 without BD-rate changes. It is proposed to adopt the proposed methods into VTM 6.0.

References

- J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, P. Helle, T. Hinz, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, March 2019.

- G. Van der Auwera, L. Li, A. Filippov, “Description of Core Experiment 3(CE3): Intra Prediction and Mode Coding”, JVET-N1023, March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019.

Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).