| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0347-v1 |
| --- | --- |

| Title: | CE10: Experimental result of CNN-based In-Loop Filter (USTC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hengrui Zhao, Yuanying Dai Dong Liu, Ning Yan, Houqiang Li, Feng Wu Xu Chen, Haitao Yang | Tel: Email: | zhenry@mail.ustc.edu.cn daiyy@mail.ustc.edu.cn dongeliu@ustc.edu.cn nyan@mail.ustc.edu.cn |
| Source: | University of Science and Technology of China, Huawei incorporation |  |  |

_____________________________

- Abstract

This document presents the experiment results of integer convolutional networks for loop filter compared with floating point number convolutional networks which was proposed in [1]. First we train a floating point number network for loop filter, then we quantize and fine-tune the network to achieve an integer network.

Introduction

Convolutional neural networks (CNN) have been proved effective in video coding like post-processing or in-loop filtering. However, typical CNN is based on floating point algorithm, which is unacceptable in video coding. In this document, we propose to transform a floating point CNN into integer CNN, providing higher speed, smaller size and virtually same performance. Specifically, we replace ReLU activation in CNN with Bounded Linear Unit (BLU), which provides much convenience for activation quantization. We provide experimental results of 8-bit weights and 7-bit activations.

Solution

- 2.1 Proposed CNN quantization scheme

Figure 1: integer convolution compared with floating point convolution

Given a pre-trained 32-bit floating-point-number (float32) convolutional network, our goal is an integer-arithmetic-only network with low-precision integer weights and activations, which will decrease the model size and accelerate inference. Similar to Jacob (CVPR2018), our integer networks use 8-bit integer (int8) for weights and activations. The convolution can be operated at higher precision, but after convolution, we quantize the activations back to int8 numbers, which are used for further convolutions.

As shown in Figure 1, for an integer convolutional layer we perform integer convolution on input Xi and weights Wi to get feature maps Ui. Ui is adjusted by the bias bi to get Yi, which is then processed by the specific activation function--bounded linear unit (BLU) and quantized to get Vi. Vi is the input of the next layer.

To achieve such an integer network, we uniformly quantize the weights and biases of a float32 network. For the quantization, we assume the parameters in a convolutional kernel Wf roughly obey a normal distribution with zero mean. To fully utilize the limited precision of short integer, we map the maximal absolute value of Wf to the maximal possible absolute value of integer. Specifically, for a given float32 convolutional kernel Wf and int8 target,

Wi=Wf∆

where ∆ indicates the quantization step for Wf:

∆=max⁡(abs(Wf))127

To achieve the best accuracy, it is natural to make the integer networks close to the float32 networks during inference, because the float32 networks are well-trained. Accordingly, we assume a linear mapping between each of the kernels, feature maps and other parameters of an integer network and its floating-point counterpart.

A low-precision network can be viewed as a high-precision network corrupted by some noise, for example, the noise is introduced by the rounding operation of Wf. During the entire process of integer network inference, there are two kinds of noise: due to weight quantization and due to activation quantization, respectively. Therefore, our work is focused on controlling the quantization noise to ensure the performance of integer networks.

To control the noise of activation quantization, we use BLU in place of ReLU. BLU is a piecewise function:

It clips the value of activations by upper bound and lower bound, providing much convenience for quantization.

Our network architecture is shown in [1]. Given previous floating point networks, first we change the input data into 8-bit integer type, then we fine-tune the network. Then we discretize the weights and replace ReLU with BLU. Finally, we fine-tune the network with accumulate gradient decent algorithm and quantize the network.

Experiment

In inference stage, the filters in VTM5.0 including DF, SAO and ALF are all enabled. The additional CNN-based filter is located between DF and SAO. The CNN operation is implemented based on the cudnn library. The tests are conducted under the common test conditions (CTC). The test QP values are 22, 27, 32, and 37. Table 1 gives the experimental results of our integer CNN network under the all intra condition.

Table 1. Coding performance of CNNLF under the all intra condition

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.31% | -0.11% | 101% | 173% |
| Class A2 | -0.35% | -0.28% | -0.62% | 91% | 169% |
| Class B | -0.34% | -0.07% | -0.31% | 88% | 157% |
| Class C | -0.72% | -0.75% | -1.68% | 91% | 198% |
| Class E | -1.04% | -0.13% | -0.13% | 93% | 291% |
| Overall | -0.51% | -0.30% | -0.60% | 92% | 188% |
| Class D | -1.37% | -0.70% | -1.84% | 114% | 437% |
| Class F |  |  |  |  |  |

Reference

- Yuanying Dai, Dong Liu, Ning Yan, Feng Wu, “CE13: Experimental results of CNN-based In-Loop Filter (USTC),” document JVET-N0513, 14th JVET meeting: Geneva, MA, 19–27 Mar. 2019.

Patent rights declaration(s)

University of Science and Technology of China and Huawei incorporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).