| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0349 |
| --- | --- |

| Title: | CE6-related: Simplified LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the LFNST (Low Frequency Non-Separable Transform) process as described in CE6-2.1a. That is, it is proposed to modify the VTM-5 LFNST design as follows:

- When LFNST is applied, all transform coefficients outside the LFNST area are zeroed out (in VTM-5 coefficients X…47 in scan order are zeroed out, but there may be “high frequency primary transform left-overs” in later positions [X = 8 for 4x4/8x8; and X = 16 otherwise)

- Applying only one secondary transform within one transform block (in VTM-5 there are two secondary transforms for 4xN and Nx4 transform blocks when N > 4)

It is asserted that the simplifications reduce encoder runtimes and decoder complexity with minor impact on coding efficiency. Similarly to JVET-O0209, it is pointed out that in the case of these modifications the check for coefficients violating the LFNST area restriction can be done either by using the lastScanPos variable or checking locations of individual coefficients.

The BD-rate impact with respect to VTM-5 is +0.06 % and +0.02 % for AI and RA configurations, respectively. In AI configuration the encoder and decoder runtimes with respect to VTM-5 are reported to be 90 % and 99 %, respectively.

# Introduction

LFNST provides interesting coding efficiency gains but comes with significant burden for encoders to take advantage of the feature. Similarly, on the decoder side LFNST introduces extra complexity compared to using a pair of separable transforms, as a full primary transform needs to be performed in addition to the secondary transform. This contribution aims at lowering these complexities while maintaining the desired coding efficiency of the LFNST tool.

# Proposed simplifications

As defined in VTM-5, LFNST zeroes out a band of coefficients between scan positions 16 and 47 if the transform block size is larger than 8x8. In the case of 8x8 and 4x4 blocks, the zero-out starts from position 8. In the case of 4xN and Nx4 transform blocks there are two secondary transforms – one for the lowest 4x4 band and another one for the second lowest 4x4 band of coefficients. Figures 1a and 1b illustrate the VTM-5 design.

(a)                                                        (b)

Figure 1: VTM-5 LFNST design for a 16x16 block (a) and a 16x4 block (b). Transform blocks consist of double transformed areas D, zero-out areas Z and single transformed areas S.

It is proposed to simplify the current design by:

- Zeroing out all single transformed coefficients and setting the maximum allowed scan position for LFNST transform blocks to 15 (except for 4x4 and 8x8 for which the maximum positions for LFNST coefficients are kept at 7 as in VTM-5).

- Limiting the secondary transform operation in 4xN and Nx4 transform blocks to the lowest 4x4 coefficients.

Figure 2 illustrates the coefficient distribution in the coded LFNST transform blocks when proposed simplifications are applied.

(a)                                                        (b)

Figure 2: Proposed LFNST design for a 16x16 block (a) and a 16x4 block (b). Transform blocks consist now of a single double transformed area D and a single zero-out areas Z.

As proposed, the inverse LFNST stage always inputs at most 16 coefficients and outputs at most 48 coefficients that reside in the top 8x8 corner of the input to the inverse primary transform. As a result, the inverse primary transform can be implemented more efficiently as the high frequency basis functions do not need to be considered at the inverse transformation process. Another benefit is the simplification of the encoder search as the number of active coefficients in the larger LFNST blocks reduce dramatically.

# Proposed specification text changes

## Alternative 1: individual coefficient check

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) { |  |
| numSigCoeff++ |  |
| if( ( n >= 8 && i = = 0 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( ( i = = 1 ¦ ¦ i = = 2 ) && log2TbWidth >= 3 && log2TbHeight >= 3 ) ) |  |
| if( ( n >= 8 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( n >= 16 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ) |  |
| numZeroOutSigCoeff++ |  |
| } |  |
| … |  |
| } |  |
| … |  |

## Alternative 2: using lastScanPos

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| QState = 0 |  |
| if( ( lastScanPos >= 8 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( lastScanPos >= 16 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ) |  |  |
| numZeroOutSigCoeff++ |  |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| … |  |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) { |  |  |
| numSigCoeff++ |  |  |
| if( ( n >= 8 && i = = 0 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( ( i = = 1 ¦ ¦ i = = 2 ) && log2TbWidth >= 3 && log2TbHeight >= 3 ) ) |  |  |
| numZeroOutSigCoeff++ |  |  |
| } |  |  |
| … |  |  |
| } |  |  |
| … |  |  |

## Common parts

8.7.4	Transformation process for scaled transform coefficients

8.7.4.1	General

…

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, numLfnstX, numLfnstY, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	()

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	()

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	()

nLfnstSize = 1  <<  log2LfnstSize		()

numLfnstX = ( nTbW > 8  &&  nTbH  = = 4 )  ?  2  :  1	()

numLfnstY = ( nTbW = = 4  &&  nTbH  > 8 )  ?  2  :  1	()

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	()

- The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

- For xSbIdx = 0..numLfnstX − 1 and ySbIdx = 0..numLfnstY − 1, the following applies:

- The values of the list u[ x ] with x = 0..nonZeroSize − 1 are derived as follows:

xC = ( xSbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 0 ]	()

yC = ( ySbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 1 ]	()

u[ x ] = d[ xC ][ yC ] 			()

- The one-dimensional low frequency non-separable transformation process as specified in clause 8.7.4.2 is invoked with the input length of the scaled transform coefficients nonZeroSize, the transform output length nTrS set equal to nLfnstOutSize, the list of scaled non-zero transform coefficients u[ x ] with x = 0..nonZeroSize − 1, the intra prediction mode for LFNST set selection predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnst_idx[ xTbY ][ yTbY ] as inputs, and the list v[ x ] with x = 0..nLfnstOutSize − 1 as output.

- The array d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] with x = 0..nLfnstSize − 1, y = 0..nLfnstSize − 1 is derived as follows:

- If predModeIntra is less than or equal to 34, or equal to INTRA_LT_CCLM, INTRA_T_CCLM, or INTRA_L_CCLM, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( y < 4 )  ?  v[ x + ( y << log2LfnstSize ) ]  : 	()
	( ( x < 4 )  ?  v[ 32 + x + ( ( y − 4 ) << 2 ) ]  :  d[ x ][ y ] )

- Otherwise, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( x < 4 )  ?  v[ y + ( x << log2LfnstSize ) ]  : 	()
	( ( y < 4 )  ?  v[ 32 + y + ( ( x − 4 ) << 2 ) ]  :  d[ x ][ y ] )

# Compression efficiency results

The tables below summarize impacts of the proposed simplifications to the coding efficiency and reference software runtimes.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00 % | 0.11 % | 0.01 % | 92 % | 99 % |
| Class A2 | 0.00 % | 0.10 % | 0.11 % | 90 % | 99 % |
| Class B | 0.08 % | -0.06 % | 0.04 % | 91 % | 101 % |
| Class C | 0.10 % | -0.12 % | 0.05 % | 89 % | 98 % |
| Class E | 0.10 % | -0.06 % | 0.03 % | 92 % | 98 % |
| Overall | 0.06 % | -0.02 % | 0.05 % | 90 % | 99 % |
| Class D | 0.10 % | 0.14 % | -0.12 % | 90 % | 97 % |
| Class F | 0.02 % | 0.09 % | 0.09 % | 93 % | 97 % |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03 % | 0.04 % | 0.08 % | 99 % | 100 % |
| Class A2 | -0.01 % | 0.08 % | 0.05 % | 99 % | 102 % |
| Class B | 0.03 % | 0.03 % | -0.13 % | 99 % | 101 % |
| Class C | 0.03 % | -0.09 % | 0.03 % | 99 % | 100 % |
| Class E |  |  |  |  |  |
| Overall | 0.02 % | 0.01 % | -0.01 % | 99 % | 101 % |
| Class D | 0.06 % | 0.23 % | 0.11 % | 99 % | 100 % |
| Class F | 0.06 % | 0.10 % | 0.05 % | 98 % | 101 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).