| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0358 |
| --- | --- |

| Title: | AHG12: On uniform tile spacing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuji Fujimoto, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 Yuji.Fujimoto@sony.com Masaru.Ikeda@sony.com teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes the improved tile partitioning methods with uniform tile spacing for equally balanced parallel processing. In our method, tile boundary is not changed by extraction of tiles (or discarding tiles). Tile extraction (or discarding tiles) can be done with few changes in PPS and can be done by simply discarding VCL NAL unit for tiles

- Option1: Improved tile partitioning method with uniform tile spacing

- Option2: Add a flag to identify if bitstream supports tile extraction without changing tile boundary. When bitstream does not support tile extraction without changing tile boundary, tile partitioning method is the same as HEVC. When it is supported, the original tile boundary is signaled in PPS.

Those methods can be applied to Brick partitioning with uniform spacing.

Introduction

HEVC

In case of HEVC, when uniform tile spacing is set equal to 1 (uniform_spacing_flag = 1), the number of tile rows and columns are signaled in PPS. The PPS syntax of HEVC is shown in Table 1.

pic_parameter_set_rbsp( ) {

・・・・

tiles_enabled_flag

entropy_coding_sync_enabled_flag

if( tiles_enabled_flag ) {

num_tile_columns_minus1

num_tile_rows_minus1

uniform_spacing_flag

if( !uniform_spacing_flag ) {

for( i = 0; i < num_tile_columns_minus1; i++ )

column_width_minus1[ i ]

for( i = 0; i < num_tile_rows_minus1; i++ )

row_height_minus1[ i ]

}

・・・・

Table 1: PPS syntax in HEVC

Column width and row height are derived as follows.

if( uniform_tile_spacing_flag )
	for( i = 0; i  <=  num_tile_columns_minus1; i++ )
		colWidth[ i ] = ( ( i + 1 ) * PicWidthInCtbsY ) / ( num_tile_columns_minus1 + 1 ) − 
						( i * PicWidthInCtbsY ) / ( num_tile_columns_minus1 + 1 )
if( uniform_tile_spacing_flag )
	for( j = 0; j  <=  num_tile_rows_minus1; j++ )
		rowHeight[ j ] = ( ( j + 1 ) * PicHeightInCtbsY ) / ( num_tile_rows_minus1 + 1 ) − 
						( j * PicHeightInCtbsY ) / ( num_tile_rows_minus1 + 1 )

For example, when PicWidthInCtbsY = 10, PicHeightInCtbsY = 7, num_tile_columns_minus1 = 3, num_tile_rows_minus1 = 2 and uniform_tile_spacing_flag = 1, tile partition will be as follows.

Figure 1: An example of tile partition of HEVC (uniform_tile_spacing_flag = 1)

A picture is partitioned into tiles with almost equal number of CTUs. In case of Figure 1, column width is derived as follows.

colWidth[ 0 ] = (1*10)/4 – (0*10)/4 = 2

colWidth[ 1 ] = (2*10)/4 – (1*10)/4 = 3

colWidth[ 2 ] = (3*10)/4 – (2*10)/4 = 2

colWidth[ 3 ] = (4*10)/4 – (3*10)/4 = 3

However, when tiles are extracted from picture (not whole picture), tile boundary could be changed from the original position. For example, when left tile column is discarded as shown in Figure 2, PicWidthInCtbsY = 8 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 2, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1.

Figure 2: Example of tile extraction (left tile column is discarded)

In this case, column width is derived as follows.

colWidth[ 0 ] = (1*8)/3 – (0*8)/3 = 2

colWidth[ 1 ] = (2*8)/3 – (1*8)/3 = 3

colWidth[ 2 ] = (3*8)/3 – (2*8)/3 = 3

In another case, when right tile column is discarded as shown in Figure 3, PicWidthInCtbsY = 7 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 2, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1.

Figure 3: Example of tile extraction (right tile column is discarded)

In this case, column width is derived as follows.

colWidth[ 0 ] = (1*7)/3 – (0*7)/3 = 2

colWidth[ 1 ] = (2*7)/3 – (1*7)/3 = 2

colWidth[ 2 ] = (3*7)/3 – (2*7)/3 = 3

In another case, when both right and left tile columns are discarded as shown in Figure 4, PicWidthInCtbsY = 5 and PicHeightInCtbsY = 5, num_tile_columns_minus1 = 1, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1.

Figure 4: Example of tile extraction (right tile column is discarded)

In this case, column width is derived as follows.

colWidth[ 0 ] = (1*5)/2 – (0*5)/2 = 2

colWidth[ 1 ] = (2*5)/2 – (1*5)/2 = 3

VVC

In the current VVC draft 5 (JVET-N1001), when uniform tile spacing is set equal to 1 (uniform_tile_split_flag = 1), tile width of columns and height of rows are signaled in PPS. The PPS syntax is shown in Table 2.

pic_parameter_set_rbsp( ) {

・・・・

single_tile_in_pic_flag

if( !single_tile_in_pic_flag ) {

uniform_tile_spacing_flag

if( uniform_tile_spacing_flag ) {

tile_cols_width_minus1

tile_rows_height_minus1

} else {

num_tile_columns_minus1

num_tile_rows_minus1

for( i = 0; i < num_tile_columns_minus1; i++ )

tile_column_width_minus1[ i ]

for( i = 0; i < num_tile_rows_minus1; i++ )

tile_row_height_minus1[ i ]

・・・・

Table 2: PPS syntax in VVC draft 5 (JVET-N1001)

Column width and row height are derived as follows.

if( uniform_tile_spacing_flag ) {
　　　remainingWidthInCtbsY = PicWidthInCtbsY
　　　i = 0
　　　while( remainingWidthInCtbsY > ( tile_cols_width_minus1 + 1 ) ) {
	colWidth[ i++ ] = tile_cols_width_minus1 + 1
	remainingWidthInCtbsY  −=  ( tile_cols_width_minus1 + 1 )
　　　}
　　　colWidth[ i ] = remainingWidthInCtbsY
　　　num_tile_columns_minus1 = i
}

if( uniform_tile_spacing_flag ) {
           remainingHeightInCtbsY = PicHeightInCtbsY
           i = 0
           while( remainingHeightInCtbsY > ( tile_rows_height_minus1 + 1 ) ) {
	rowHeight[ i++ ] = tile_rows_height_minus1 + 1
	remainingHeightInCtbsY  −=  ( tile_rows_height_minus1 + 1 )
           }
           rowHeight[ i ] = remainingHeightInCtbsY
           num_tile_rows_minus1 = i
}

For example, when PicWidthInCtbsY = 10, PicHeightInCtbsY = 7, uniform_tile_spacing_flag = 1, tile_cols_width_minus1 = 2 and tile_rows_height_minus1 = 2, tile partition will be as follows.

Figure 5: An example of tile partition of VVC (uniform_tile_spacing_flag = 1)

In case of VVC, when tiles are extracted from picture, tile boundary will not be changed, as proposed by JVET-N0498. But when picture size is not a multiple of each tile size, right hand side and bottom row tiles could be smaller than other tiles. If its size is much smaller, as shown in Figure 5, computational cost distribution is not equally balanced, and such not-well balanced tile partition is not good for parallel processing.

Proposal

This contribution proposes the improved tile partitioning methods with uniform tile spacing for equally balanced parallel processing. In our method, tile boundary is not changed by extraction of tiles (or discarding tiles). Tile extraction (or discarding tiles) can be done with few changes in PPS and can be done by simply discarding VCL NAL unit for tiles

- Option1: Improved tile partitioning method with uniform tile spacing

- Option2: Add a flag to identify if bitstream supports tile extraction without changing tile boundary. When bitstream does not support tile extraction without changing tile boundary, tile partitioning method is the same as HEVC. When it is supported, the original tile boundary is signaled in PPS.

Those methods can be applied to Brick partitioning with uniform spacing.

Option 1

The proposed tile partitioning method for uniform tile spacing is described below. The initial value of parameters, e.g. picture size in CTU unit (PicWidthInCtbsY, PicHeightInCtbsY), tile column width and tile rows height (tileColsWidth , tileRowsHeight) are set as follows.

remainWidthInCtbsY = PicWidthInCtbsY, remainHeightInCtbs = PicHeightInCtbsY

remainTileColumns = tileColsWidth, RemaingTileRows = tileRowsHeight

There could be three variants to derive tile column width and row height.

Option1-a: width and height are calculated by truncation

Each tile size (colWidth[ i ], rowHeight[j]) is derived as follows.

colWidth[i] = remainWidthInCtbs/remaingTileColumns		(truncated integer)

remainWidthInCtbsY -= colWidth[i], remaingTileColumns--

rowHeight[j] = remainHeightInCtbs/remaingTileRows		(truncated integer)

remaingHeightInCtbsY -= rowHeight[j], remaingTileRows--

PPS syntax is shown in Table 3. This is the same as HEVC syntax.

pic_parameter_set_rbsp( ) {

・・・・

single_tile_in_pic_flag

if( !single_tile_in_pic_flag ) {

num_tile_columns_minus1

num_tile_rows_minus1

uniform_tile_spacing_flag

if( !uniform_tile_spacing_flag ) {

for( i = 0; i < num_tile_columns_minus1; i++ )

tile_column_width_minus1[ i ]

for( i = 0; i < num_tile_rows_minus1; i++ )

tile_row_height_minus1[ i ]

・・・・

Table 3: PPS syntax for Option 1-a and 1-b

Column width and row height are derived as follows.

if( uniform_tile_spacing_flag ) {
　　　remainingWidthInCtbsY = PicWidthInCtbsY

remainingTileColumns = tile_cols_width_minus1 + 1

i = 0
　　　while( remainingTileColumns > 0 ) {
	colWidth[ i ] = remainingWidthInCtbsY/ remainingTileColumns
	remainingWidthInCtbsY  −= colWidth[ i ]

remainingTileColumns--

i++
　　　}
}

if( uniform_tile_spacing_flag ) {
      remainingHeightInCtbsY = PicHeightInCtbsY

remainingTileRows = tile_rows_height_minus1 + 1

i = 0
      while( remainingTileRows > 0 ) {

rowHeight[ i ] = remainingHeightInCtbsY/ remainingTileRows
	remainingHeightInCtbsY  −= rowHeight[ i ]

remainingTileRows—

i++

}
}

For example, when PicWidthInCtbsY = 10, PicHeightInCtbsY = 7, uniform_tile_spacing_flag = 1, tile_cols_width_minus1 = 3 and tile_rows_height_minus1 = 2, tile partition will be as follows.

Figure 6: An example of tile partition of Option 1-a (uniform_tile_spacing_flag = 1)

In case of Figure 6, column width is derived as follows.

colWidth[ 0 ] = 10/4 = 2

colWidth[ 1 ] = 8/3 = 2

colWidth[ 2 ] = 6/2 = 3

colWidth[ 3 ] = 3/1 = 3

When left tile column is discarded as shown in Figure 7, PicWidthInCtbsY = 8 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 2, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1.

Figure 7: Example of tile extraction (left tile column is discarded)

Tile boundary is not changed from the original position. In this case, column width is derived as follows.

colWidth[ 0 ] = 8/3 = 2

colWidth[ 1 ] = 6/3 = 3

colWidth[ 2 ] = 3/1 = 3

When right tile column is discarded as shown in Figure 8, PicWidthInCtbsY = 7 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 2, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1.

Figure 8: Example of tile extraction (right tile column is discarded)

In this case, column width is derived as follows.

colWidth[ 0 ] = 7/3 = 2

colWidth[ 1 ] = 5/2 = 2

colWidth[ 2 ] = 3/1 = 3

When both right and left tile columns are discarded as shown in Figure 9, PicWidthInCtbsY = 5 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 1, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1.

Figure 9: Example of tile extraction (right tile column is discarded)

In this case, column width is derived as follows.

colWidth[ 0 ] = 5/2 = 2

colWidth[ 1 ] = 3/1 = 3

Option1-b: width and height are calculated by rounding up

Each tile size (colWidth[ i ], rowHeight[j]) is derived as follows.

colWidth[i] = ceil(remainWidthInCtbs/remaingTileColumns)	(round up)

remainWidthInCtbs -= colWidth[i], remaingTileColumns--

rowHeight[j] = ceil(remainHeightInCtbs/remaingTileRows)		(round up

remaingHeightInCtbsY -= rowHeight[j], remaingTileRows--

PPS syntax is the same as Option 1-a as shown in Table 3. This is the same as HEVC syntax.

Column width and row height are derived as follows.

if( uniform_tile_spacing_flag ) {
　　　remainingWidthInCtbsY = PicWidthInCtbsY

remainingTileColumns = tile_cols_width_minus1 + 1

i = 0
　　　while( remainingTileColumns > 0 ) {
	colWidth[ i ] = ceil(remainingWidthInCtbsY/ remainingTileColumns)
	remainingWidthInCtbsY  −= colWidth[ i ]

remainingTileColumns--

i++
　　　}
}

if( uniform_tile_spacing_flag ) {
      remainingHeightInCtbsY = PicHeightInCtbsY

remainingTileRows = tile_rows_height_minus1 + 1

i = 0
      while( remainingTileRows > 0 ) {

rowHeight[ i ] = ceil(remainingHeightInCtbsY/ remainingTileRows)
	remainingHeightInCtbsY  −= rowHeight[ i ]

remainingTileRows—

i++

}
}

For example, when PicWidthInCtbsY = 10, PicHeightInCtbsY = 7, uniform_tile_spacing_flag = 1, tile_cols_width_minus1 = 3 and tile_rows_height_minus1 = 2, tile partition will be as follows.

Figure 10: An example of tile partition of Option 1-b (uniform_tile_spacing_flag = 1)

In case of Figure 10, column width is derived as follows.

colWidth[ 0 ] = ceil(10/4) = 3

colWidth[ 1 ] = ceil(7/3) = 3

colWidth[ 2 ] = ceil(4/2) = 2

colWidth[ 3 ] = ceil(2/1) = 2

When left tile column is discarded as shown in Figure 11, PicWidthInCtbsY = 7 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 2, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1.

Figure 11: Example of tile extraction (left tile column is discarded)

Tile boundary is not changed from the original position. In this case, column width is derived as follows.

colWidth[ 0 ] = ceil(7/3) = 3

colWidth[ 1 ] = ceil(4/2) = 2

colWidth[ 2 ] = ceil(2/1) = 2

When right tile column is discarded as shown in Figure 12, PicWidthInCtbsY = 8 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 2, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1.

Figure 12: Example of tile extraction (right tile column is discarded)

In this case, column width is derived as follows.

colWidth[ 0 ] = ceil(8/3) = 3

colWidth[ 1 ] = ceil(5/2) = 3

colWidth[ 2 ] = ceil(3/1) = 3

When both right and left tile columns are discarded as shown in Figure 13, PicWidthInCtbsY = 5 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 1, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1.

Figure 13: Example of tile extraction (right tile column is discarded)

In this case, column width is derived as follows.

colWidth[ 0 ] = ceil(5/2) = 3

colWidth[ 1 ] = ceil(3/1) = 3

Option1-c

Option 1-c is another variant. In this case, the flag to indicate which Options (1-a or 1-b) is used to derive tile width and height. The PPS syntax is shown as below.

| pic_parameter_set_rbsp( ) { |  |
| --- | --- |
| ・・・・ |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| uniform_tile_spacing_mode | u(1) |
| } else { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| ・・・・ |  |

Table 4: PPS syntax for Option 1-a and 1-b

Option 2

In Option 2, a flag is added to identify if bitstream supports tile extraction without changing tile boundary. When bitstream does not support tile extraction without changing tile boundary, tile partitioning method is the same as HEVC. When HEVC bitstream is transcoded to VVC bitstream, it is not necessary to change its tile partitioning. When it is supported, the original tile boundary is signaled in PPS.

PPS syntax is shown below.

| pic_parameter_set_rbsp( ) { |  |
| --- | --- |
| ・・・・ |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| uniform_tile_spacing_extract_column_flag | u(1) |
| if( uniform_tile_spacing_extract_column_flag ) { |  |
| uniform_tile_spacing_extract_column_start_offset | ue(v) |
| uniform_tile_spacing_extract_width_in_ctb | ue(v) |
| uniform_tile_spacing_extract_tile_column_offset | ue(v) |
| } |  |
| uniform_tile_spacing_extract_row_flag | u(1) |
| if( uniform_tile_spacing_extract_row_flag ) { |  |
| uniform_tile_spacing_extract_row_start_offset | ue(v) |
| uniform_tile_spacing_extract_height_in_ctb | ue(v) |
| uniform_tile_spacing_extract_tile_row_offset | ue(v) |
| } |  |
| } else { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| ・・・・ |  |

Table 5: PPS syntax for Option 2

The semantics of the additional syntax elements are as follows.

uniform_tile_spacing_extract_column_flag: A flag to identify if bitstream supports tile extraction without changing vertical tile boundary

uniform_tile_spacing_extract_column_start_offset: The location that can be extracted tile column

uniform_tile_spacing_extract_width_in_ctb: The number of removing CTB columns of tile column extraction

uniform_tile_spacing_extract_tile_column_offset: The number of removing tile columns of tile column extraction

uniform_tile_spacing_extract_row_flag: A flag to identify if bitstream supports tile extraction without changing horizontal tile boundary

uniform_tile_spacing_extract_row_start_offset: The location that can be extracted tile row

uniform_tile_spacing_extract_height_in_ctb: The number of removing CTB rows of tile row extraction

uniform_tile_spacing_extract_tile_row_offset: The number of removing tile rows of tile row extraction

Column width and row height are derived as follows.

if( uniform_tile_spacing_flag ){

PicWidthInCtbsY2 = PicWidthInCtbsY + uniform_tile_spacing_extract_width_in_ctb

numTileColumns2 =  num_tile_columns_minus1 + 1

+ uniform_tile_spacing_extract_tile_column_offset

for( i = 0; i  <=  num_tile_columns_minus1; i++ ){

i2 = i +  uniform_tile_spacing_extract_column_start_offset

ColWidth[ i ] = ( ( i2+ 1 ) * PicWidthInCtbsY2 ) /  numTileColumns2  − 
						( i2 * PicWidthInCtbsY2 ) /  numTileColumns2

}

}
if( uniform_tile_spacing_flag ){

PicHeightInCtbsY2 = PicHeightInCtbsY + uniform_tile_spacing_extract_height_in_ctb

numTileRows2 = num_tile_rows_minus1 + 1 + uniform_tile_spacing_extract_tile_row_offset

for( j = 0; j  <=  num_tile_rows_minus1; j++ ){

j2 = j + uniform_tile_spacing_extract_row_start_offset

RowHeight[ j ] = ( ( j2 + 1 ) * PicHeightInCtbsY2 ) /  numTileRows2 − 
						( j2 * PicHeightInCtbsY2 ) / numTileRows2

}

}

if (!uniform_tile_spacing_extract_column_flag){

uniform_tile_spacing_extract_column_start_offset = 0

uniform_tile_spacing_extract_width_in_ctb = 0

uniform_tile_spacing_extract_tile_column_offset = 0

}

if (!uniform_tile_spacing_extract_row_flag){

uniform_tile_spacing_extract_row_start_offset = 0

uniform_tile_spacing_extract_height_in_ctb = 0

uniform_tile_spacing_extract_tile_row_offset = 0

}

For example, when PicWidthInCtbsY = 10, PicHeightInCtbsY = 7, uniform_tile_spacing_flag = 1, tile_cols_width_minus1 = 3 and tile_rows_height_minus1 = 2, uniform_tile_spacing_extract_column_flag = 0, uniform_tile_spacing_extract_row_flag = 0, tile partition will be as follows.

Figure 14: An example of tile partition of Option 2 (uniform_tile_spacing_flag = 1)

In case of Figure 14, column width is derived as follows.

colWidth[ 0 ] = (1*10)/4 – (0*10)/4 = 2

colWidth[ 1 ] = (2*10)/4 – (1*10)/4 = 3

colWidth[ 2 ] = (3*10)/4 – (2*10)/4 = 2

colWidth[ 3 ] = (4*10)/4 – (3*10)/4 = 3

When left tile column is discarded as shown in Figure 15, PicWidthInCtbsY = 8 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 2, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1, uniform_tile_spacing_extract_column_flag = 1, uniform_tile_spacing_extract_column_start_offset = 1, uniform_tile_spacing_extract_width_in_ctb = 2, uniform_tile_spacing_extract_tile_column_offset = 1, uniform_tile_spacing_extract_row_flag = 0.

Figure 15: Example of tile extraction (left tile column is discarded)

Tile boundary is not changed from the original position. In this case, column width is derived as follows.

colWidth[ 0 ] = ((1+1)*(8+2))/(3+1) – ((0+1)*(8+2))/(3+1) = 3

colWidth[ 1 ] = ((2+1)*(8+2))/(3+1) – ((1+1)*(8+2))/(3+1) = 2

colWidth[ 2 ] = ((3+1)*(8+2))/(3+1) – ((2+1)*(8+2))/(3+1) = 3

When right tile column is discarded as shown in Figure 16, PicWidthInCtbsY = 7 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 2, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1, uniform_tile_spacing_extract_column_flag = 1, uniform_tile_spacing_extract_column_start_offset = 0, uniform_tile_spacing_extract_width_in_ctb = 3, uniform_tile_spacing_extract_tile_column_offset = 1, uniform_tile_spacing_extract_row_flag = 0.

Figure 16: Example of tile extraction (right tile column is discarded)

In this case, column width is derived as follows.

colWidth[ 0 ] = (1*(7+3))/(3+1) – (0*(7+3))/(3+1) = 2

colWidth[ 1 ] = (2*(7+3))/(3+1) – (1*(7+3))/(3+1) = 3

colWidth[ 2 ] = (3*(7+3))/(3+1) – (2*(7+3))/(3+1) = 2

When both right and left tile columns are discarded as shown in Figure 17, PicWidthInCtbsY = 5 and PicHeightInCtbsY = 7, num_tile_columns_minus1 = 1, num_tile_rows_minus1 = 2, uniform_spacing_flag = 1, uniform_tile_spacing_extract_column_flag = 1, uniform_tile_spacing_extract_column_start_offset = 1, uniform_tile_spacing_extract_width_in_ctb = 5, uniform_tile_spacing_extract_tile_column_offset = 2, uniform_tile_spacing_extract_row_flag = 0.

Figure 17: Example of tile extraction (right tile column is discarded)

In this case, column width is derived as follows.

colWidth[ 0 ] = ((1+1)*(5+5))/(2+2) – ((0+1)*(5+5)/(2+2) = 3

colWidth[ 1 ] = ((2+1)*(5+5))/(2+2) – ((1+1)*(5+5))/(2+2) = 2

Uniform spacing for Brick

The same approach can be applied to Brick. The PPS syntax for Option 1-a and 1-b is as follows.

pic_parameter_set_rbsp( ) {

・・・・

brick_splitting_present_flag

for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) {

brick_split_flag[ i ]

if( brick_split_flag[ i ] ) {

num_brick_rows_minus1[ i ]

uniform_brick_spacing_flag[ i ]

if( !uniform_brick_spacing_flag[ i ] ){

for( j = 0; j < num_brick_rows_minus1[ i ]; j++ )

brick_row_height_minus1[ i ][ j ]

}

}

}

Table 6: PPS syntax for Brick (Option 1-a and 1-b)

The Brick row height is derived as follows for Option 1-a.

if( uniform_brick_spacing_flag[ i ] ) {
       remainingHeightInCtbsY = RowHeight[ tileY ]

remainingTileRows = num_brick_rows_mius1[i] + 1
       j = 0
      while( remainingTiileRows > 0 ) {
               rowHeight2[ j ] = remainingHeightInCtbsY/ remainingTileRows
               remainingHeightInCtbsY  −=rowHeight2[ j ]

remainingTileRows—

j++

}
}

In case of Option 1-b, Brick row height is derived as follows.

if( uniform_brick_spacing_flag[ i ] ) {
       remainingHeightInCtbsY = RowHeight[ tileY ]

remainingTileRows = num_brick_rows_mius1[i] + 1
       j = 0
      while( remainingTiileRows > 0 ) {
               rowHeight2[ j ] = ceil(remainingHeightInCtbsY/ remainingTileRows)
               remainingHeightInCtbsY  −=rowHeight2[ j ]

remainingTileRows—

j++

}
}

Conclusions

This contribution proposes the improved tile partitioning methods with uniform tile spacing for equally balanced parallel processing. In our method, tile boundary is not changed by extraction of tiles (or discarding tiles). Tile extraction (or discarding tiles) can be done with few changes in PPS and can be done by simply discarding VCL NAL unit for tiles

- Option1: Improved tile partitioning method with uniform tile spacing

- Option2: Add a flag to identify if bitstream supports tile extraction without changing tile boundary. When bitstream does not support tile extraction without changing tile boundary, tile partitioning method is the same as HEVC. When it is supported, the original tile boundary is signaled in PPS.

Those methods can be applied to Brick partitioning with uniform spacing.

References

[1] JVET-N0498, “AHG12: On Uniform Tile Partitioning”, R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).