| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0359 |
| --- | --- |

| Title: | AHG17: On identification of reference and non-reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

With current VVC specification, there is no syntax to indicate whether the current picture is reference picture or non-reference picture. In this document, new NAL units are proposed to address this issue.

Introduction

In the current VVC design, there is no syntax to indicate whether the current picture is reference picture or non-reference picture. If there is no such syntax element at higher layer of syntax, it is hard to identify if the current picture can be discarded. This was discussed in the 14th JVET meeting in Geneva, but no consensus was achieved.

Proposal

First octet of NAL unit header without forbidden_zero_bit was proposed in JVET-N0823. As in JVET-N0823 v1, NAL unit types to identify reference or non-reference TRAL NUT should be added as below.

| NalUnit Type Category | Name of NalUnitType | nuh_first_octet | Content of NAL unit and RBSP structure | NAL unit type class |
| --- | --- | --- | --- | --- |
| 0 | AUD_NUT | 00000 TID | Access unit delimiter access_unit_delimiter_rbsp( ) | Non VCL |
| 1 | TRAIL_R_NUT | 00001 TID | Coded tile group of non-STSA trailing picture, reference, tile_group_layer_rbsp( ) | VCL |
| 2 | TRAIL_N_NUT | 00010 TID | Coded tile group of a non- STSA trailing picture, non-reference tile_group_layer_rbsp( ) | VCL |
| 3 | STSA_R_NUT | 00011 TID | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 4 | STSA_N_NUT | 00100 TID | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 5 | RASL_NUT | 00101 TID | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 6 | RADL_NUT | 00110 TID | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 7..9 | RSV_VCL_7 .. RSV_VCL_9 | 00111 TID .. 01001 TID | Reserved non-IRAP VCL NAL unit types | VCL |
| 10 | APS_NUT | 01010 TID | Adaptation parameter set adaptation_parameter_set_rbsp( ) | Non VCL |
| 11 | PREFIX_SEI_NUT | 01011 TID | Supplemental enhancement information sei_rbsp( ) | Non VCL |
| 12 | POSTFIX_SEI_NUT | 01100 TID | Supplemental enhancement information sei_rbsp( ) | Non VCL |
| 13..15 | RSV_NVCL_13 .. RSV_NVCL_15 | 01101 TID .. 01111 TID | Reserved non-VCL NAL unit types | Non VCL |

References

[1] JVET-N0823, “AHG17: First octet of NAL unit header without forbidden_zero_bit”, S. Wenger, B. Choi (Tencent), M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)

Conclusions

With current VVC specification, there is no syntax to indicate whether the current picture is reference picture or non-reference picture. In this document, new NAL units (TRAIL_N_NUT and TRAIL_R_NUT) are proposed to address this issue.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).