| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0361-v4 |
| --- | --- |

| Title: | Non-CE4: Remove the Redundancy among Flag coding for Merge Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

Since only one merge mode can be adopted for each CU, the coding of one certain flag can be skipped if all the remaining modes can be ensured to be disabled. Based on the idea, the redundancy among the flag coding is removed in this contribution. It is reported that the modification brings 0.22%, 0.21%, and 0.23% gains for RA, LB, and LP configurations respectively compared to VTM-5.0.

Introduction

Currently, there are five modes for merge mode, namely, regular merge mode, MMVD mode, sub-block mode, CIIP mode, and triangle mode. And four flags are used to signalling these modes. For one certain flag, if all the remaining modes can be ensured to be disabled, there is no need to code/decode the flag.

Proposed Modification

When the conditions for coding/decoding the flag signalling the current mode have been satisfied, the following strategy should be also checked to infer whether the flag is equal to 1 without CABAC process.

For regular merge mode flag, its coding can be skipped if the following conditions are satisfied.

a) If current mode is skip mode, the Condition 1 or the Condition 2 is satisfied.

Condition 1: The SPS level control flag for MMVD is 0, and the area of current block is equal to 32.

Condition 2: The SPS level control flag for MMVD is 0, and both of Condition 3 and Condition 4 are satisfied.

Condition 3: The SPS level control flags for Affine and SbTMVP are 0, or MaxNumSubblockMergeCand is smaller than 1, or the width or height of current block is equal to 4.

Condition 4: The SPS level control flags for triangle mode is 0 or MaxNumTriangleMergeCand is smaller than 2, or current slice type is P slice.

b) If current mode is merge mode, the Condition 1 or the Condition 5 is satisfied.

Condition 5: The SPS level control flag for MMVD is 0, and all of Condition 3, Condition 4, and Condition 6 are satisfied.

Condition 6: The SPS level control flags for CIIP mode is 0, or the width or height of current block is equal to 128.

For MMVD mode flag, its coding can be skipped if the following conditions are satisfied:

a) If current mode is skip mode, one of the following two conditions is satisfied.

- The area of current block is equal to 32.

- Condition 3 and Condition 4 are satisfied.

b) If current mode is merge mode, one of the following two conditions is satisfied.

- The area of current block is equal to 32.

- Condition 3, Condition 4, and Condition 6 are satisfied.

For sub-block merge mode flag, its coding can be skipped if the following conditions are satisfied:

a) If current mode is skip mode, Condition 4 is satisfied.

b) If current mode is merge mode, Condition 4 and Condition 6 are satisfied.

For CIIP mode flag, its coding can be skipped if Condition 4 is satisfied.

To simplify the text modification, the above method can be summarized as:

if(allowRegularMerge&&(allowMMVD | | allowSubblock | | allowCIIP | | allowTriangle))

decode regular_merge_flag

else if ( allowMMVD && ( allowSubblock | | allowCIIP | | allowTriangle ))

decode mmvd_merge_flag

else if (allowSubblock && ( allowCIIP | | allowTriangle ))

decode merge_subblock_flag

else if (allowCIIP && allowTriangle)

decode ciip_flag

And the values of variables allowRegularMerge, allowMMVD, allowSubblock, allowCIIP, and allowTriangle are defined as follows.

allowRegularMerge = MaxNumMergeCand > 0

allowMMVD = sps_mmvd_enabled_flag

allowSubblock = (sps_affine_enabled_flag ¦¦ sps_sbtmvp_enabled_flag) && MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8

allowCIIP = sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth*cbHeight >= 64 && cbWidth < 128 && cbHeight < 128

allowTriangle = sps_triangle_enabled_flag && MaxNumTriangleMergeCand >= 2 && slice_type = = B && cbWidth*cbHeight >= 64

If the related flag is not decoded, its value can be inferred as follows.

regular_merge_flag = general_merge_flag

mmvd_merge_flag = allowMMVD && general_merge_flag && !regular_merge_flag

merge_subblock_flag = allowSubblock && general_merge_flag && !regular_merge_flag && !mmvd_merge_flag

ciip_flag = allowCIIP && general_merge_flag && !regular_merge_flag && !mmvd_merge_flag && !merge_subblock_flag

MergeTriangleFlag = allowTriangle && general_merge_flag && !regular_merge_flag && !mmvd_merge_flag && !merge_subblock_flag && !ciip_flag

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform. The RA, LB, and LP configurations specified in [1] with disabling triangle mode, are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

In the implementation, the CIIP may be enabled for skip mode in the encoder, thus this is not allowed in Test 1, and allowed in Test 2 as a comparison. The results of Test 1 and Test 2 are given in Table 1 and Table 2. It is reported that the modification in Test 1 brings 0.22%, 0.21%, and 0.23% gains for RA, LB, and LP configurations respectively compared to VTM-5.0. And Test 2 brings 0.20%, 0.22%, and 0.22% gains for RA, LB, and LP configurations respectively compared to VTM-5.0.

Table 1 Experimental Results of Test 1

Table 2 Experimental Results of Test 2

Conclusion

In this contribution, the redundancy among the flag coding is removed. The modification brings significant coding gains. Based on the above results, it is suggested to adopt them into the VTM.

Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).