- Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(allowRegularMerge&&(allowMMVD ¦ ¦ allowSubblock ¦ ¦ allowCIIP ¦ ¦ allowTriangle) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(allowMMVD && ( allowSubblock ¦ ¦ allowCIIP ¦ ¦ allowTriangle ) ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(allowSubblock && ( allowCIIP ¦ ¦ allowTriangle )) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(allowCIIP && allowTriangle) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

And the values of variables allowRegularMerge, allowMMVD, allowSubblock, allowCIIP, and allowTriangle are defined as follows.

allowRegularMerge = MaxNumMergeCand > 0

allowMMVD = sps_mmvd_enabled_flag

allowSubblock = (sps_affine_enabled_flag ¦¦ sps_sbtmvp_enabled_flag) && MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8

allowCIIP = sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth*cbHeight >= 64 && cbWidth < 128 && cbHeight < 128

allowTriangle = sps_triangle_enabled_flag && MaxNumTriangleMergeCand >= 2 && slice_type = = B && cbWidth*cbHeight >= 64

- Merge data semantics

regular_merge_flag[ x0 ][ y0 ] equal to 1 specifies that regular merge mode is used to generate the inter prediction parameters of the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When regular_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_mmvd_enabled_flag is equal to 0.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- cbWidth*cbHeight is equal to 32 allowRegularMerge is equal to 1.

- Otherwise, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

mmvd_merge_flag[ x0 ][ y0 ] equal to 1 specifies that merge mode with motion vector difference is used to generate the inter prediction parameters of the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When mmvd_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_mmvd_enabled_flag allowMMVD is equal to 1.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- cbWidth*cbHeight is equal to 32.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

merge_subblock_flag[ x0 ][ y0 ] specifies whether the subblock-based inter prediction parameters for the current coding unit are inferred from neighbouring blocks. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_subblock_flag[ x0 ][ y0 ] is not present, , it is inferred as follows:

–	If all the following conditions are true, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 1.

–	general_merge_flag[ x0 ][ y0 ] is equal to 1.

–	regular_merge_flag[x0][y0] is equal to 0.

–	mmvd_merge_flag[x0][y0] is equal to 0.

–	allowSubblock is true.

–Otherwise, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 0.

ciip_flag[ x0 ][ y0 ] specifies whether the combined inter-picture merge and intra-picture prediction is applied for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When ciip_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

–	If all the following conditions are true, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 1:

–	general_merge_flag[ x0 ][ y0 ] is equal to 1.

–	regular_merge_flag[x0][y0] is equal to 0.

–	mmvd_merge_flag[x0][y0] is equal to 0.

–	Merge_subblock_flag[x0][y0] is equal to 0.

–	allowCIIP is true.

–Otherwise, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When ciip_flag[ x0 ][ y0 ] is equal to 1, the variable IntraPredModeY[ x ][ y ] with x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to INTRA_PLANAR.

The variable MergeTriangleFlag[ x0 ][ y0 ], which specifies whether triangular shape based motion compensation is used to generate the prediction samples of the current coding unit, when decoding a B slice. is derived as follows:

- If all the following conditions are true, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 1:

- sps_triangle_enabled_flag allowTriangle is equal to 1.

- slice_type is equal to B.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- MaxNumTriangleMergeCand is greater than or equal to 2.

- cbWidth * cbHeight is greater than or equal to 64.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- mmvd_merge_flag[ x0 ][ y0 ] is equal to 0.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- ciip_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 0.