| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0362-v1 |
| --- | --- |

| Title: | Non-CE4: Modified Enabling Condition for Triangle Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

Currently, the triangle prediction mode could be enabled for block with width or height equal to 128. However, the VPDU size is 64x64, and the different weighting matrices should be kept for different VPDUs when triangle prediction unit mode is enabled for these large blocks. Hence, in this contribution, the size condition of the triangle prediction mode is modified as that both the width and the height of current block should be less than 128. It is reported that the modification bring 0.02% gains and 0.03% loss for RA and LB configurations respectively compared to VTM-5.0.

Introduction

Currently, the triangle prediction mode is enabled for block with the area equal or larger than 64. Thus this mode could be enabled for block with width or height equal to 128, in which case the prediction unit has at least two VPDUs. In the blending process of triangle prediction mode, the weighting matrices need to be used. And for block with width or height equal to 128, the weighting matrices for different VPDUs are different. Thus, this requires additional matrices storage when processing different VPDUs.

Proposed Modification

To address the problem, the size condition of the triangle prediction mode is modified as that both the width and the height of current block should be less than 128, and the related text is modified as:

- If all the following conditions are true, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 1:

- sps_triangle_enabled_flag is equal to 1.

- slice_type is equal to B.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- MaxNumTriangleMergeCand is greater than or equal to 2.

- cbWidth * cbHeight is greater than or equal to 64.

- cbWidth is smaller than 128 and cbHeight is smaller than 128.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- mmvd_merge_flag[ x0 ][ y0 ] is equal to 0.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- ciip_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 0.

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform. The RA and LB configurations specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

The results are given as Table 1. It is reported that the modification has 0.02% gains and 0.03% loss for RA and LB configurations respectively compared to VTM-5.0.

Table 1 Results of Test A

Conclusion

In this contribution, we have a modification on enabling condition of triangle prediction mode. The modification brings no coding loss and help to save storage of the weighting matrices cross VPDUs. Based on the above results, it is suggested to adopt them into the VTM.

Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).