| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0363-v1 |
| --- | --- |

| Title: | CE3-related: Context Reduction for intra MPM related syntax element |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | L. Xu X. Cao F. Chen L. Wang | Tel: Email: | +86-18100191186 xuliying5@hikvision.com caoxiaoqiang@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution proposed to simplify the context modelling and reduces the number of contexts for CABAC for syntax intra_luma_not_planar_flag to simplify the coding process. With the context reduction, the BD-rate increase is reported as 0.00% and 0.00% for AI and RA, respectively.

Introduction

The syntax intra_luma_not_planar_flag represents if the intra prediction mode is Planar or not. In the current scheme, intra_luma_not_planar_flag is coded by CABAC.

Table 1 the original assignment of ctxInc to syntax elements with context coded bins

| Syntax element | Bin Idx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| intra_luma_not_planar_flag[ ][ ] | intra_subpartitions_mode_flag | na | na | na | na | na |

Proposed scheme

Method1

In the proposed scheme, intra_luma_not_planar_flag is coded by CABAC with one context.

Table 2 the first proposed assignment of ctxInc to syntax elements with context coded bins

| Syntax element | Bin Idx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| intra_luma_not_planar_flag[ ][ ] | 0 | na | na | na | na | na |

Method2

In the proposed scheme, intra_luma_not_planar_flag is coded by bypass.

Table 3 the second proposed assignment of ctxInc to syntax elements with context coded bins

| Syntax element | Bin Idx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| intra_luma_not_planar_flag[ ][ ] | Bypass | na | na | na | na | na |

Experimental Result

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform. The CTC specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

The proposed technique is implemented on VTM5.0 and tested under All Intra, and Random Access configurations. The results are shown in Table 4-5. It can be observed that the proposed method almost has no impact on the coding performance.

Test1: One context for intra_luma_not_planar_flag

Test2: Bypass coding for intra_luma_not_planar_flag

Test1

Table 4

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | -0.02% | 101% | 100% |
| Class A2 | 0.01% | -0.04% | -0.05% | 101% | 100% |
| Class B | 0.00% | 0.01% | 0.07% | 100% | 102% |
| Class C | -0.01% | -0.07% | 0.05% | 101% | 104% |
| Class E | -0.01% | -0.07% | -0.07% | 100% | 103% |
| Overall | 0.00% | -0.02% | 0.01% | 101% | 102% |
| Class D | -0.01% | 0.05% | -0.07% | 101% | 108% |
| Class F | 0.01% | -0.01% | 0.05% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.15% | 100% | 100% |
| Class A2 | 0.03% | 0.04% | 0.09% | 100% | 100% |
| Class B | -0.01% | -0.05% | -0.12% | 100% | 101% |
| Class C | -0.02% | -0.05% | -0.05% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.01% | 100% | 101% |
| Class D | 0.05% | -0.04% | 0.41% | 101% | 102% |
| Class F | 0.01% | 0.01% | 0.14% | 100% | 101% |

Test2

Table 5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.14% | 0.08% | 101% | 101% |
| Class A2 | 0.07% | 0.03% | 0.08% | 100% | 99% |
| Class B | 0.10% | 0.05% | 0.11% | 101% | 101% |
| Class C | 0.10% | 0.09% | 0.15% | 101% | 103% |
| Class E | 0.05% | 0.04% | 0.03% | 100% | 104% |
| Overall | 0.08% | 0.07% | 0.09% | 101% | 102% |
| Class D | 0.00% | 0.10% | 0.00% | 101% | 103% |
| Class F | 0.13% | 0.18% | 0.19% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.09% | -0.04% | 100% | 100% |
| Class A2 | 0.04% | 0.07% | 0.12% | 100% | 100% |
| Class B | 0.07% | 0.01% | 0.00% | 100% | 101% |
| Class C | 0.01% | -0.03% | -0.01% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.01% | 0.01% | 100% | 101% |
| Class D | 0.00% | -0.05% | 0.26% | 101% | 102% |
| Class F | 0.07% | -0.03% | 0.18% | 100% | 101% |

Conclusions

This contribution proposes to reduce the complexity of entropy coding and the number of CABAC contexts. The two modifications almost have no impact on the coding efficiency.

References

[1] F. Bossen, J. Boyce, X. Li, et al. “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).