| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0365-v1 |
| --- | --- |

| Title: | CE3-related: Simplification of Multi-reference line prediction scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | L. Xu X. Cao F. Chen L. Wang | Tel: Email: | +86-18100191186 xuliying5@hikvision.com caoxiaoqiang@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

Firstly, this contribution proposes to disable the MRL for a 4x4 block as it would be a lot of computation to get the final reference line while the coding impact is quite small.

Secondly, this contribution proposed to simplify the context modelling and reduces the number of contexts for CABAC by using bypass coding to replace the CABAC coding for the second bin for intra_luma_ref_idx coding to simplify the coding process.

Introduction

In the current scheme, the max ref line is 3 lines. The intra_luma_ref_idx coding uses two contexts to code the syntax element.

Table 1 specification of IntraLumaRefLineIdx[ x ][ y ] based on intra_luma_ref_idx[ x0 ][ y0 ]

| intra_luma_ref_idx[ x0 ][ y0 ] | IntraLumaRefLineIdx[ x ][ y ] x = x0..x0 + cbWidth − 1 y = y0..y0 + cbHeight − 1 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 3 |

The assignment of ctxInc to syntax elements with context coded bins is listed as Table2.

Table 2 the original assignment of ctxInc to syntax elements with context coded bins

| Syntax element | Bin Idx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| intra_luma_ref_idx[ ][ ] | 0 | 1 | na | na | na | na |

Proposed scheme

In the proposed scheme, the second bin of intra_luma_ref_idx is coded by bypass.

Table 3 The proposed assignment of ctxInc to syntax elements with context coded bins

| Syntax element | Bin Idx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| intra_luma_ref_idx[ ][ ] | 0 | Bypass | na | na | na | na |

Experimental Result

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform. The CTC specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

The proposed technique is implemented on VTM5.0 and tested under All Intra, and Random Access configurations. The results are shown in Table 4 to Table 5. It can be observed that the proposed method almost has no impact on the coding performance.

The experimental result is listed as Table 4 and Table 5.

Test1: Disable 4x4 for MRL

Test2: Reduce context by using bypass to replace CABAC to code the second bin.

Test1

Table 4

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.01% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | -0.01% | 100% | 99% |
| Class B | 0.00% | -0.06% | 0.08% | 100% | 101% |
| Class C | -0.01% | 0.06% | 0.01% | 101% | 104% |
| Class E | -0.01% | -0.02% | -0.01% | 101% | 103% |
| Overall | 0.00% | -0.01% | 0.02% | 101% | 102% |
| Class D | 0.00% | 0.11% | -0.13% | 100% | 105% |
| Class F | 0.00% | 0.07% | 0.02% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | 0.01% | 99% | 101% |
| Class A2 | 0.02% | 0.05% | 0.02% | 99% | 100% |
| Class B | -0.01% | -0.10% | -0.21% | 99% | 100% |
| Class C | -0.03% | -0.20% | -0.22% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.09% | -0.12% | 99% | 100% |
| Class D | 0.03% | 0.16% | 0.15% | 100% | 101% |
| Class F | 0.01% | -0.05% | 0.01% | 100% | 102% |

Test2

Table 5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | -0.05% | -0.02% | 100% | 99% |
| Class B | 0.01% | -0.06% | 0.03% | 101% | 102% |
| Class C | 0.01% | -0.07% | 0.04% | 102% | 103% |
| Class E | 0.01% | -0.09% | -0.04% | 100% | 103% |
| Overall | 0.01% | -0.06% | 0.01% | 101% | 101% |
| Class D | 0.00% | 0.08% | -0.04% | 101% | 103% |
| Class F | 0.00% | -0.04% | 0.02% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.16% | 0.07% | 100% | 100% |
| Class A2 | 0.04% | 0.09% | -0.01% | 100% | 99% |
| Class B | 0.02% | -0.02% | -0.03% | 100% | 101% |
| Class C | -0.04% | -0.10% | -0.19% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.05% | -0.05% | 100% | 101% |
| Class D | 0.03% | -0.02% | 0.05% | 101% | 102% |
| Class F | 0.02% | 0.11% | 0.11% | 100% | 101% |

Conclusions

This contribution proposes to reduce the complexity of entropy coding and the number of CABAC contexts. Besides, the MRL is disabled for 4x4. The two modifications almost have no impact on the coding efficiency.

References

[1]  F. Bossen, J. Boyce, X. Li, et al. “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).