| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0367 |
| --- | --- |

| Title: | Non-CE4 : The proposed BCW index derivation for pairwise candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | [LG Electronics Inc.] |  |  |

_____________________________

- Abstract

In regular merge mode, the BCW index of spatial neighbor block is inherited when current CU is bi-predicted block. And, BCW index of the pairwise candidate is set to default mode (i.e. equal weight). The pairwise candidate has similar characteristics with affine constructed mode because this candidate is constructed by combining between merge candidates. Hence, derivation methods of BCW index for pairwise candidate are proposed for consistency in derivation design as follows.

- CASE 1: BCW indices of the two candidates for pairwise candidate are used.

- CASE 2: BCW index of the first candidate in the merge list is used.

The experimental results for CASE1 reportedly show 0.01% (-0.02%) BD-rate changes with 100% (100%) and 100% (100%) encoding and decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA(LB) configurations. And the experimental results for CASE2 reportedly show 0.02% (-0.01%) BD-rate changes with 101% (101%) and 100% (100%) encoding and decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA(LB) configurations.

Proposed method

The pairwise candidate [1] is constructed with motion vectors from the first and the second merge candidates and the BCW index of the pairwise candidate is set to default value (i.e., equal weight). But, pairwise candidate and affine constructed candidate has similar characteristics considering that these candidates are generated by combining candidates. For consistency in design, two methods for BCW index derivation are proposed.

CASE 1: BCW index depends on whether BCW index of two candidates are the same.

As shown below, BCW index is set to the BCW index of the first candidate if BCW index of two candidates are the same. If not, BCW index is set to default value.

if (bcwIdx0 == bcwIdx1) bcwIdx = bcwIdx0 else bcwIdx = Default value

CASE 2: BCW index is simply set to the BCW index of the first candidate in the merge list as follow.

bcwIdx = bcwIdx0

Experimental results

The proposed method has been implemented on top of the VTM5.0 with bug-fix (Ticket #312) with default setting and tested using JVET SDR common test condition [2]. The experimental results for CASE1 reportedly show 0.01% (-0.02%) BD-rate changes with 100% (100%) and 100% (100%) encoding and decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA(LB) configurations. And the experimental results for CASE2 reportedly show 0.02% (-0.01%) BD-rate changes with 101% (101%) and 100% (100%) encoding and decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA(LB) configurations.

CASE 1: BCW index depends on whether BCW index of two candidates are the same.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with Ticket#312 bug-fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.26% | 0.10% | 100% | 100% |
| Class A2 | 0.04% | -0.05% | 0.02% | 100% | 100% |
| Class B | 0.01% | 0.02% | -0.08% | 100% | 100% |
| Class C | 0.00% | -0.05% | 0.09% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.02% | 100% | 100% |
| Class D | 0.01% | 0.08% | -0.02% | 100% | 100% |
| Class F | 0.00% | 0.03% | 0.01% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with Ticket#312 bug-fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.10% | -0.23% | 100% | 99% |
| Class C | 0.01% | 0.08% | -0.19% | 100% | 100% |
| Class E | -0.04% | -0.39% | -0.20% | 101% | 101% |
| Overall | -0.02% | -0.11% | -0.21% | 100% | 100% |
| Class D | -0.04% | 0.07% | -0.31% | 101% | 102% |
| Class F | -0.02% | -0.41% | -0.66% | 100% | 101% |

CASE 2: BCW index is simply set to the BCW index of the first candidate in the merge list as follow.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with Ticket#312 bug-fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.20% | 0.20% | 101% | 100% |
| Class A2 | 0.06% | 0.01% | 0.04% | 101% | 100% |
| Class B | 0.00% | 0.02% | 0.07% | 101% | 100% |
| Class C | 0.01% | -0.03% | 0.01% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.04% | 0.07% | 101% | 100% |
| Class D | 0.01% | 0.12% | -0.08% | 100% | 100% |
| Class F | 0.00% | 0.03% | 0.01% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with Ticket#312 bug-fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.07% | -0.13% | 100% | 99% |
| Class C | -0.03% | 0.27% | -0.31% | 101% | 101% |
| Class E | 0.04% | -0.07% | -0.63% | 101% | 100% |
| Overall | -0.01% | 0.04% | -0.32% | 101% | 100% |
| Class D | 0.02% | 0.43% | -0.01% | 101% | 101% |
| Class F | -0.03% | -0.22% | -0.85% | 101% | 100% |

Conclusion

In this contribution, BCW index derivation for pairwise candidate is proposed for consistency in design. The experimental results reportedly show negligible changes in BD-rate and encoding/decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA/LB configurations. It is recommended to consider this method in the next VTM.

Reference

- Yu-Ling Hsiao, et al., “CE4.4.12: Pairwise average candidates”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0090, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).