| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0368-v4 |
| --- | --- |

| Title: | Non-CE6: An MTS-based Restriction for LFNST beyond Transform Skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Adarsh K. Ramasubramonian, Amir Said, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In VTM-5.0, the Low-Frequency Non-separable Transformation (LFNST) is disabled for the transform skip, and it can be used with all other MTS candidates. This contribution proposes a further restriction on LFNST, where it is enabled with 2-D DCT-2 only (i.e., only when MTS index is 0). It is asserted that the transformation process is simplified with the proposed method, and the experimental results show that the proposed changes provide slight coding gains in RA configuration and do not lead to a significant loss in AI configuration. On average, the proposed normative restrictions lead to coding loss of 0.07% in AI and gain of 0.04% in RA over VTM-5.0 under common test conditions (CTC). With further encoder changes, the BD-rate performance of 0.00% in AI and gain of 0.05% in RA is achieved.

# Introduction

In VTM-5.0, the multiple transform selection (MTS) consists of 6 transform candidates signalled using an index, namely mts_index, where:

- mts_index = 0 corresponds to 2-D DCT-2, applying DCT-2 both vertically and horizontally,

- mts_index = 1 corresponds to transform skip (TS),

- mts_index = 2 corresponds to applying DST-7 both vertically and horizontally,

- mts_index = 3 corresponds to applying DST-8 vertically and DST-7 horizontally,

- mts_index = 4 corresponds to applying DST-7 vertically and DST-8 horizontally, and

- mts_index = 5 corresponds to applying DCT-8 both vertically and horizontally.

For low-frequency non-separable transformation (LFNST), an index called lfnst_index is used to determine the non-separable transform selected among a set of 2 transform candidates, where the set is inferred based on the block size and intra mode used to predict the block.

In VTM-5.0, among other restrictions, LFNST is normatively disabled if mts_index is equal to 1, whereas LFNST can be used for other MTS candidates. This contribution proposes to use LFNST only with 2-D DCT-2 (i.e., only if MTS index is 0).

As will be shown in the experimental results on VTM-5.0, in terms of coding efficiency, there is no benefit of using LFNST for MTS candidates other than 2-D DCT-2. Since DCT-2 has faster implementations as compared to DST-7 and DCT-8, both worst-case and average latency in encoding/decoding processes can be reduced with the proposed approach. Thus, the latency due to the additional transform stage of LFNST can be alleviated.

# Proposed methods

In this contribution, it is proposed to normatively disable LFNST for all MTS candidates except 2-D DCT-2. In other words, LFNST is applied only if mts_index is set to 0. Otherwise, LFNST is not applied. Based on this scheme, the following tests are presented.

Test 1: The proposed restriction is implemented non-normatively (at encoder only).

Test 2: The proposed restriction is implemented normatively (at both encoder and decoder).

Test 3: The proposed normative restriction is implemented with the following additional encoder-only changes. The VTM-5.0 software modifies the MTS candidate search by testing each MTS candidate, and comparing with a threshold to determine whether the subsequent MTS candidates are to be tested. In Test 3, instead of testing one MTS candidate each time and comparing to a threshold, two MTS candidates are tested, and the better cost is compared with the threshold to determine whether subsequent MTS candidates are to be tested.

VTM-5.0:

int    startMTSIdx       [ 4 ] = { 0, 1, 2, 3 };
int    endMTSIdx         [ 4 ] = { 0, 1, 2, 3 };

Proposed encoder change:

int    startMTSIdx       [ 2 ] = { 0, 2 };
int    endMTSIdx         [ 2 ] = { 1, 3 };

# Experimental results

The proposed three tests discussed in the previous section are implemented in VTM-5.0, and the results are presented in the following tables. Note that low-delay B results are omitted, since LFNST is disabled in CTC for low-delay B.

Table 1: Results of Test 1 over VTM-5.0 under CTC.

Table 2: Results of the proposed normative changes (Test 2) over VTM-5.0 under CTC.

Table 3: Results of Test 3 over VTM-5.0 under CTC.

# Conclusions

This document proposes to add the restriction that LFNST is only enabled when 2-D DCT2 primary transforms are used (i.e., mts_index is equal to 0). Compared to VTM-5.0, the restrictions result in lower complexity with negligible BD-rate loss, or with some encoder changes minor BD-rate gains with negligible complexity difference. The proposal reduces the average and worst-case latency in the encoding/decoding processes with respect to the secondary transform stage. It is proposed to adopt this restriction in VVC.

# Draft Text Updates

The working draft was updated on top of JVET-N1001-v8, and the proposed changes are highlighted in yellow.

#### Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( cbWidth = = 4 && cbHeight = = 4 && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && sps_ibc_enabled_flag && ( cbWidth != 128 ¦ ¦ cbHeight != 128 ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| … |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 && !( tu_cbf_luma[ x0 ][ y0 ] && tu_mts_idx[ x0 ][ y0 ] != 0 ) ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Additional Results

Tables 4, 5 and 6 present additional experimental results over adopted JVET-O0094 (CE6-2.1a). The proposed method is implemented on top of CE6-2.1a software and tested.

Variations in encoding and decoding time measures (EncT and DecT) are due to use of multiple types of cores available on the cluster. It is claimed that the EncT and DecT measures should not significantly differ from the results in Table 1, 2 and 3.

Table 4: Results of Test 1 over JVET-O0094 CE6-2.1a under CTC.

Table 5: Results of Test 2 over JVET-O0094 CE6-2.1a under CTC.

Table 6: Results of Test 3 over JVET-O0094 CE6-2.1a under CTC.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th meeting, Geneva, CH, 19-27 March 2019.

- M. Koo, M. Salehifar, J. Lim, S. Kim, “CE6: Reduced Secondary Transform (RST)(CE6-3.1)”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip”, JVET-M0464, 13th meeting, Marrakech, MA, Jan. 2019.

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-L1026, 12th meeting, Macao, CN, Oct. 2018.

- X. Zhao and H. E. Egilmez, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-M1026, 13th meeting, Marrakech, MA, Jan. 2019.

- X. Zhao, H. E. Egilmez, “CE6: Summary Report on Transforms and Transform Signaling”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

- M. Siekmann, H. Schwarz, D. Marpe, T. Wiegand, “CE6-2.1: Simplification of Low Frequency Non-Separable Transform”, JVET-O0094, 15th meeting, Gothenburg, SW, 2-13 July 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).