| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0370-v1 |
| --- | --- |

| Title: | CE6-related: Applying LFNST Only for Top-Left Subblocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Alican Nalci, Amir Said, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VTM-5.0, depending on the block size, the Low-Frequency Non-separable Transformation (LFNST) can be either applied to a single subblock located at the top-left or applied to two subblocks (i.e., top-left blocks and its adjacent block). This contribution document proposes to remove the latter case, so the LFNST is applied to a single top-left subblock only. The experimental results show that the proposed changes do not lead to a significant loss in coding efficiency. On average, 0.02% AI and 0.01% RA luma coding losses are observed over VTM-5.0 under common test conditions (CTC).

# Description of the Method

In VTM-5.0, the low-frequency non-separable transformation (LFNST) design has the following block-size dependency on the number of subblocks transformed using LFNST:

- LFNST is applied to two subblocks (see Fig.1 for an example) if a TU is of size 4xN/Nx4 for N≥16,

- Otherwise, LFNST is applied to a single subblock located at top left.

In this contribution, the special case of applying LFNST to two subblocks is removed as depicted in Fig.2.

Fig.1: An example of LFNST applied to two (red) subblocks.

Fig. 2: Proposed simplification applying LFNST to the top-left (red) subblock for the case in Fig.1.

# Experimental results

The proposed method is implemented in VTM-5.0, and the results are presented in Table 1. Note that low-delay B results are omitted, since LFNST is disabled in CTC for low-delay B.

Table 1: Results of Test 1 over VTM-5.0 under CTC.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.02% | 100% | 99% |
| Class A2 | 0.01% | -0.03% | 0.01% | 99% | 100% |
| Class B | 0.02% | -0.03% | 0.05% | 100% | 100% |
| Class C | 0.04% | 0.02% | 0.05% | 99% | 100% |
| Class E | 0.02% | 0.06% | -0.03% | 99% | 101% |
| Overall | 0.02% | 0.01% | 0.03% | 100% | 100% |
| Class D | 0.03% | 0.01% | 0.00% | 100% | 101% |
| Class F | -0.01% | 0.03% | 0.13% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.06% | 100% | 101% |
| Class A2 | 0.02% | 0.04% | 0.14% | 100% | 101% |
| Class B | 0.02% | -0.06% | -0.26% | 100% | 100% |
| Class C | 0.01% | -0.20% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.08% | -0.06% | 100% | 100% |
| Class D | 0.02% | 0.13% | 0.22% | 99% | 99% |
| Class F | -0.02% | -0.10% | 0.04% | 99% | 100% |

# Conclusions

This contribution document introduces a simplification for LFNST by removing a block-size dependent special case, where a non-separable transform is applied to two subblock. Since this special case do not provide a substantial benefit in terms of coding gain, its removal is proposed and recommended for adoption in VVC.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th meeting, Geneva, CH, 19-27 March 2019.

- M. Koo, M. Salehifar, J. Lim, S. Kim, “CE6: Reduced Secondary Transform (RST)(CE6-3.1)”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-L1026, 12th meeting, Macao, CN, Oct. 2018.

- X. Zhao and H. E. Egilmez, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-M1026, 13th meeting, Marrakech, MA, Jan. 2019.

- X. Zhao, H. E. Egilmez, “CE6: Summary Report on Transforms and Transform Signaling”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

# Patent rights declaration(s)

Qualcomm Technology Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).