| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0372-v1 |
| --- | --- |

| Title: | Non-CE6: A Simplification for MTS Index Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Hilmi E. Egilmez, Amir Said, Muhammed Coban, Vadim Seregin and Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | analci@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a simplification for CABAC contexts when signaling multiple transform selection (MTS) index in VTM-5.0. Specifically, dependency between the MTS index signaling and the CU level quad-tree depth (cqtDepth) is removed. This reduces the number of total CABAC contexts by 5. Negligible gains/losses are observed for both luma and chroma, with average BD rates of  (0.00%, -0.06% -0.01%) for all intra (AI), (-0.01%, -0.12% -0.05%) for random access (RA), and (0.02%, 0.04% 0.22%) for low delay B (LDB) under common test conditions (CTC).

# Introduction

In VTM-5.0, multiple transform selection (MTS) is used for residual coding for both inter and intra coded blocks. MTS determines an appropriate set of transforms to be applied on the residual signal from a list of available candidates; including DCT-2, DCT8 and DST-7. The appropriate transform set is then signaled using an MTS index (mtsIdx or tu_mts_idx[][]) at the transform unit (TU) level.

Currently, whether a DCT-2 transform or an alternative transform applied is context coded based on the value of quad-tree depth (i.e. syntax element: cqtDepth). This dependency is highlighted in Table 1 with yellow color. A maximum of 5 contexts are allocated for quad-tree depth.

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx[ ][ ] | cqtDepth (5 contexts) | 6 | 7 | 8 | na | na |

Table 1. Assignment of ctxInc to syntax elements with context coded bins

# Proposed method

In this proposal, we remove the quad-tree depth dependency when signaling the MTS index. That is whether a DCT-2 transform or an alternative transform applied is context coded with the same context index (ctxInc) of ‘0’ regardless of the quad-tree depth.

On the code below the proposed change can be reflected as:

void CABACWriter::mts_coding( const TransformUnit& tu, ComponentID compID )

{

…

symbol = tu.mtsIdx != MTS_DCT2_DCT2 ? 1 : 0;

ctxIdx = 0;

ctxIdx = std::min( (int)cu.qtDepth, 5 );

m_BinEncoder.encodeBin( symbol, Ctx::MTSIndex( ctxIdx ) );

…

}

where the removed part of code is crossed out with red color and the new code is shown with green color.

Moreover, the proposed change can be reflected on the JVET-N1001-v3 document as follows:

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx[ ][ ] | cqtDepth 0 | 1 | 2 | 3 | na | na |

Table 2. Proposed assignment of ctxInc to syntax elements with context coded bins

where the removed syntax element is crossed out with red color and the new syntax is shown with green color.

# Experimental Results

The proposed method is tested over VTM-5.0 on top of CTC configurations. These results are presented below for AI, RA, LDB cases through Tables 3-5 after context retraining. We also provide supplementary results in Tables 6-7 for AI and RA without context retraining.

## After Context Retraining

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.03% | 100% | 102% |
| Class A2 | 0.00% | -0.05% | -0.04% | 102% | 106% |
| Class B | 0.00% | -0.05% | 0.01% | 101% | 100% |
| Class C | 0.00% | -0.11% | -0.03% | 101% | 101% |
| Class E | -0.01% | -0.04% | -0.05% | 101% | 103% |
| Overall | 0.00% | -0.06% | -0.01% | 101% | 102% |
| Class D | 0.00% | 0.05% | -0.12% | 100% | 98% |
| Class F | -0.01% | 0.02% | 0.06% | 100% | 99% |

Table 3. AI results on top of CTC over VTM5.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.15% | 0.11% | 102% | 103% |
| Class A2 | 0.01% | -0.05% | -0.07% | 98% | 99% |
| Class B | 0.00% | 0.06% | -0.11% | 100% | 102% |
| Class C | -0.04% | -0.36% | -0.08% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.12% | -0.05% | 100% | 100% |
| Class D | -0.05% | 0.19% | -0.07% | 100% | 102% |
| Class F | -0.05% | 0.13% | -0.06% | 98% | 98% |

Table 4. RA results on top of CTC over VTM5.0

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.04% | 0.14% | 99% | 98% |
| Class C | 0.04% | -0.01% | 0.05% | 99% | 94% |
| Class E | 0.01% | -0.20% | 0.56% | 100% | 97% |
| Overall | 0.02% | -0.04% | 0.22% | 100% | 96% |
| Class D | 0.04% | -0.07% | 0.09% | 101% | 99% |
| Class F | -0.06% | -0.11% | 0.40% | 96% | 95% |

Table 5. LDB results on top of CTC over VTM5.0

## Without Context Retraining (Supplementary)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.03% | 101% | 100% |
| Class A2 | 0.00% | -0.05% | -0.06% | 101% | 100% |
| Class B | 0.01% | -0.04% | 0.07% | 100% | 100% |
| Class C | 0.00% | -0.08% | 0.05% | 101% | 100% |
| Class E | -0.01% | -0.02% | -0.04% | 100% | 100% |
| Overall | 0.00% | -0.04% | 0.01% | 100% | 100% |
| Class D | 0.02% | 0.01% | -0.09% | 100% | 100% |
| Class F | -0.01% | -0.04% | -0.09% | 101% | 100% |

Table 6. AI results on top of CTC over VTM5.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | 0.09% | 100% | 100% |
| Class A2 | 0.01% | 0.13% | -0.01% | 100% | 99% |
| Class B | 0.00% | -0.13% | -0.14% | 100% | 100% |
| Class C | -0.08% | 0.01% | -0.15% | 100% | 100% |
| Class E | -0.04% | 0.11% | 0.14% | 100% | 100% |
| Overall | -0.02% | -0.03% | -0.04% | 100% | 100% |
| Class D | -0.02% | 0.23% | 0.14% | 100% | 101% |
| Class F | 0.00% | -0.05% | 0.03% | 100% | 101% |

Table 7. RA results on top of CTC over VTM5.0

# Conclusions

In this contribution, context simplification/reduction for MTS index signaling is proposed. This approach removes the CU level quad-tree depth dependency when coding the MTS index and reduces the number of contexts for MTS index by 5.

It is recommended to adopt the proposed method in VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th meeting, Geneva, CH, 19-27 March 2019.

- J. Chen, T. Ye and S. H. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5),” JVET-N1002, 14th Meeting, Geneva, CH, Mar. 2019.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).