| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0374-v2 |
| --- | --- |

| Title: | CE5-related: Context Reduction for ALF related Syntax Element |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | L. Xu X. Cao F. Chen L. Wang | Tel: Email: | +86-18100191186 xuliying5@hikvision.com caoxiaoqiang@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

Abstract

This contribution proposed to simplify the context modelling and reduces the number of contexts for CABAC. The CABAC context modelling for alf_ctb_flag is investigated. With the contexts reduction, the BD-Rate increase for method 1 is reported as 0.00% 0.00% and 0.03% for AI, RA and LDB configuration, the BD-Rate increase for method 2 is reported as 0.00% 0.05% and 0.05% for AI, RA and LDB configuration, and the BD-Rate increase for method 3 is reported as 0.00% 0.06% and 0.00% for AI, RA and LDB configuration.

Introduction

Table 1 The original assignment of ctxInc to syntax elements with context coded bins

| Syntax element | condL | condA | ctxSetIdx |
| --- | --- | --- | --- |
| alf_ctb_flag[ x0 ][ y0 ][ cIdx ] | alf_ctb_flag[ xNbL ][ yNbL ][ cIdx ] | alf_ctb_flag[ xNbA ][ yNbA ][cIdx ] | cIdx |

CtxInc is calculated as following formula.

ctxInc = ( condL  &&  availableL ) + ( condA  &&  availableA ) + ctxSetIdx * 3

For this method, there will be 9 contexts used to code the syntax element.

Proposed scheme

Method1

Table 2 The proposed assignment of ctxInc to syntax elements with context coded bins

| Syntax element | condL | condA | ctxSetIdx |
| --- | --- | --- | --- |
| alf_ctb_flag[ x0 ][ y0 ][ cIdx ] | alf_ctb_flag[ xNbL ][ yNbL ][ cIdx ] | alf_ctb_flag[ xNbA ][ yNbA ][cIdx ] | cIdx == 0 ？ 0 ： 1 |

CtxInc is calculated as following formula.

ctxInc = ( condL  &&  availableL ) + ( condA  &&  availableA ) + ctxSetIdx * 3

For this method, there will be 6 contexts used to code the syntax element.

Method2

For luma component, the CtxInc is calculated as formula.

Table 3 The proposed assignment of ctxInc to syntax elements with context coded bins

| Syntax element | condL | condA |
| --- | --- | --- |
| alf_ctb_flag[ x0 ][ y0 ][ cIdx ] | alf_ctb_flag[ xNbL ][ yNbL ][ cIdx ] | alf_ctb_flag[ xNbA ][ yNbA ][cIdx ] |

ctxInc = ( condL  &&  availableL ) + ( condA  &&  availableA )

For chroma component, another two contexts will be used.

For this method, there will be 5 contexts used to code the syntax.

Method3

Table 4 The proposed assignment of ctxInc to syntax elements with context coded bins

| Syntax element | condL | condA | ctxSetIdx |
| --- | --- | --- | --- |
| alf_ctb_flag[ x0 ][ y0 ][ cIdx ] | 0 | 0 | cIdx |

CtxInc is calculated as following fomula.

ctxInc = cIdx

For this method, there will be 3 contexts used to code the syntax.

In VTM-5.0, the context derivation process of alf_ctb_flag syntax element uses left and above neighboring syntax elements. In this method, the dependence is released.

Experimental Result

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform. The CTC specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

The proposed technique is implemented on VTM5.0 and tested under All Intra, and Random Access and Low delay B configurations. The results are shown in Table 5 to Table 7. It can be observed that the proposed method almost has no impact on the coding performance.

Test1: Experimental result for Method1

Test2: Experimental result for Method2

Test3: Experimental result for Method3

Test1

Table 5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.02% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | -0.01% | -0.01% | 101% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 105% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 107% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | 0.15% | 100% | 100% |
| Class A2 | 0.01% | 0.17% | 0.08% | 100% | 100% |
| Class B | 0.00% | 0.03% | -0.12% | 100% | 102% |
| Class C | 0.00% | 0.01% | -0.04% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.00% | 100% | 101% |
| Class D | 0.00% | -0.07% | -0.07% | 101% | 102% |
| Class F | -0.01% | 0.04% | -0.01% | 100% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.10% | -0.17% | 100% | 99% |
| Class C | 0.03% | 0.07% | 0.00% | 101% | 100% |
| Class E | 0.07% | 0.82% | -0.11% | 100% | 101% |
| Overall | 0.03% | 0.27% | -0.10% | 100% | 100% |
| Class D | -0.02% | -0.50% | 0.80% | 101% | 103% |
| Class F | -0.04% | 0.24% | -0.40% | 99% | 99% |

Test2

Table 6

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | -0.03% | -0.04% | 100% | 101% |
| Class C | 0.00% | -0.01% | -0.01% | 101% | 104% |
| Class E | 0.00% | -0.01% | -0.02% | 101% | 103% |
| Overall | 0.00% | -0.01% | -0.03% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class F | 0.00% | -0.01% | -0.02% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.15% | -0.16% | 100% | 100% |
| Class A2 | 0.06% | 0.00% | -0.28% | 100% | 100% |
| Class B | 0.06% | -0.36% | -0.42% | 100% | 100% |
| Class C | 0.05% | -0.30% | -0.25% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.23% | -0.29% | 100% | 100% |
| Class D | 0.04% | -0.13% | -0.10% | 101% | 103% |
| Class F | 0.05% | -0.26% | -0.24% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -1.78% | -1.44% | 100% | 98% |
| Class C | 0.08% | -0.83% | -1.03% | 100% | 99% |
| Class E | 0.03% | -2.13% | -3.03% | 100% | 99% |
| Overall | 0.05% | -1.55% | -1.70% | 100% | 99% |
| Class D | 0.11% | -0.84% | -0.34% | 100% | 102% |
| Class F | 0.08% | -1.49% | -1.80% | 100% | 99% |

Test3

Table 7

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.04% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | 0.00% | -0.03% | -0.04% | 100% | 101% |
| Class C | 0.00% | -0.01% | -0.01% | 101% | 103% |
| Class E | 0.00% | -0.01% | -0.02% | 100% | 103% |
| Overall | 0.00% | -0.01% | -0.03% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class F | 0.00% | -0.01% | -0.02% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.18% | -0.09% | 99% | 96% |
| Class A2 | 0.07% | 0.05% | -0.21% | 99% | 97% |
| Class B | 0.07% | -0.39% | -0.44% | 100% | 97% |
| Class C | 0.05% | -0.33% | -0.27% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.24% | -0.28% | 100% | 98% |
| Class D | 0.06% | -0.09% | -0.06% | 101% | 100% |
| Class F | 0.06% | -0.25% | -0.26% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -1.53% | -1.38% | 99% | 94% |
| Class C | 0.02% | -0.92% | -1.14% | 100% | 96% |
| Class E | -0.05% | -1.58% | -2.61% | 100% | 96% |
| Overall | 0.00% | -1.34% | -1.61% | 100% | 95% |
| Class D | -0.01% | -0.46% | -0.17% | 101% | 101% |
| Class F | -0.03% | -0.91% | -1.46% | 100% | 95% |

Conclusions

This contribution proposes some ways to reduce the CABAC context. The results are listed as following table.

As can be seen, the impact on the coding efficiency is acceptable. Besides, the coding gain for chroma components prove that using the left and above neighboring syntax elements is needless.

| Method | Nb contexts | AI | RA | LB |
| --- | --- | --- | --- | --- |
| Method1 | -3 | 0.00% 0.00% 0.00% | 0.00% 0.03% 0.00% | 0.03% 0.27% -0.10% |
| Method2 | -4 | 0.00% -0.01% -0.03% | 0.05% -0.23% -0.29% | 0.05% -1.55% -1.70% |
| Method3 | -6 | 0.00% -0.01% -0.03% | 0.06% -0.24% -0.28% | 0.00% -1.34% -1.61% |

References

[1] F. Bossen, J. Boyce, X. Li, et al. “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).