| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0375-v1 |
| --- | --- |

| Title: | Non-CE7: A Simplified CBF Coding for Luma and Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Hilmi E. Egilmez, Amir Said, Muhammed Coban, Vadim Seregin and Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | analci@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to remove the transform tree depth (i.e. trDepth) dependency in coded block flag (CBF) coding. In VTM-5.0 a total of 12 CABAC contexts for CBF coding for both luma and chroma. In this contribution, it is proposed to remove trDepth dependency when coding the CBF flag. As a result, the total number of CBF related contexts for luma and chroma is reduced by 2 (e.g. 1 for luma and 1 for Chroma). This approach results in BD rates of (0.00%, 0.00% 0.00%) for all intra (AI), (0.00%, -0.03% -0.03%) for random access (RA), and (0.03%, 0.17% 0.02%) for LDB under common test conditions (CTC).

# Introduction

In VTM-5.0, a coded block flag (CBF) is signaled for both luma (Y) and chroma (both Cb, and Cr) transform blocks. CBF flag specifies whether a respective transform block contains at least one non-zero coefficient. Currently, CBF coding depends on trDepth, which specifies that current transform block is smaller than current CU.

Below, Syntax Table 1 and Clause 1 summarizes the context derivation rules (ctxInc) for CBF coding for luma and chroma, where tree depth dependency is highlighted with yellow.

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_cbf | 0 | na | na | na | na | na |
| tu_cbf_luma[ ][ ][ ] | (Clause 1: Given Below) | na | na | na | na | na |
| tu_cbf_cb[ ][ ][ ] | trDepth = = 0 ? 0 : 1 | na | na | na | na | na |
| tu_cbf_cr[ ][ ][ ] | prevTuCbf (See Clause 1) | na | na | na | na | na |

Table 1. Assignment of ctxInc to syntax elements with context coded bins for CBF coding in VTM-5.0

##### Clause 1: Derivation process of ctxInc for the syntax element tu_cbf_luma

Inputs to this process are the variable binIdx, the colour component index cIdx,  current transform tree depth trDepth, intra bdpcm flag, and previous TU CBF flag (prev CBF) for luma. Output of this process is the context index variable ctxInc.

The variable ctxInc is derived as follows:

- If intra_bdpcm_flag[ ][ ]  flag is 1 then ctxInc is 4, otherwise

- If IntraSubpartitionsSplitType is equal to ISP_NO_SPLIT or block is not luma then,

ctxInc = trDepth = = 0  ?  1  :  0

- Otherwise ( IntraSubpartitionsSplitType is not equal to ISP_NO_SPLIT and block is not luma) then, the following applies:

  - The variable prevTuCbf is derived as follows: If the current transform unit is the first one to be parsed in a coding unit, prevTuCbf is set equal to 0. Otherwise, prevTuCbf is set equal to the value of the previous transform unit in the current coding unit.

- The variable ctxInc is derived as: ctxInc = 2 + prevTuCbf.

# Proposed method

In this proposal, we remove the contexts that depend on current transform depth (trDepth) for CBF coding for both luma and chroma. The simplified context modeling is shown in Syntax Table 2 and Clause 2 below, where we crossed out the removed syntax with red color, and added new syntax with green color.

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_cbf | 0 | na | na | na | na | na |
| tu_cbf_luma[ ][ ][ ] | (Clause 2: Given Below) | na | na | na | na | na |
| tu_cbf_cb[ ][ ][ ] | trDepth = = 0 ? 0 : 1 → 0 | na | na | na | na | na |
| tu_cbf_cr[ ][ ][ ] | prevTuCbf (See Clause 1) | na | na | na | na | na |

Table 2. Proposed assignment of ctxInc to syntax elements with context coded bins for CBF coding

##### Clause 2: Proposed Derivation process of ctxInc for the syntax element tu_cbf_luma

Inputs to this process are the variable binIdx, the colour component index cIdx,  current transform tree depth trDepth, intra bdpcm flag, and previous TU CBF flag (prev CBF) for luma. Output of this process is the context index variable ctxInc.

The variable ctxInc is derived as follows:

- If intra_bdpcm_flag[ ][ ]  flag is 1 then ctxInc is 4, otherwise

- If IntraSubpartitionsSplitType is equal to ISP_NO_SPLIT or block is not luma then,

ctxInc = trDepth = = 0  ?  1  :  0  → ctxInc = 0

- Otherwise ( IntraSubpartitionsSplitType is not equal to ISP_NO_SPLIT and block is not luma) then, the following applies:

  - The variable prevTuCbf is derived as follows: If the current transform unit is the first one to be parsed in a coding unit, prevTuCbf is set equal to 0. Otherwise, prevTuCbf is set equal to the value of the previous transform unit in the current coding unit.

- The variable ctxInc is derived as: ctxInc = 2 + prevTuCbf.

# Experimental Results

The proposed method is tested over VTM-5.0 on top of CTC configurations. These results are presented for AI, RA, and LDB cases in Tables 1 through 3 below.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 105% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 108% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 112% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 102% |

Table 3. AI results on top of CTC over VTM5.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.13% | 0.11% | 102% | 102% |
| Class A2 | 0.06% | -0.08% | 0.04% | 99% | 100% |
| Class B | 0.00% | 0.03% | -0.04% | 101% | 102% |
| Class C | -0.04% | 0.00% | -0.15% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.03% | 100% | 101% |
| Class D | 0.02% | 0.04% | 0.00% | 99% | 100% |
| Class F | -0.01% | -0.08% | -0.06% | 98% | 99% |

Table 4. RA results on top of CTC over VTM5.0

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.35% | 0.29% | 99% | 97% |
| Class C | 0.01% | -0.22% | 0.00% | 101% | 102% |
| Class E | 0.08% | 0.40% | -0.43% | 101% | 100% |
| Overall | 0.03% | 0.17% | 0.02% | 100% | 100% |
| Class D | 0.04% | 0.56% | -0.17% | 101% | 98% |
| Class F | -0.02% | -0.40% | 0.20% | 96% | 94% |

Table 5. LDB results on top of CTC over VTM5.0

# Conclusions

In this contribution, simplified coding for CBF flag is proposed. The simplification removed the transform tree depth dependency when coding the CBF flag for both luma and chroma and reduced the number of contexts by 2. This simplification results in BD rates of (0.00%, 0.00% 0.00%) for all intra (AI), (0.00%, -0.03% -0.03%) for random access (RA), and (0.03%, 0.17% 0.02%) for LDB under CTC conditions in VTM5.

It is recommended to adopt the proposed method in VVC.

# References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001, 14th Meeting, Geneva, CH, Mar. 2019.

- J. Chen, T. Ye and S. H. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5),” JVET-N1002, 14th Meeting, Geneva, CH, Mar. 2019.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).