| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0376-v3 |
| --- | --- |

| Title: | CE8-related: A SPS Level Flag For BDPCM and JCCR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | L. Xu X. Cao F. Chen | Tel: Email: | +86-18100191186 xuliying5@hikvision.com caoxiaoqiang@hikvision.com chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution proposed to add a SPS level flag to enable the BDPCM and JCCR tool. The BD-rate  is reported as 0.00% , 0.00% and 0.00% for AI , RA and LDB configuration, respectively.

Introduction

In the last meeting, Block-based quantized residual domain DPCM (BDPCM) is adopted and integrated into VTM5.0. While opening the BDPCM is controlled by configuration file. A SPS level flag is needed.

Besides, joint coding of chrominance residuals (JCCR) is also adopted and integrated into VTM5.0. A SPS level flag is also needed.

Proposed scheme

In the proposed scheme, a SPS flag sps_bdpcm_enabled_flag is added as a switch to open BDPCM.In the proposed scheme, a SPS flag sps_joint_cbcr_enabled_flag is added as a switch to open JCCR.

Method1

This part proposes to add a SPS switch for BDPCM.

Table 1

| if(sps_bdpcm_enable_flag && cbWidth <= 32 && cbHeight <= 32 ) |  |
| --- | --- |
| intra_bdpcm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bdpcm_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_dir_flag [ x0 ][ y0 ] | ae(v) |
| else { |  |

Method2

This part proposes to add a SPS switch for JCCR.

Table 2

| pps_cb_qp_offset | se(v) |
| --- | --- |
| pps_cr_qp_offset | se(v) |
| if (sps_joint_cbcr_enabled_flag) |  |
| pps_joint_cbcr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |

Table 3

if( tu_cbf_cr[ x0 ][ y0 ] ) {

if( tu_cbf_cb[ x0 ][ y0 ] && sps_joint_cbcr_enabled_flag )

tu_joint_cbcr_residual[ x0 ][ y0 ]

if( !tu_joint_cbcr_residual[ x0 ][ y0 ] )

residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 )

}

Experimental Result

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform. The CTC specified in [1] and [2] are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

The proposed technique is implemented on VTM5.0 and tested under All Intra, Random Access and Low delay B configurations. The results are shown in Table x. It can be observed that the proposed method almost has no impact on the coding performance.

Test1: Experimental result for BDPCM SPS switch

Test2: Experimental result for JCCR SPS switch

Test1

Table 4

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class SCC 1080p | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Random Access Main 10 - VTM configuration |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class SCC 1080p | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Low delay B Main10 - VTM configuration |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class SCC 1080p | 0.00% | 0.00% | 0.00% | 101% | 105% |

Test2

Table 5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |

Additional test is conducted for method1 under CTC. As BDPCM is disabled for ClassA1 to ClassE,

the bdpcm related syntax will never need to code such as intra_bdpcm_flag and intra_bdpcm_dir_flag.

Table 6

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | -0.02% | 100% | 99% |
| Class A2 | -0.03% | -0.06% | -0.03% | 100% | 99% |
| Class B | -0.04% | -0.13% | -0.04% | 100% | 99% |
| Class C | -0.04% | -0.06% | -0.05% | 101% | 100% |
| Class E | -0.05% | -0.07% | -0.01% | 100% | 99% |
| Overall | -0.04% | -0.08% | -0.03% | 100% | 99% |
| Class D | -0.04% | 0.02% | -0.01% | 101% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | 0.10% | 100% | 99% |
| Class A2 | -0.03% | 0.04% | 0.11% | 100% | 99% |
| Class B | 0.00% | 0.05% | -0.28% | 100% | 100% |
| Class C | -0.05% | -0.09% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.08% | 100% | 100% |
| Class D | 0.03% | 0.06% | 0.02% | 100% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Conclusions

This contribution proposed to add a SPS level flag to enable the BDPCM and JCCR tool.

References

[1]  F. Bossen, J. Boyce, X. Li, et al. “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

[2]    X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, JVET-N1028, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, Geneva, March 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).