      - Motion vector storing process for triangle merge mode

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable triangleDir specifying the partition direction,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

The variables numSbX and numSbY specifying the number of 4x4 blocks in the current coding block in horizontal and vertical direction are set equal to numSbX = cbWidth >> 2 and numSbY = cbHeight >> 2.

The variable minSb is set equal to min( numSbX, numSbY ).

The variable cbRatio is derived as follows:

cbRatio = ( cbWidth > cbHeight ) ? ( cbWidth / cbHeight ) : ( cbHeight / cbWidth )	(8-814)

The variable isTwoRefListIdentical specifying whether the two reference lists are totally the same, and it is derived as follows:

The variable isTwoRefListIdentical is set equal to TRUE:

If NumRefIdxActive[0] != NumRefIdxActive[1], isTwoRefListIdentical is set to FALSE; Otherwise:

For the variable refIdxm with m = 0…NumRefIdxActive[0]-1, only if RefPicList[0][m] is always euqal to RefPicList[1][m], isTwoRefListIdentical is set equal to TRUE; Otherwise, isTwoRefListIdentical is set equal to FALSE.

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xIdx and yIdx are derived as follows:

xIdx = ( cbWidth > cbHeight )  ?  ( xSbIdx / cbRatio ) : xSbIdx	(8-815)

yIdx = ( cbWidth > cbHeight )  ?  ySbIdx  :  ( ySbIdx / cbRatio )	(8-816)

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-817)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-818)

- Depending on the value of sType, the following assignments are made:

- If a) sType is equal to 0 or b) sType is equal to 2, predListFlagA is equal to predListFlagB, isTwoRefListIdentical is equal to FALSE, and triangleDir is equal to 0 , the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0		(8-819)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1		(8-820)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(8-821)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(8-822)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(8-823)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(8-824)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(8-825)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(8-826)

- Otherwise, if a) sType is equal to 1, or b) sType is equal to 2, predListFlagA is equal to predListFlagB, isTwoRefListIdentical is equal to FALSE, and triangleDir is equal to 1, the following applies:

predFlagL0 = ( predListFlagB = = 0 )  ?  1  :  0		(8-827)

predFlagL1 = ( predListFlagB = = 0 )  ?  0  :  1		(8-828)

refIdxL0 = ( predListFlagB = = 0 )  ?  refIdxB : −1	(8-829)

refIdxL1 = ( predListFlagB = = 0 )  ?  −1  :  refIdxB	(8-830)

mvL0[ 0 ] = ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0	(8-831)

mvL0[ 1 ] = ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0	(8-832)

mvL1[ 0 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]	(8-833)

mvL1[ 1 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]	(8-834)

- Otherwise (sType is equal to 2), the following applies:

  - If predListFlagA + predListFlagB is equal to 1,

predFlagL0 = 1		(8-835)

predFlagL1 = 1		(8-836)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA  :  refIdxB	(8-837)

refIdxL1 = ( predListFlagA = = 0 )  ?  refIdxB  :  refIdxA	(8-838)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  mvB[ 0 ]	(8-839)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  mvB[ 1 ]	(8-840)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  mvB[ 0 ]  :  mvA[ 0 ]	(8-841)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  mvB[ 1 ]  :  mvA[ 1 ]	(8-842)

  - If predListFlagA + predListFlagB is equal to 0, the following applies:

predFlagL0 = 1		(8-843)

predFlagL1 =	1                                                  (8-844)

refIdxL0 = refIdxA                            	(8-845)

refIdxL1 = refIdxB                     	(8-846)

mvL0[ 0 ] = mvA[ 0 ]                     	(8-847)

mvL0[ 1 ] = mvA[ 1 ] 	                                            (8-848)

mvL1[ 0 ] = mvB[ 0 ]    	                                         (8-849)

mvL1[ 1 ] = mvB[ 1 ]    	                                         (8-850)

  - If predListFlagA + predListFlagB is equal to 2, the following applies:

predFlagL0 = 1	                                                  (8-851)

predFlagL1 = 1		(8-852)

refIdxL0 = refIdxB       	                                        (8-853)

refIdxL1 =refIdxA                             	(8-854)

mvL0[ 0 ] = mvB[ 0 ]                         	(8-855)

mvL0[ 1 ] =mvB[ 1 ]   	                                          (8-856)

mvL1[ 0 ] =mvA[ 0 ]    	                                         (8-857)

mvL1[ 1 ] = mvA[ 1 ]    	                                         (8-858)

- The following assignments are made for x = 0..3 and y = 0..3:

MvL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL0	(8-859)

MvL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL1	(8-860)

RefIdxL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL0	(8-861)

RedIdxL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL1	(8-862)

PredFlagL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL0	(8-863)

PredFlagL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL1	(8-864)

-

-

-

-