| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0379-v1 |
| --- | --- |

| Title: | Non-CE4: Modified Mode Decision Process for Triangle Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

In VTM-5.0, the triangle prediction mode brings additional 8% and 12% encoding time for RA and LB configurations, respectively. Especially for LB configuration, the complexity is a heavy burden. In this contribution, a fast mode decision method of the triangle prediction mode is reported for encoder. This method forces the triangle prediction mode in the encoder to conduct rough RDO process to save the candidate number of true RDO modes. It is reported that the method saves 5% and 7% encoding time for RA and LB configurations respectively. And it brings 0.01% gains and 0.02% loss for RA and LB configurations respectively compared to VTM-5.0.

Introduction

For the triangle prediction mode in VTM-5.0, if the temporary best mode has no residual (the value bestIsSkip is true), the true RDO (with entropy coding) will be applied for all the candidates. Although in these cases, the residual coding could be skipped, it still requires much encoding time to choose the best among the large number of candidates (40 by default). However, if the value bestIsSkip is false, a rough RDO process will conducted for all the candidates, then only a few candidates (less or equal than 3 by default) will be selected in the true RDO process. To reduce the encoding time, the value bestIsSkip is always forced to be false even the temporary best mode has no residual.

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform. The RA and LB configurations specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

The results are given as Table 1. It is reported that the method save 5% and 7% encoding time for RA and LB configurations respectively. And it brings 0.01% gains and 0.02% loss for RA and LB configurations respectively compared to VTM-5.0.

Table 1 Experimental Results

Conclusion

In this contribution, to reduce the encoding time, the value bestIsSkip is forced to be false even the temporary best mode has no residual. The modification brings negligible coding loss and help to save much encoding time. Based on the above results, it is suggested to adopt them into the VTM.

Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).