| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0380-v3 |
| --- | --- |

| Title: | Non-CE6: Simplification on MTS index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoqiang Cao Fangdong Chen Li Wang | Tel: Email: | caoxiaoqiang@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution presents a simplification on MTS index coding, the proposed method removes the dependency with the depth of CU’s quartree partition for the first bin of MTS signalling. And for the last 2 bins are using bypass coding.

For the method 1, 5 contexts reduced, it is reported that the BD-rate are 0.00%, -0.02% and 0.03% for AI, RA and LB respectively. When enable inter MTS on (MTS=3), the BD-rate are -0.02% and -0.03% for RA and LB respectively.

For the method 2, 7 contexts and 2 context-coded bin are reduced, it is reported that the BD-rate are 0.00%, -0.02% and 0.04% for AI, RA and LB respectively. When enable inter MTS on (MTS=3), the BD-rate are -0.03% and -0.03% for RA and LB respectively.

Introduction

In current MTS design in VTM5.0[1], when signalling the first bin to indicate DCT2 and other transform pair, it depends on cu quadtree depth to encode and decode the first CABAC bin.

Table 1 – context modelling for MTS index in VTM5.0

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx | 0...5 (cqtDepth) | 6 | 7 | 8 | na | na |

Proposed Method

In this contribution, we remove the dependency on depth when signalling MTS index.

For the method 1, as show in table 2, for the first bin, the dependency with the depth of CU’s quartree partition are removed, the total number of contexts for MTS are reduced from 9 to 4, 5 contexts are reduced.

For the method 2, As show in table 3, for the first bin, the dependency with the depth of CU’s quartree partition are removed, for the last 2 bin are coded with bypass coding, the total number of contexts for MTS are reduced from 9 to 2, 7 contexts are reduced, and 2 context coded bin are reduced.

#method 1

Table 2 – context modelling for MTS index with proposed method 1

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx | 0 | 1 | 2 | 3 | na | na |

#method 2

Table 3 – context modelling for MTS index with proposed method 2

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx | 0 | 1 | bypass | bypass | na | na |

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform, the CTC specified in [2], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

Experimental results of method 1

Experimental results of method 2

Additional experimental results with InterMTS on

Additional results of method 1

Additional results of method 2

Conclusion

In this contribution, we have present two method to simplified MTS index coding, it removes the dependency on depth and uses bypass coding for the last two bin. 7 contexts and 2 context coded bin are reduced at most, From the results of proposed methods, no loss in AI configuration. It is suggested to adopt the proposed method into the VTM.

Reference

[1]	B. Bross, J. Chen, S. Liu. “Versatile Video Coding (Draft 5),” JVET-N1001-v8, Geneva, CH, 19–27 Mar. 2019.

[2]	F. Bossen, J. Boyce, X. Li, et al. “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).