| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0381-v1 |
| --- | --- |

| Title: | Non-CE6: Simplification on SBT coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoqiang Cao Fangdong Chen Li Wang | Tel: Email: | caoxiaoqiang@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution present two methods on simplification for SBT coding, both them reduced context coded bin for SBT.

The first method reduces 3 context coded bin by using bypass coding to replace CABAC based coding, and 5 contexts are removed, the BD-rate are 0.01% and 0.02% for RA and LB configuration respectively.

The second method reduces 3 context coded bin by using bypass coding to replace CABAC based coding, and 6 contexts are removed, the BD-rate are 0.01% and 0.00% for RA and LB configuration respectively.

Introduction

In current subblock transform design in VTM5.0[1], there have several related syntaxes to signal the SBT mode. cu_sbt_flag indicated that for current coding unit, SBT is used, cu_sbt_flag equal to 1 specifies that for the current coding unit, SBT is used. cu_sbt_flag equal to 0 specifies that for the current coding unit, SBT is not used. cu_sbt_quad_flag equal to 1 specifies that for the current coding unit, the subblock transform includes a transform unit of 1/4 size of the current coding unit. cu_sbt_quad_flag equal to 0 specifies that for the current coding unit the subblock transform includes a transform unit of 1/2 size of the current coding unit. cu_sbt_horizontal_flag equal to 1 specifies that the current coding unit is split horizontally into 2 transform units. cu_sbt_horizontal_flag equal to 0 specifies that the current coding unit is split vertically into 2 transform units. cu_sbt_pos_flag equal to 1 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the first transform unit in the current coding unit are not present in the bitstream. cu_sbt_pos_flag equal to 0 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the second transform unit in the current coding unit are not present in the bitstream. There are totally 7 contexts for SBT as in Table 1, and each syntax is coded with context coded bin.

Table 1 - Number of contexts for SBT related syntax in VTM5.0

| SBT Syntax element | binIdx | Number of contexts |
| --- | --- | --- |
| cu_sbt_flag | ( cbWidth * cbHeight < 256 ) ? 1 : 0 | 2 |
| cu_sbt_quad_flag | 0 | 1 |
| cu_sbt_horizontal_flag | ( cbWidth = = cbHeight ) ? 0 : ( cbWidth < cbHeight ) ? 1 : 2 | 3 |
| cu_sbt_pos_flag | 0 | 1 |
| Total contexts | 7 |  |

Proposed Method

Method 1

Only cu_sbt_flag coded with context which is as same as VTM5.0, other 3 additional flag are coded with bypass coding, so the total number of contexts for SBT are reduced from 7 to 2, 5 contexts are removed.

Table 2 – Number of contexts for SBT related syntax with proposed method 1

| SBT Syntax element | binIdx | Number of contexts |
| --- | --- | --- |
| cu_sbt_flag | ( cbWidth * cbHeight < 256 ) ? 1 : 0 | 2 |
| cu_sbt_quad_flag | Bypass | 0 |
| cu_sbt_horizontal_flag | Bypass | 0 |
| cu_sbt_pos_flag | Bypass | 0 |
| Total contexts | 2 |  |

Method 2

Compare with method 1, cu_sbt_flag with 1 context, other 3 additional flag are coded with bypass coding, so the total number of contexts for SBT are reduced from 7 to 1, 6 contexts are removed.

Table 3 – Number of contexts for SBT related syntax with proposed method 2

| SBT Syntax element | binIdx | Number of contexts |
| --- | --- | --- |
| cu_sbt_flag | 0 | 1 |
| cu_sbt_quad_flag | Bypass | 0 |
| cu_sbt_horizontal_flag | Bypass | 0 |
| cu_sbt_pos_flag | Bypass | 0 |
| Total contexts | 1 |  |

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform, the CTC specified in [2], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

Method1

The results for method 1 show only 0.01% loss in RA configuration, and only 0.02% loss for LB configuration.

- Anchor: VTM5.0(CTC)

- Test:   Method1(CTC)

Method2

The results for method 2 show only 0.01% loss in RA configuration, and no change of coding efficiency for LB configuration.

- Anchor: VTM5.0(CTC)

- Test:   Method2(CTC)

Method2 VS method1

Compare with method 1, method 2 also reduce 1 context for cu_sbt_flag, but there is minor gain of -0.01% both for RA and LB configuration.

- Anchor: method1(CTC)

- Test:   Method2(CTC)

Conclusion

In this contribution, we have present two methods to simplified SBT coding, both two methods are reduced 3 context coded bin for SBT, and at most 6 contexts are reduced, from the results of proposed methods, both two methods are having minor loss for SBT. Based on the above results, method 2 have better trade off than method 1, it is suggested to adopt method 2 into the VTM.

Reference

[1]	B. Bross, J. Chen, S. Liu. “Versatile Video Coding (Draft 5),” JVET-N1001-v8, Geneva, CH, 19–27 Mar. 2019.

[2]	F. Bossen, J. Boyce, X. Li, et al. “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).