| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0383_WD |
| --- | --- |

| Title: | Non-CE7: Harmonization of scaling matrix and LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Eiichi Sasaki Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-50-5213-6277 tomonori.hashimoto@sharp.co.jp eiichi.sasaki@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

Proposed Working Draft Changes

This document provides the proposed text of VVC Working Draft based on JVET-N1001-v8. The changes are highlighted in yellow.

…

- 8.7.2 	Scaling and transformation process

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Output of this process is the (nTbW)x(nTbH) array of residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-934)

bdShift = Max( 20 − bitDepth, 0 )		(8-935)

tsShift = 5 + ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )		(8-936)

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

- If cIdx is equal to 2 and tu_joint_cbcr_residual[ xTbY ][ yTbY ] is equal to 1, the residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

resSamples[ x ][ y ]  =  −resSamplesCb[ x ][ y ]		(8-937)

where resSamplesCb is the residual sample array resSamples that is generated in the last invokation of this clause for the Cb component block with cIdx euqal to 1 at the transform block location ( xTbY, yTbY ).

- Otherwise (cIdx is less than 2 or tu_joint_cbcr_residual[ xTbY ][ yTbY ] is equal to 0), the following ordered steps apply:

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

  - The (nTbW)x(nTbH) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  <<  tsShift	(8-938)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the transformation process for scaled transform coefficients as specified in clause 8.7.4.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples r.

  - The residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

resSamples[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-939)

- 8.7.3	Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(8-940)

- Otherwise, if cIdx is equal to 1 and tu_joint_cbcr_residual[ xTbY ][ yTbY ] is equal to 1, the following applies:

qP = Qp′CbCr		(8-941)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(8-942)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(8-943)

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-944)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The variables  bdShift, rectNorm and bdOffset are derived as follows:

bdShift = bitDepth + ( ( rectNonTsFlag  ?  1  :  0 ) + 		(8-945)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + dep_quant_enabled_flag

bdOffset = ( 1 << bdShift ) >> 1		(8-946)

The list levelScale[ ][ ] is specified as levelScale[ j ][ k ] = { { 40, 45, 51, 57, 64, 72 }, { 57, 64, 72, 81, 91, 102 } } with j = 0..1, k = 0..5.

The (nTbW)x(nTbH) array dz is set equal to the (nTbW)x(nTbH) array TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is set equal to 16.

- The intermediate scaling factor m[ x ][ y ] is set equal to 16derived as follows:

–	if one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- scaling_list_enabled_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0.

–	Otherwise, the following applies:

- m[ x ][ y ] = ScalingFactor2[ sizeIdW ][ sizeIdH ][ matrixId ][ x ][ y ]			       (8-XXX)

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(8-947)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << ( qP / 6 )	(8-948)

- When BdpcmFlag[ xTbY ][ yYbY ] is equal to 1, dz[ x ][ y ] is modified as follows:

–	If BdpcmDir[ xTbY ][ yYbY ] is equal to 0 and x is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x − 1 ][ y ] + dz[ x ][ y ])	(8-949)

–	Otherwise, if BdpcmDir[ xTbY ][ yYbY ] is equal to 1 and y is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x ][ y − 1 ] + dz[ x ][ y ])	(8-950)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = ( dz[ x ][ y ] * ls[ x ][ y ] +bdOffset )  >>  bdShift	(8-951)

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-952)

It is noted that the JVET-N1001-v8 has not incorporated scaling matrix process yet. In our proposal, scaling matrix process is performed in 8.7.4. Here we always use m[][] = 16 constantly. Compared to scaling matrix applied version is shown in yellow.

- 8.7.4	Second scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of modified scaled transform coefficients with elements d[ x ][ y ].

The variables  bdShift and bdOffset are derived as follows:

bdShift = 4 		(8-xxxx)

bdOffset = ( 1 << bdShift ) >> 1		(8-xxxx)

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The scaling factor m[ x ][ y ] is derived as follows:

–	m[ x ][ y ] = ScalingFactor2[ sizeIdW ][ sizeIdH ][ matrixId ][ x ][ y ]			       (8-XXX)

Where sizeIdW is set equal to Log2( nTbW ), sizeIdH is set equal to Log2( nTbH ) and matrixId is specified in Table 7-4 for sizeId set equal to Max( sizeIdW, sizeIdH ), CuPredMode[ xTbY ][ yTbY ] and cIdx, respectively.

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = 			(8-xxxx)
	( d[ x ][ y ] * m[ x ][ y ] +bdOffset )  >>  bdShift

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )

- 8.7.5	Transformation process for scaled transform coefficients

- 8.7.5.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, numLfnstX, numLfnstY, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(8-953)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(8-954)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-955)

nLfnstSize = 1  <<  log2LfnstSize		(8-956)

numLfnstX = ( nTbW > 8  &&  nTbH  = = 4 )  ?  2  :  1	(8-957)

numLfnstY = ( nTbW = = 4  &&  nTbH  > 8 )  ?  2  :  1	(8-958)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(8-959)

- The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

- For xSbIdx = 0..numLfnstX − 1 and ySbIdx = 0..numLfnstY − 1, the following applies:

- The values of the list u[ x ] with x = 0..nonZeroSize − 1 are derived as follows:

xC = ( xSbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 0 ]	(8-960)

yC = ( ySbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 1 ]	(8-961)

u[ x ] = d[ xC ][ yC ] 			(8-962)

- The one-dimensional low frequency non-separable transformation process as specified in clause 8.7.4.2 is invoked with the input length of the scaled transform coefficients nonZeroSize, the transform output length nTrS set equal to nLfnstOutSize, the list of scaled non-zero transform coefficients u[ x ] with x = 0..nonZeroSize − 1, the intra prediction mode for LFNST set selection predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnst_idx[ xTbY ][ yTbY ] as inputs, and the list v[ x ] with x = 0..nLfnstOutSize − 1 as output.

- The array d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] with x = 0..nLfnstSize − 1, y = 0..nLfnstSize − 1 is derived as follows:

- If predModeIntra is less than or equal to 34, or equal to INTRA_LT_CCLM, INTRA_T_CCLM, or INTRA_L_CCLM, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( y < 4 )  ?  v[ x + ( y << log2LfnstSize ) ]  : 	(8-963)
	( ( x < 4 )  ?  v[ 32 + x + ( ( y − 4 ) << 2 ) ]  :  d[ x ][ y ] )

- Otherwise, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( x < 4 )  ?  v[ y + ( x << log2LfnstSize ) ]  : 	(8-964)
	( ( y < 4 )  ?  v[ 32 + y + ( ( x − 4 ) << 2 ) ]  :  d[ x ][ y ] )

- When scaling_list_enabled_flag is equal to 1, The second scaling process for transform coefficients as specified in clause 8.7.4 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of modified scaled transform coefficients d.

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are both equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or both sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-965)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-966)

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-17 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-16 depending on tu_mts_idx[ xTbY ][ yTbY ].

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, ( trTypeHor > 0 )  ?  16  :  32 )	(8-967)

nonZeroH = Min( nTbH, ( trTypeVer > 0 )  ?  16  :  32 )	(8-968)

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- When nTbH is greater than 1, each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.4 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list d[ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- When nTbH and nTbW are both greater than 1, the intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(8-969)

- When nTbW is greater than 1, each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.4 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

…

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).