| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0383-v2 |
| --- | --- |

| Title: | Non-CE7: Harmonization of scaling matrix and LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Eiichi Sasaki Takeshi Chujoh Tomohiro Ikai | Tel: Email: | tomonori.hashimoto@sharp.co.jp eiichi.sasaki@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

Scaling matrix and LFNST have been adopted in VVC-5 draft. In VVC-5, scaling matrices are applied even in the case of LFNST mode, which is asserted to be wrong since LFNST’s coefficient does not correspond to conventional transform coefficients in spatial frequency meaning. This contribution proposes a fix for this problem which performs scaling matrices in LFNST case as well at the normal transform coefficient domain. Specifically this proposal applies scaling matrices after secondary transform in decoder side. The proposed scheme provides transform coefficient control capability in all conventional and LFNST transform cases. It is also reported that experimental result confirmed LFNST coding gain in scaling matrices is similar to that of non-scaling matrices.

In version 2, wrong statements was fixed.

Introduction

Scaling matrix [1] and low frequency non-separable transform (LFNST) [2] have been adopted in VVC-5. In VVC-5 and HEVC, scaling matrices is performed in the decoded transform coefficients which are to be input for secondary transform and core transform. In the current design, scaling matrices are applied even in the case of LFNST mode. This special case is contradicted to the transform coefficient control capability in spatial frequency domain, which should be provided by scaling matrix.

Because scaling matrices are designed based on conventional transform coefficients, scaling matrices must be performed in the normal transform domain.

This contribution proposes a scheme of scaling matrix which supports LFNST and resolves the problem that there are no transform coefficient control capability in LFNST case.

Proposed methods

Since scaling matrices are designed based on core transform coefficients or spatially frequency domain, we propose to perform scaling matrix process after secondary transform as 2nd scaling in Figure 1. Likewise, in encoder side, the scaling matrices are performed after core transform as 2nd scaling in Figure 2.

Figure 1: Scaling, transform and quantization order in decoder side

Figure 2: Scaling, transform and quantization order in encoder side

Therefore, the proposed scheme performs scaling process with scaling matrices for coefficients of core transform reasonably.

Experimental Results

The proposed method is implemented on top of VTM software including scaling matrix of JVET-N0847 [1]. It is noted that following software is used for code base, which means only scaling matrix is implemented:

VTM-5.0 (33e05832) + initial JVET-N0847 (difference between b7a117bb and 5e4ac7ea) + bugfix JVET-N0847 (difference between d4c54c39 and 81d3894a)

The simulations are conducted in the condition of ‘All Intra’, ‘Random Access’ defined in JVET-N1010 [3] with scalingLists.txt as attached in JVET-O0383 zip file.

Following Table1 and Table 2 show the results and Table 3 shows the performance of LFNST in the proposed method. These results show LFNST is effectively even though the coefficients are scaled with scaling matrices.

Table 1. Result of scaling matrix on case with proposed method and LFNST enabled 
(Test: Proposed + LFNST on, Anchor: VTM50 CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (Sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.89% | 2.30% | -0.24% | 95% | 107% |
| Class A2 | 2.67% | 0.25% | -0.02% | 92% | 107% |
| Class B | 2.03% | -1.49% | -1.80% | 97% | 109% |
| Class C | 2.78% | 1.00% | 0.96% | 89% | 122% |
| Class E | 1.80% | -0.40% | -0.52% | 100% | 113% |
| Overall | 2.24% | 0.17% | -0.42% | 95% | 112% |
| Class D | 4.54% | 2.34% | 2.36% | 90% | 151% |
| Class F | 1.24% | 2.39% | 2.35% | 96% | 113% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (Sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 1.44% | -0.87% | 98% | 105% |
| Class A2 | -0.19% | -0.04% | -0.63% | 95% | 106% |
| Class B | 1.02% | -1.56% | -2.05% | 99% | 109% |
| Class C | 1.70% | 0.26% | 0.27% | 97% | 132% |
| Class E |  |  |  |  |  |
| Overall | 0.76% | -0.17% | -0.91% | 97% | 113% |
| Class D | 2.35% | 0.23% | 0.22% | 97% | 195% |
| Class F | 0.94% | 2.24% | 2.21% | 99% | 125% |

Table 2. Result of scaling matrix on case with LFNST disabled 
(Test: Proposed + LFNST off, Anchor: VTM50 CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (Sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 3.83% | 2.84% | 0.27% | 74% | 107% |
| Class A2 | 3.90% | 0.13% | -0.03% | 69% | 107% |
| Class B | 3.03% | -1.15% | -1.37% | 74% | 107% |
| Class C | 4.28% | 1.74% | 1.89% | 68% | 118% |
| Class E | 3.21% | 0.94% | 0.38% | 79% | 110% |
| Overall | 3.62% | 0.72% | 0.14% | 73% | 110% |
| Class D | 5.76% | 3.21% | 3.62% | 68% | 145% |
| Class F | 2.59% | 2.98% | 2.89% | 81% | 110% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (Sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.42% | 2.56% | 0.74% | 84% | 105% |
| Class A2 | 0.43% | -0.20% | -0.61% | 83% | 106% |
| Class B | 1.53% | -0.79% | -1.09% | 85% | 108% |
| Class C | 2.53% | 0.69% | 0.65% | 81% | 129% |
| Class E |  |  |  |  |  |
| Overall | 1.55% | 0.39% | -0.17% | 83% | 112% |
| Class D | 2.94% | 0.83% | 1.21% | 82% | 188% |
| Class F | 1.61% | 3.03% | 3.02% | 85% | 123% |

Table 3. Result of LFNST gain when scaling matrix is enabled 
(Test: Proposed + LFNST on, Anchor: Proposed + LFNST off)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Proposed Method w/o LFNST |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.87% | -0.58% | -0.50% | 129% | 100% |
| Class A2 | -1.24% | 0.05% | -0.01% | 133% | 101% |
| Class B | -0.97% | -0.34% | -0.44% | 131% | 102% |
| Class C | -1.43% | -0.71% | -0.90% | 131% | 104% |
| Class E | -1.36% | -1.32% | -0.90% | 127% | 102% |
| Overall | -1.33% | -0.56% | -0.56% | 130% | 102% |
| Class D | -1.15% | -0.84% | -1.22% | 132% | 104% |
| Class F | -1.30% | -0.56% | -0.51% | 118% | 103% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Proposed Method w/o LFNST |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.37% | -1.10% | -1.59% | 117% | 100% |
| Class A2 | -0.62% | 0.16% | -0.04% | 115% | 100% |
| Class B | -0.51% | -0.80% | -0.98% | 116% | 100% |
| Class C | -0.81% | -0.43% | -0.37% | 119% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.78% | -0.57% | -0.75% | 117% | 101% |
| Class D | -0.58% | -0.59% | -0.98% | 118% | 103% |
| Class F | -0.65% | -0.78% | -0.80% | 116% | 101% |

Conclusion

In this contribution, we propose a scheme of scaling matrix. The proposed scheme provides transform coefficient control capability in all conventional and LFNST transform cases. It is recommended to adopt this method in the next VVC WD and VTM software.

Reference

- O. Chubach, C-Y. Lai, C-Y. Chen, T-D. Chuang, C-W. Hsu, Y-W. Huang, S-M. Lei, “CE7-related: Support of signalling default and user-defined scaling matrices,” JVET-N0847, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

- M. Koo, M. Salehifar, J. Lim, S. Kim, “CE6: Reduced Secondary Transform (RST) (CE6-3.1),” JVET-N0193, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).