| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0384 |
| --- | --- |

| Title: | CE4-related: CIIP Simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee (Sejong Universtiy) Woong Lim, Gun Bang, Hui Young Kim (ETRI) | Email: | jinyounglee@sejong.ac.kr gbang@etri.re.kr |
| Source: | Sejong University and ETRI |  |  |

_____________________________

- Abstract

At the last meeting, the number of intra prediction modes in CIIP is reduced to one. This contribution further simplifies the CIIP mode by simply reducing the number of merge candidates from six to two. Similar to MMVD, the proposed method only checks the first and second Merge candidates, and then signals a flag to indicate what Merge candidate is combined with the intra mode in CIIP. The experimental result shows that the proposed simplification has negligible loss.

Introduction

CIIP is used to improve the performance of the Merge mode. It combines the intra prediction mode and the Merge mode. In VTM 4.0, planar, DC, horizontal, and vertical prediction modes are evaluated to combine with the Merge mode candidates. However, the number of the intra prediction modes was reduced to one at the last meeting, so the CIIP mode only considers the planar mode in VTM 5.0. On the other hand, the number of Merge modes candidates is still six.

Proposed Method

The proposed method further simplifies the CIIP mode by simply reducing the number of Merge candidates from six to two, as displayed in Fig. 1. Similar to MMVD, the first and second Merge candidates are only checked, and then a flag is signaled to indicate what Merge candidate is combined with the Planar mode.

Fig. 1. CIIP simplification

Experimental Results

The proposed method was implemented on top of VTM 5.0. Table 1 shows the performance of the proposed method under the RA and LDB configuration.

Table 1 Experimental results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.16% | 0.04% | 118% | 99% |
| Class A2 | 0.04% | 0.05% | -0.02% | 90% | 99% |
| Class B | 0.05% | -0.05% | -0.11% | 94% | 100% |
| Class C | -0.01% | -0.09% | -0.14% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.06% | -0.07% | 99% | 99% |
| Class D | 0.07% | 0.01% | 0.12% | 99% | 99% |
| Class F | 0.03% | 0.10% | 0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.28% | 0.29% | 101% | 108% |
| Class C | 0.04% | -0.06% | 0.31% | 101% | 113% |
| Class E | 0.09% | 0.23% | -0.04% | 100% | 111% |
| Overall | 0.05% | 0.15% | 0.21% | 101% | 111% |
| Class D | 0.05% | -0.03% | 1.06% | 143% | 109% |
| Class F | -0.03% | 0.17% | -0.73% | 100% | 111% |

Proposed Specification Text

- 7.3.7.7 Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| …. |  |
| } else { |  |
| if( sps_mmvd_enabled_flag && cbWidth * cbHeight != 32 ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| ciip_cand_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| …. |  |
| } |  |

- 7.4.8.7 Merge data semantics

mmvd_cand_flag[ x0 ][ y0 ] specifies whether the first (0) or the second (1) candidate in the merging candidate list is used with the motion vector difference derived from mmvd_distance_idx[ x0 ][ y0 ] and mmvd_direction_idx[ x0 ][ y0 ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When mmvd_cand_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

…

ciip_cand_flag[ x0 ][ y0 ] specifies whether the first (0) or the second (1) candidate in the merging candidate list is used in the combined inter-picture merge and intra-picture prediction. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When ciip_cand_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

…

merge_idx[ x0 ][ y0 ] specifies the merging candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_idx[ x0 ][ y0 ] is not present, it is inferred as follows:

- If mmvd_merge_flag[ x0 ][ y0 ] is equal to 1, merge_idx[ x0 ][ y0 ] is inferred to be equal to mmvd_cand_flag[ x0 ][ y0 ].

- Otherwise, if ciip_flag[ x0 ][ y0 ] is equal to 1, merge_idx[ x0 ][ y0 ] is inferred to be equal to ciip_cand_flag[ x0 ][ y0 ].

- Otherwise (mmvd_merge_flag[ x0 ][ y0 ] is equal to 0 and ciip_flag[ x0 ][ y0 ] is equal to 0), merge_idx[ x0 ][ y0 ] is inferred to be equal to 0.

Table 9-11 – Syntax elements and associated binarizations

| merge_data( ) | regular_merge_flag[ ][ ] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | mmvd_merge_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_cand_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_distance_idx[ ][ ] | TR | cMax = 7, cRiceParam = 0 |
|  | mmvd_direction_idx[ ][ ] | FL | cMax = 3 |
|  | ciip_flag[ ][ ] | FL | cMax = 1 |
|  | ciip_cand_flag[][] | FL | cMax = 1 |
|  | merge_subblock_flag[ ][ ] | FL | cMax = 1 |
|  | merge_subblock_idx[ ][ ] | TR | cMax = MaxNumSubblockMergeCand − 1, cRiceParam = 0 |
|  | merge_triangle_split_dir[ ][ ] | FL | cMax = 1 |
|  | merge_triangle_idx0[ ][ ] | TR | cMax = MaxNumTriangleMergeCand − 1, cRiceParam = 0 |
|  | merge_triangle_idx1[ ][ ] | TR | cMax = MaxNumTriangleMergeCand − 2, cRiceParam = 0 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |

Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins

| regular_merge_flag[ ][ ] | cu_skip_flag[ ][ ] ? 0 : 1 | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| mmvd_merge_flag[ ][ ] | 0 | na | na | na | na | na |
| mmvd_cand_flag[ ][ ] | 0 | na | na | na | na | na |
| mmvd_distance_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | bypass |
| mmvd_direction_idx[ ][ ] | bypass | bypass | na | na | na | na |
| merge_subblock_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| merge_subblock_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| ciip_flag[ ][ ] | 0 | na | na | na | na | na |
| ciip_cand_flag[ ][ ] | bypass | na | na | na | na | na |
| merge_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| merge_triangle_split_dir[ ][ ] | bypass | na | na | na | na | na |
| merge_triangle_idx0[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| merge_triangle_idx1[ ][ ] | 0 | bypass | bypass | bypass | na | na |

Conclusions

This contribution proposes to simplify the CIIP mode by simply reducing the number of Merge candidates from six to two. The results demonstrated that proposed simplification has negligible loss. Therefore, it is recommended to adopt the proposed method into VVC.

- Patent rights declaration(s)

Sejong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).