| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0385-v2 |
| --- | --- |

| Title: | Non-CE9: An applying condition considering temporal correlation for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

In this contribution, it is proposed that an applying condition for DMVR considering low-correlation case between L0 and L1 reference blocks. The proposed method adds a condition to the early termination of DMVR. Concretely, if SAD between the initial L0 and L1 block is greater than the threshold, DMVR process is terminated. BD-rate for luma is 0.01% (RA) in comparison with VTM-5.0 for CTC sequences. The experimental results for non-CTC sequences used in JVET-N0828 are also reported.

Introduction

Decoder-side motion vector refinement (DMVR) is adopted to VVC Draft 5 [1] and VTM. DMVR modifies motion vectors derived by the merge mode to minimize SAD between a L0 and a L1 reference block. These processes are based on the assumption that correlation between the L0 reference block, the L1 reference block and the current block is high. However, DMVR in VVC Draft 5 doesn’t have any applying condition considering low correlation blocks.

Meanwhile, it was reported DMVR brings coding loss at some non-CTC test sequences in JVET-N0828 [2]. Some of the non-CTC sequences are low temporal correlation scene such as swimming with a lot of spray and cross-fade.

In this contribution, it is proposed that an improvement of applying condition for DMVR considering correlation between a L0 and a L1 reference block. The remainder of this contribution is divided into two parts;

- Proposed method, and

- Experimental results.

Proposed Method

DMVR in VVC Draft 5 has an early termination process by using the SAD value with initial MV as follows.

- The condition for early termination of DMVR in VVC Draft 5

  - SAD is less than 4 * sub-block width * (sub-block height >> 1).

The above condition considers a high temporal correlation case between the initial L0 and L1 blocks that don’t need further refinement.

In this contribution, it is proposed that additional condition for DMVR early termination considering low temporal correlation case as follows.

- The condition for early termination of DMVR in the proposed method

  - SAD is less than 4 * sub-block width * (sub-block height >> 1), or

  - SAD is greater than 400 * sub-block width * (sub-block height >> 1).

Experimental Results

The proposed method is implemented on top of VTM-5.0. In order to verify the performance of the proposed method, we have run simulations according to the common test conditions [3].

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Test results

Table 1 shows the experimental results. BD-rate for luma is 0.01% for RA in comparison with VTM-5.0. Differences of BD-rates are small for CTC sequences.

Table 1 Experimental results for RA condition under the CTCs.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.16% | -0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.04% | 0.01% | 100% | 100% |
| Class B | 0.03% | 0.03% | 0.06% | 100% | 100% |
| Class C | 0.00% | 0.02% | -0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.01% | 0.02% | 0.02% | 100% | 100% |
| Class F | 0.03% | 0.06% | 0.05% | 100% | 100% |

Figure 1 shows an example of a block in which DMVR is early terminated by the proposed method. As shown in the figure, correlation between the L0 reference block and the L1 reference block is low due to cross-fade. DMVR cannot refine MVs correctly at those blocks.

Figure 1 An example of DMVR early terminated block by the proposed method (MarketPlace POC:520)

Reference data

From Table 2 to Table 4 show the experimental results for the non-CTC sequences used in [2]. The test conditions are also same as [2] except down-sampling filter for converting YUV4:2:2 to YUV4:2:0. We used the FFmpeg for down-sampling. Please note that the anchor in below tables is “DMVR OFF” and the test is “DMVR ON”.

Table 2 shows the results of DMVR OFF vs. DMVR ON by VTM-4.0. And Table 3 shows the results of DMVR OFF vs. DMVR ON by VTM-5.0. From Table 2 and Table 3, it can be seen that the coding losses by DMVR in VTM-5.0 are smaller than that of VTM-4.0. It seems be the effect of the applying condition proposed in [4] and newly adopted in VTM-5.0 by our preliminary experimental results.

Table 4 shows the result of DMVR OFF vs DMVR ON with the proposed method by VTM-5.0. It can be seen that the coding losses by DMVR with the proposed method are even smaller than that of VTM-5.0.

Table 2 Experimental results of DMVR OFF/ON for RA condition for non-CTC sequences by VTM-4.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 DMVRoff |  |  |
|  | Y | U | V | EncT | DecT |
| c05_Fireworks_barrage | 0.31% | -0.46% | -0.86% | 102% | 116% |
| c07_Drama_sea | 0.01% | -0.61% | -0.37% | 102% | 110% |
| c10_SwimRace_crawl | 0.12% | -0.14% | -0.31% | 101% | 105% |
| c14_Volleyball_follow | 0.05% | -0.33% | -0.41% | 102% | 118% |
| c15_Paddock_fixed | 0.40% | 0.04% | 0.53% | 103% | 108% |
| Overall | 0.18% | -0.30% | -0.28% | 102% | 111% |

Table 3 Experimental results of DMVR OFF/ON for RA condition for non-CTC sequences by VTM-5.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 DMVRoff |  |  |
|  | Y | U | V | EncT | DecT |
| c05_Fireworks_barrage | -0.08% | -0.45% | -0.36% | 101% | 107% |
| c07_Drama_sea | -0.21% | -0.71% | -0.35% | 101% | 108% |
| c10_SwimRace_crawl | 0.03% | -0.07% | 0.09% | 100% | 101% |
| c14_Volleyball_follow | -0.60% | -0.58% | -0.57% | 101% | 105% |
| c15_Paddock_fixed | 0.14% | -0.12% | 0.07% | 101% | 102% |
| Overall | -0.14% | -0.38% | -0.23% | 101% | 105% |

Table 4 Experimental results of DMVR OFF/ON + Proposal for RA condition for non-CTC sequences by VTM-5.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 DMVRoff |  |  |
|  | Y | U | V | EncT | DecT |
| c05_Fireworks_barrage | -0.07% | -0.32% | -0.19% | 101% | 108% |
| c07_Drama_sea | -0.13% | -0.54% | -0.68% | 101% | 107% |
| c10_SwimRace_crawl | -0.01% | -0.07% | -0.07% | 100% | 100% |
| c14_Volleyball_follow | -0.61% | -0.85% | -0.76% | 100% | 105% |
| c15_Paddock_fixed | 0.05% | -0.21% | -0.07% | 101% | 101% |
| Overall | -0.15% | -0.40% | -0.35% | 101% | 104% |

Conclusion

In this contribution, it was proposed that an applying condition for DMVR considering low-correlation case between L0 and L1 reference blocks. The proposed method added a condition to the early termination of DMVR. Concretely, if SAD between the initial L0 and L1 block is greater than the threshold, DMVR process is terminated. BD-rate for luma was 0.01% (RA) in comparison with VTM-5.0 for CTC sequences. The experimental results for non-CTC sequences used in JVET-N0828 were also reported. It could be seen that coding losses by DMVR got smaller by the proposed method.

- Acknowledgment

We would like to thank NHK for doing crosschecking of our proposal.

References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Geneva, CH, Mar. 2019.

- S. Iwamura, S. Nemoto and A. Ichigaya, “[AHG13] Compression performance analysis for 4K and 8K HLG test sequences”, JVET-N0828, Geneva, CH, Mar. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

- T. Chujoh and T. Ikai, “Non-CE9: On conditions for DMVR and BDOF”, JVET-N0146, Geneva, CH, Mar. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).