| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET- O0387-v1 |
| --- | --- |

| Title: | Non-CE6: MTS Index Signaling Simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anish Tamse Min Woo Park Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com m.w.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

MTS index coding in VTM5.0 uses up to 4 context coded bins and 9 context models. The context model derivation process also makes use of the current block’s quad-tree depth in order to derive the contexts. This proposal simplifies the coding process by using only 2 context models and 2 context coded bins. The rest of the bins are by-pass coded. The derivation logic is removed too by using a fixed context model for each bin. The coding performance change after these simplifications is 0.00% for Y, -0.10% for U and -0.08% for V under RA conditions without any change in encoder and decoder run-time.

# Introduction

The MTS coding scheme in VVC draft 5 makes use of up to 4 context coded bins and 9 context models. The steps of the coding process are summarized below.

- If the bin is 1, the process ends, otherwise, if MTS is allowed, first bin of MTS index is coded

  - ctx = min((int)cu.qtDepth, 5 )

- If the previous bin is 0, the process ends otherwise, the next bin is coded using ctx = 6

- If the previous bin is 0, the process ends otherwise, the next bin is coded using ctx = 7

- If the previous bin is 0, the process ends otherwise, the next bin is coded using ctx = 8

This contribution proposes to simplify the coding process by removing the context derivation logic and reducing the context coded bins.

# Proposed Methods

## Removing the context derivation logic

The context derivation logic used in step 1 in the description in the previous section is completely removed and a fixed context is used for the second bin

Additionally, all the following bins after step 1 use the same context model.

## Reducing the context coded bins

Under this simplification, the bins coded in steps 3 and 3 are bypass coded to improve the CABAC throughput.

Combining the above-described simplifications, the following options are proposed to simplify the MTS coding without any performance degradation.

- Method 1) Use only 2 context models for the four bins, with the bin and model mapping decided beforehand.

- Method 2) Method 1 + last two bins are bypass coded

The methods are summarized more clearly in the table below

| Bin | VTM 5.0 | Method 1 | Method 2 |
| --- | --- | --- | --- |
| 1 | min( cu.qtDepth, 5) (0 - 5) | 0 | 0 |
| 2 | 6 | 1 | 1 |
| 3 | 7 | 1 | EP |
| 4 | 8 | 1 | EP |

The method simplifies the MTS coding by removing the computational logic required for context derivation, reduces contexts (9 to 2) and reduces the number of context coded bins (4 to 2) without affecting performance.

# Working Draft Changes

Method 1:

Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins

| tu_mts_idx[ ][ ] | cqtDepth 0 | 6 1 | 7 1 | 8 1 | na | na |
| --- | --- | --- | --- | --- | --- | --- |

Method 2:

Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins

| tu_mts_idx[ ][ ] | cqtDepth 0 | 6 1 | 7 bypass | 8 bypass | na | na |
| --- | --- | --- | --- | --- | --- | --- |

# Experimental Results

The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Table 1 shows a summary of all results.

Table 1. Summary of all results

|  | Test | BD-Rate (Y) | BD-Rate (U) | BD-Rate (V) | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| AI | Method 1 | 0.00% | -0.03% | 0.03% | 99% | 99% |
|  | Method 2 | 0.00% | -0.02% | 0.03% | 99% | 99% |
| RA | Method 1 | -0.01% | -0.09% | -0.05% | 97% | 98% |
|  | Method 2 | 0.00% | -0.10% | -0.08% | 99% | 99% |
| LDB | Method 1 | 0.04% | 0.19% | 0.13% | 99% | 98% |
|  | Method 2 | 0.04% | 0.06% | -0.04% | 100% | 100% |

Table 2. Results for Method 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.00% | 99% | 99% |
| Class A2 | 0.00% | -0.04% | -0.04% | 99% | 99% |
| Class B | 0.00% | -0.02% | 0.07% | 99% | 99% |
| Class C | -0.01% | -0.07% | 0.10% | 99% | 99% |
| Class E | -0.01% | 0.03% | -0.01% | 100% | 101% |
| Overall | 0.00% | -0.03% | 0.03% | 99% | 99% |
| Class D | 0.01% | 0.03% | -0.09% | 99% | 98% |
| Class F | -0.03% | 0.00% | -0.10% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.19% | 0.07% | 96% | 101% |
| Class A2 | 0.01% | 0.02% | -0.04% | 99% | 99% |
| Class B | 0.01% | -0.08% | -0.04% | 98% | 99% |
| Class C | -0.03% | -0.09% | -0.16% | 96% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.09% | -0.05% | 97% | 98% |
| Class D | 0.01% | 0.13% | -0.17% | 95% | 92% |
| Class F | -0.01% | -0.15% | 0.02% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.39% | -0.17% | 100% | 100% |
| Class C | 0.01% | 0.00% | -0.10% | 99% | 96% |
| Class E | 0.14% | 0.10% | 0.94% | 98% | 98% |
| Overall | 0.04% | 0.19% | 0.13% | 99% | 98% |
| Class D | 0.06% | -0.17% | 0.73% | 99% | 94% |
| Class F | -0.15% | -0.29% | -0.22% | 100% | 96% |

Table 3. Results for Method 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | -0.08% | -0.05% | 98% | 96% |
| Class B | 0.00% | -0.01% | 0.06% | 98% | 97% |
| Class C | -0.01% | -0.07% | 0.09% | 100% | 102% |
| Class E | -0.01% | 0.05% | -0.01% | 99% | 101% |
| Overall | 0.00% | -0.02% | 0.03% | 99% | 99% |
| Class D | 0.01% | 0.04% | -0.10% | 100% | 100% |
| Class F | -0.03% | 0.00% | -0.10% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.13% | 0.08% | 97% | 100% |
| Class A2 | 0.02% | -0.10% | 0.03% | 99% | 97% |
| Class B | 0.01% | -0.10% | -0.13% | 100% | 102% |
| Class C | -0.03% | -0.07% | -0.23% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.10% | -0.08% | 99% | 99% |
| Class D | 0.01% | 0.12% | -0.18% | 96% | 93% |
| Class F | -0.01% | -0.15% | -0.01% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.03% | -0.16% | 98% | 96% |
| Class C | 0.06% | 0.20% | -0.27% | 102% | 102% |
| Class E | 0.05% | -0.07% | 0.49% | 101% | 105% |
| Overall | 0.04% | 0.06% | -0.04% | 100% | 100% |
| Class D | -0.03% | -0.89% | 0.75% | 99% | 97% |
| Class F | -0.02% | -0.31% | -0.52% | 100% | 99% |

# Conclusion

This contribution proposes to simplify MTS index coding. The method simplifies the MTS coding as follows:

- Removes the computational logic required for context derivation

- Reduces context models needed (9 to 2)

- Reduces the number of context coded bins (4 to 2)

There is no significant performance change observed when all these simplifications are implemented together as seen from the results. It is hence suggested to adopt the proposed method 2 into the next VVC working draft text.

# References

[1] “Versatile Video Coding (Draft 5),” JVET-N1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, March 2019.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, March 2019.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).