| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0388-v2 |
| --- | --- |

| Title: | Non-CE4: On the Context of Triangle Merge Index |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

Currently, both of the two indices of the triangle prediction mode reuse the context of the regular merge index. Although the candidates of the triangle prediction mode is the same as that of regular merge mode, the two indices of the triangle prediction mode have different distribution feature since they must be different from each other. For this purpose, two more contexts are added for the triangle index coding. It is reported that the modification bring 0.00% gains and 0.06% gains for RA and LB configurations respectively compared to VTM-5.0.

Introduction

When the triangle prediction mode is used, a CU is split evenly into two triangle-shaped partitions, using either the diagonal split or the anti-diagonal split (Figure 1). Each triangle partition in the CU is inter-predicted using its own motion; only uni-prediction is allowed for each partition, that is, each partition has one motion vector and one reference index. Since the motions of each partition are different, they have different motion indices.

Figure 1 – Triangle partition based inter prediction

Proposed Modification

Here, two more contexts for different split modes are added for the triangle index coding, thus the context of merge_triangle_idx0[][] and merge_triangle_idx1[][] are modified as follows.

Table 1- Assignment of ctxInc to merge_triangle_idx0/1 with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| merge_triangle_idx0[ ][ ] | merge_triangle_split_dir[ ][ ] ? 1 : 0 | bypass | bypass | bypass | bypass | na |
| merge_triangle_idx1[ ][ ] | merge_triangle_split_dir[ ][ ] ? 1 : 0 | bypass | bypass | bypass | na | na |

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform. The RA and LB configurations specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

The results are given as Table 1. It is reported that the modification has 0.00% gains and 0.06% gains for RA and LB configurations respectively compared to VTM-5.0.

Table 1 Experimental Results

Conclusion

In this contribution, two more contexts for different split modes are added for the triangle index coding. The modification brings 0.06% gains on average. Based on the above results, it is suggested to adopt them into the VTM.

Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).