| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0389-v1 |
| --- | --- |

| Title: | On Supporting 64x64 Chroma Transform Unit |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenting Cai, Jianqing Zhu, Jie Yao, Kimihiko Kazui 601,Yongda International Tower, 2277Longyang Road Shanghai, 201204, China | Email: | caiwenting@cn.fujitsu.com zhujianqing@cn.fujitsu.com yaojie@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This contribution proposes to support the maximum transform unit size of chroma components up to 64x64 for inter CU. When inter CU size in luma samples is larger or equal to 128x128, the size of transform unit of chroma components is 64x64. Otherwise, the maximum transform unit size of chroma is 32x32. By sharing transform core and internal buffer between luma TU and chroma TU, there is no extra hardware cost on decoder pipeline except scheduling mechanic of pipeline. The simulation results show 0.xx%, 0.xx%, and 0.xx% BD-rates for Y, Cb, and Cr components respectively in RA configuration on top of VTM-5.0. It has no impact on the performance of AI.

Introduction

In VTM-5.0[1], the maximum supported luma transform size is 64×64 and the maximum supported chroma transform size is 32×32. When the width or height of the CB is larger the maximum transform width or height, the CB is automatically split in the horizontal and/or vertical direction to meet the transform size restriction in that direction.

Therefore, a 128x128 CU in 4:2:0 format will be tiled with four 64x64 luma TUs and four 32x32 chroma TUs for Cb and Cr respectively.

In VTM5, 3.5.1	large block-size transforms with high-frequency zeroing brings benefit on coding efficiency. Therefore, theoretically, if chroma TU size is extended to 64x64, the coding gain could be obtained, especially for higher resolution video, e.g., 1080p and 4K sequences.

Proposed method

This contribution proposes to use maximum transform TB size 64x64 for both luma and chroma components for inter CU in P and B slices in which CTU is partitioned by using a single tree. When inter CU size in luma samples is larger or equal to 128x128, the size of transform unit of chroma components is 64x64. Otherwise, the maximum transform unit size of chroma is 32x32.

Figure 1 illustrates the case of 128x128 CU in 4:2:0 format.

The processing order can be Y0->Y1->Y2->Y3->Cb->Cr. Transform core and internal buffer can be shared between luma and chroma TU.

Figure 1. Maximum TB partition of 128x128 CU for inter slices

Experimental results

The proposed scheme is implemented on top of VTM-5.0 software [2]. The simulations were carried out under the JVET common test conditions [3].

The test results for JVET common test conditions are tabulated in Table 1.

Table 1. Experimental results for random-access (RA) test condition; anchor is VTM5.0, only 1 GoP

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.95% | -0.38% | 84% | #DIV/0! |
| Class A2 | 0.05% | -0.16% | 0.02% | 94% | #DIV/0! |
| Class B | 0.03% | -0.29% | -0.32% | 126% | #DIV/0! |
| Class C | 0.01% | 0.12% | 0.15% | 110% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.28% | -0.14% | 106% | #DIV/0! |
| Class D | 0.00% | 0.00% | 0.00% | 95% | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusions

In this contribution, the maximum transform unit size of chroma for inter CU is extended to 64x64 which is the same as that of luma. By sharing transform core and internal buffer between luma and chroma TU, there is no extra hardware cost on decoder pipeline except scheduling mechanic of pipeline. Appling this technique brings benefit on coding efficiency, especially for higher resolution sequences.

References

- J. Chen, Y. Ye and S. H. Kim “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5),” JVET-N1002, Geneva, CH, Mar. 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, Mar. 2019.

Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).