| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0392 |
| --- | --- |

| Title: | AHG17: Output of SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes to include a flag (sei_output_flag) in the general SEI message syntax, i.e. sei_message( ). When the flag is equal to 1, the SEI message is output together with the associated picture.

# Problem Statement

Picture-synchronized metadata might be needed for rendering, e.g. for region-wise packed 360-degree video or for 3DoF+ video where packing or patch metadata, respectively, could change dynamically. In some operating systems and/or device architectures, the player application might not be able to pass metadata to the rendering process in a picture-synchronized manner but rather only the video decoder might be capable of doing that. This might apply particularly for encrypted video.

The metadata needed for rendering includes for example the projected omnidirectional video SEI messages and the frame packing arrangement SEI message as specified in HEVC. However, it would be impractical to specify a list of SEI messages that shall be output with decoded pictures. Some SEI messages might be specified in later versions of VVC; yet a decoder according to VVC v1 should conclude correctly if the SEI messages is to be output to the rendering process.

# Proposed specification text changes

## SEI message syntax

| sei_message( ) { | Descriptor |
| --- | --- |
| payloadType = 0 |  |
| do { |  |
| payload_type_byte | u(8) |
| payloadType += payload_type_byte |  |
| } while (payload_type_byte = = 0xFF ) |  |
| sei_output_flag | u(1) |
| payload_size_7bits | u(7) |
| payloadSize = 0payload_size_7bits |  |
| if( payload_size_7bits = = 0x7F ) |  |
| do { |  |
| payload_size_byte | u(8) |
| payloadSize += payload_size_byte |  |
| } while (payload_size_byte = = 0xFF ) |  |
| sei_payload( payloadType, payloadSize ) |  |
| } |  |

## SEI message semantics

The following semantics are added:

sei_output_flag equal to 1 specifies that the SEI message is output together with the decoded and cropped picture resulting from decoding the coded picture with which the SEI message is associated. sei_output_flag equal to 0 specifies that the SEI message may or may not be output together with the decoded and cropped picture resulting from decoding the coded picture with which the SEI message is associated.

payload_size_7bits affects of the derivation of the payload size of an SEI message as specified in the syntax table.

## HRD

### C.3.3	Picture output

…

When output, the picture is cropped, using the conformance cropping window specified in the active SPS for the picture, and the SEI messages that have sei_output_flag equal to 1 and are associated with the picture are output with the picture.

…

- C.5.2.4	"Bumping" process

The "bumping" process consists of the following ordered steps:

- The picture that is first for output is selected as the one having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- The picture is cropped, using the conformance cropping window specified in the active SPS for the picture, the cropped picture is output, and the picture is marked as "not needed for output", and the SEI messages that have sei_output_flag equal to 1 and are associated with the picture are output with the picture.

- When the picture storage buffer that included the picture that was cropped and output contains a picture marked as "unused for reference", the picture storage buffer is emptied.

NOTE – For any two pictures picA and picB that belong to the same CVS and are output by the "bumping process", when picA is output earlier than picB, the value of PicOrderCntVal of picA is less than the value of PicOrderCntVal of picB.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).