| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0393 |
| --- | --- |

| Title: | AHG17: On cross-RAP referencing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes an approach that is asserted to achieve the functionality of cross-RAP referencing using the multiple independent layers functionality of VVC Draft 5. The proposed approach consists of the following parts:

- Output layer sets in VPS

- A flag per each output layer set to indicate that there is one and only one picture per each access unit among all layers of the output layer set. When the flag is equal to 1:

  - A single profile-tier-level syntax structure that applies to the output layer set (as if there were only a single layer)

  - max_dec_pic_buffering_minus1, max_num_reorder_pics, and max_latency_increase_plus1 that apply collectively to the output layer set (rather than per layer)

- Possibility to provide an output layer set index as an externally controlled variable. If the variable is provided by external means, a sub-bitstream extraction process that extracts all layers of the output layer set is invoked. Otherwise, the current layer-wise operation of VVC is carried out.

If the decoding control NAL unit proposed in JVET-O0391 is adopted, it is proposed to carry the target output layer set index in the decoding control NAL unit rather than control the respective variable by external means.

# Introduction

The capability of referencing to pictures preceding the previous IRAP picture, in decoding order, have been studied in JVET-M0360 and JVET-N0119. Decoded "library pictures" and coded "external pictures" are provided as input to the VVC decoding process in JVET-M0360 and JVET-N0119, respectively. The proposed approaches are asserted to require:

- a specific interface to the decoder to pass the library or external pictures,

- signalling and decoding process changes in the VVC standard for using the library or external pictures as reference for prediction, and

This contribution proposes an approach that re-uses the independent layer design of VVC to achieve the cross-RAP referencing functionality.

# Overview of the proposal

Figure 1 provides an example of the proposed approach. The bitstream originates from three video sources (e.g. different cameras), which the content author has cut to shots and interleaved to the same bitstream. There are non-adjacent shots that originate from the same video source and hence correlate with each other. The shots that are originate from the same video source are encoded into the same independent layer. Shots can start with a DRAP picture that is predicted from the IRAP picture within the same independent layer.

Figure 1 – Shots originating from three video sources interleaved to the same bitstream as separate independent layers.

The following syntax and processes are proposed:

- Output layer sets in VPS

- A flag per each output layer set to indicate that there is one and only one picture per each access unit among all layers of the output layer set. When the flag is equal to 1:

  - A single profile-tier-level syntax structure that applies to the output layer set (as if there were only a single layer)

  - max_dec_pic_buffering_minus1, max_num_reorder_pics, and max_latency_increase_plus1 that apply collectively to the output layer set (rather than per layer)

- Possibility to provide an output layer set index as an externally controlled variable. If the variable is provided by external means, a sub-bitstream extraction process that extracts all layers of the output layer set is invoked. Otherwise, the current layer-wise operation of VVC is carried out.

If the decoding control NAL unit proposed in JVET-O0391 is adopted, it is proposed to carry the target output layer set index in the decoding control NAL unit rather than control the respective variable by external means.

# Proposed specification text

## Definitions

The following definitions are proposed similarly to the respective definitions in HEVC:

output layer: A layer of an output layer set that is output when TargetOlsIdx is equal to the index of the output layer set.

output layer set (OLS): A set of layers, where one or more layers in the set of layers are indicated to be output layers.

## VPS syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_bit | u(1) |
| } |  |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| vps_num_ols_minus1 | ue(v) |
| for( i = 0; i <= vps_num_ols_minus1; i++ ) { |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) { |  |
| ols_layer_idx_included_flag[ i ][ j ] | u(1) |
| if( ols_layer_idx_included_flag[ i ][ j ] ) |  |
| ols_layer_idx_output_flag[ i ][ j ] | u(1) |
| } |  |
| ols_one_pic_per_au_flag[ i ] | u(1) |
| if( ols_one_pic_per_au_flag[ i ] ) { |  |
| ols_max_sub_layers_minus1[ i ] | u(3) |
| profile_tier_level( ols_max_sub_layers_minus1[ i ] ) |  |
| ols_sub_layer_ordering_info_present_flag[ i ] | u(1) |
| for( j = ( ols_sub_layer_ordering_info_present_flag[ i ] ? 0 : ols_max_sub_layers_minus1[ i ] ); j <= ols_max_sub_layers_minus1[ i ]; j++ ) { |  |
| ols_max_dec_pic_buffering_minus1[ i ][ j ] | ue(v) |
| ols_max_num_reorder_pics[ i ][ j ] | ue(v) |
| ols_max_latency_increase_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Additions to VPS semantics

vps_num_ols_minus1 plus 1 specifies the number of output layer sets.

ols_layer_idx_included_flag[ i ][ j ] equal to 0 specifies that the j-th layer is not included in the i-th output layer set. ols_layer_idx_included_flag[ i ][ j ] equal to 1 specifies that the j-th layer is included in the i-th output layer set.

ols_layer_idx_output_flag[ i ][ j ] equal to 0 specifies that the j-th layer is not output when the i-th output layer set is decoded. ols_layer_idx_output_flag[ i ][ j ] equal to 1 specifies that the j-th layer is an output layer when the i-th output layer set is decoded. In this version of this Specification, ols_layer_idx_output_flag[ i ][ j ], when present, shall be equal to 1. When not present, ols_layer_idx_output_flag[ i ][ j ] is inferred to be equal to 0.

ols_one_pic_per_au_flag[ i ] equal to 0 specifies that an access unit may contain a coded picture at any number of layers included in the i-th output layer set. ols_one_pic_per_au_flag[ i ] equal to 1 specifies that there is one and only one coded picture per each access unit among all layers of the i-th output layer set. In this version of this Specification, ols_one_pic_per_au_flag[ i ] shall be equal to 1.

ols_max_sub_layers_minus1[ i ] specifies the maximum number of temporal sub-layers that may be present in the i-th output layer set in each CVS referring to the VPS.

ols_sub_layer_ordering_info_preset_flag[ i ] equal to 1 specifies that ols_max_dec_pic_buffering_minus1[ i ][ j ], ols_max_num_reorder_pics[ i ][ j ], and ols_max_latency_increase_plus[ i ][ j ] are present for ols_max_sub_layers_minus1[ i ] + 1 sub-layers. ols_sub_layer_ordering_info_present_flag[ i ] equal to 0 specifies that the values of ols_max_dec_pic_buffering_minus1[ ols_max_sub_layers_minus1[ i ] ], ols_max_num_reorder_pics[ ols_max_sub_layers_minus1[ i ] ], and ols_max_latency_increase_plus1[ ols_max_sub_layers_minus1[ i ] ] apply to all sub-layers.

ols_max_dec_pic_buffering_minus1[ i ][ j ], ols_max_num_reorder_pics[ i ][ j ], and ols_max_latency_increase_plus[ i ][ j ] are specified identically to sps_max_dec_pic_buffering_minus1[ j ], sps_max_num_reorder_pics[ j ], and sps_max_latency_increase_plus1[ j ] for a bitstream where the layers of the i-th output layer set are present and layers not included in the i-th output layer set are not present.

## Additions to the decoding process in clause 8.1.1

…

For each CVS in the bitstream, the variable TargetLayerId, which identifies the target layer to be decoded, and the variable HighestTid, which identifies the highest temporal sub-layer to be decoded, are specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetLayerId, TargetLayerId is set by the external means.

–	If some external means, not specified in this Specification, is available to set TargetOlsIdx, TargetOlsIdx is set by the external means.

–	Otherwise, TargetLayerId is set equal to vps_included_layer_id[ 0 ].

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

If TargetLayerId is provided by external means or inferred, the following applies:

–	for each CVS in the bitstream, the sub-bitstream extraction process as specified in clause 10.1 is applied with the CVS, TargetLayerId, and HighestTid as inputs, and the output is assigned to a bitstream referred to as CvsToDecode.

–	After that, the instances of CvsToDecode of all the CVSs are concatenated, in decoding order, and the result is assigned to the bitstream BitstreamToDecode.

–	Clause 8.1.2 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

Otherwise (TargetOlsIdx is provided by external means), the following applies:

–	For each CVS in the bitstream, the sub-bitstream extraction process as specified in clause 10.2 is applied with the CVS, TargetOlsIdx, and HighestTid as inputs, and the output is assigned to a bitstream referred to as CvsToDecode.

–	After that, the instances of CvsToDecode of all the CVSs are concatenated, in decoding order, and the result is assigned to the bitstream BitstreamToDecode.

–	Separate instances of variables starting with an upper case letter are derived and kept for each layer included in the output layer set with the index TargetOlsIdx. Clause 8.1.2 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order. The upper case variables pertaining to the NuhLayerId of the coded picture provided as input to the decoding process are used in the decoding process.

## Sub-bitstream extraction process for an output layer set

The sub-bitstream extraction process of clause 10 is moved under clause 10.1 and clause 10.2 is introduced as follows:

Inputs to this process are a bitstream, the index of a target output layer set olsIdxTarget, and a target highest TemporalId value tIdTarget.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitstream, olsIdxTarget equal to any value in the range of 0 to vps_num_ols_minus1, inclusive, and tIdTarget equal to any value in the range of 0 to 6, inclusive, as inputs, and that satisfies the following condition shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget and NuhLayerId equal to vps_included_layer_id[ layerIdx ] with any layerIdx value in the range of 0 to vps_max_layers_minus1, inclusive, such that ols_layer_idx_included_flag[ olsIdxTarget ][ layerIdx ] is equal to 1.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0.

The output sub-bitstream is derived as follows:

–	Remove all NAL units with TemporalId greater than tIdTarget.

–	Remove all NAL units with NuhLayerId not equal to any of the values vps_included_layer_id[ layerIdx ] for all  layerIdx values in the range of 0 to vps_max_layers_minus1, inclusive, such that ols_layer_idx_included_flag[ olsIdxTarget ][ layerIdx ] is equal to 1.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).