| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0395-v2 |
| --- | --- |

| Title: | AHG8: On adaptive resolution changing and scalable coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Alireza Aminlou Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes an approach that uses a single layer (i.e. all NAL units have the same NuhLayerId value) and can be used for adaptive resolution changing as well as for spatial, quality, and view scalability. It is asserted that the contribution could be adapted  to use different NuhLayerId values too.

# Introduction

AHG8 mandates include the following:

- Study adaptive-resolution coding approaches for real-time communication, adaptive streaming, and 360-degree viewport-dependent streaming, including filters for resampling, reference picture management, and related scope and signalling

- Study potential approaches for support of layered coding scalability including spatial, temporal, quality, and view scalability

This contribution proposes an approach that uses a single layer (i.e. all NAL units have the same NuhLayerId value) and can be used for adaptive resolution changing as well as for spatial, quality, and view scalability.

# Overview of the proposal

From POC, RPL, and RPM perspective, the proposal can be summarized with the following table:

| POC | RPL syntax | RPM |
| --- | --- | --- |
| Syntax and semantics not changed. No two pictures have the same POC. | Not changed. | The RPM process remains unchanged. - O0395 presents a single-layer approach where "layers" are typically represented as sub-layers. Thus, a decoded picture may only be marked as "unused for reference" when decoding a picture of the same or lower sub-layer. O0395 could be combined with O0334 to use different NuhLayerId values. In this case, a decoded picture may only be marked as "unused for reference" when decoding a picture of the same layer. |

The proposal includes the following proposed items:

- sps_max_rpr is added in the SPS syntax. It specifies the maximum number of reference pictures to be resampled per any picture to be decoded. When equal to 0, reference picture resampling is not used.

- Picture width and height signalling is modified as follows:

- max_width_in_luma_samples and max_height_in_luma_samples are included in the SPS syntax. They do not affect the decoding process but only provide the maximum value for pic_width_in_luma_samples and pic_height_in_luma_samples, respectively.

- pic_width_in_luma_samples and pic_height_in_luma_samples are moved from SPS to PPS.

- The following signalling is proposed to control normatively whether the decoder outputs only the highest "layer" (for spatial and quality scalability) or all "layers" (view scalability etc.).

poc_output_period_minus1 is added in the SPS syntax to specify a POC output period in terms of adjacent POC values. Pictures of the same POC output period represent the same capture time. Out of the decoded pictures of the same POC output period, the decoded pictures that precede the last decoded picture in decoding order are not output. poc_output_period_minus1 greater than 0 is used for spatial and quality scalability to avoid outputting more than one picture of the same capture time.

Figure 1 presents an example of a spatially scalable bitstream with two resolutions. poc_output_period_minus1 is equal to 1, specifying that the POC output period consists of two adjacent POC values. When both resolutions are present in the bitstream, pictures with POC equal to 0, 2, 4, and so on are not output, whereas pictures with POC equal to 1, 3, 5, and so on are output. When the higher resolution (i.e., pictures with POC equal to 1, 3, 5, and so on) has been removed from the bitstream, pictures with POC equal to 0, 2, 4, and so on are output.

Figure 1 – Spatially scalable bitstream with two resolutions. poc_output_period_minus1 is equal to 1. 
The values within solid rectangles indicate the POC value of the picture

- The reference picture resampling process is invoked implicitly for the active reference picture(s) that have a different width and height from those of the current picture. The resampled reference pictures are used only for decoding the current picture after which they are discarded.

- The SHVC resampling process (HEVC clause H.8.1.4.2), with an addition of selection of input samples for filtering in wraparound manner (for 360° video, particularly ERP), is proposed.

- Constraints are added

- to disallow using a resampled reference picture as a source for temporal motion vector prediction,

- to require the number of distinct active reference pictures that require resampling to be less than or equal to sps_max_rpr.

# Proposed specification text

## sps_max_rpr

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_max_rpr | ue(v) |
| … |  |

sps_max_rpr specifies the maximum number of active reference pictures in reference picture list 0 or 1 having pic_width_in_luma_samples and pic_height_in_luma_samples not equal to pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the current picture.

## Picture width and height

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| max_width_in_luma_samples | ue(v) |
| max_height_in_luma_samples | ue(v) |
| … | ue(v) |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| … |  |

max_width_in_luma_samples specifies that it is a requirement of bitstream conformance that pic_width_in_luma_samples for any picture for which this SPS is active is less than or equal to max_width_in_luma_samples.

max_height_in_luma_samples specifies that it is a requirement of bitstream conformance that pic_height_in_luma_samples for any picture for which this SPS is active is less than or equal to max_height_in_luma_samples.

## poc_output_period_minus1 and invoking the resampling process

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| poc_output_period_minus1 | ue(v) |
| … |  |

poc_output_period_minus1 plus 1 specifies the number of adjacent POC values assigned to the same POC output period. Out of the decoded pictures of the same POC output period, the decoded pictures that precede the last decoded picture in decoding order are not output.

### 8.1.2	Decoding process for a coded picture

…

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause 8.2.

- The processes in clause 8.3 specify the following decoding processes using syntax elements in the slice header layer and above:

–	Variables and functions relating to picture order count are derived as specified in clause 8.3.1. This needs to be invoked only for the first slice of a picture.

–	At the beginning of the decoding process for each slice of a non-IDR picture, the decoding process for reference picture lists construction specified in clause 8.3.2 is invoked for derivation of reference picture list 0 (RefPicList[ 0 ]) and reference picture list 1 (RefPicList[ 1 ]).

–	The decoding process for reference picture marking in clause 8.3.3 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice of a picture.

–	When the current picture is a CRA picture with NoIncorrectPicOutputFlag equal to 1 or GRA picture with NoIncorrectPicOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.4 is invoked, which needs to be invoked only for the first slice of a picture.

–	For each active reference picture in RefPicList[ 0 ] and RefPicList[ 1 ] that has pic_width_in_luma_samples or pic_height_in_luma_samples not equal to pic_width_in_luma_samples or pic_height_in_luma_samples, respectively, of CurrPic, the following applies:

–	The resampling process in clause X.Y.Z is invoked [Ed. (MH): details of invocation parameters to be added] with the output picture having the same PicOrderCntVal as the input picture.

–	The resampled reference pictures are used for decoding CurrPic instead of the respective input pictures for the resampling process.

–	The resampled reference pictures are discarded after decoding CurrPic.

–	PictureOutputFlag for CurrPic is set as follows:

–	If one of the following conditions is true, PictureOutputFlag is set equal to 0:

–	the current picture is a RASL picture and NoIncorrectPicOutputFlag of the associated IRAP picture is equal to 1.

–	gra_enabled_flag is equal to 1 and the current picture is GRA picture with NoIncorrectPicOutputFlag equal to 1.

–	gra_enabled_flag is equal to 1, the current picture is associated with a GRA picture with NoIncorrectPicOutputFlag equal to 1, and PicOrderCntVal of the current picture is less than RpPicOrderCntVal of the associated GRA picture.

–	Otherwise, PictureOutputFlag is set equal to 1.

–	The variable PocOutputPeriod for CurrPic is derived as follows:

–	If PicOrderCntVal is greater than or equal to 0, PocOutputPeriod is set equal to PicOrderCntVal / ( poc_output_period_minus1 + 1 ).

–	Otherwise, PocOutputPeriod is set equal to −( ( Abs( PicOrderCntVal ) − 1 ) / ( poc_output_period_minus1 + 1 ) + 1 ).

–	It is a requirement of bitstream conformance that the decoded picture preceding CurrPic in decoding order and having the same PocOutputPeriod value as CurrPic has not been output.

–	PictureOutputFlag is set equal to 0 for all decoded pictures having the same PocOutputPeriod value as CurrPic.

- [Ed. (YK): Add herein the invocation of the decoding processes for coding tree units, scaling, transform, in-loop filtering, etc.]

- After all slices of the current picture have been decoded, the current decoded picture is marked as "used for short-term reference".

## Resampling process

SHVC resampling process (HEVC clause H.8.1.4.2) is proposed with the following additions:

…

If sps_ref_wraparound_enabled_flag is equal to 0, the sample value tempArray[ n ] with n = 0..7, is derived as follows:

tempArray[ n ] = ( fL[ xPhase, 0 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef − 3 ), yPosRL ] +
					fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef − 2 ), yPosRL ] +
					fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef − 1 ), yPosRL ] +
					fL[ xPhase, 3 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef ), yPosRL ] +
					fL[ xPhase, 4 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 1 ), yPosRL ] +	(H-38)
					fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 2 ), yPosRL ] +
					fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 3 ), yPosRL ] +
					fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 4 ), yPosRL ] )  >>  shift1

Otherwise, the sample value tempArray[ n ] with n = 0..7, is derived as follows:

refOffset = ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY

tempArray[ n ] = ( fL[ xPhase, 0 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef − 3 ), yPosRL ] +
					fL[ xPhase, 1 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef − 2 ), yPosRL ] +
					fL[ xPhase, 2 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef − 1 ), yPosRL ] +
					fL[ xPhase, 3 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef ), yPosRL ] +
					fL[ xPhase, 4 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef + 1 ), yPosRL ] +
					fL[ xPhase, 5 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef + 2 ), yPosRL ] +
					fL[ xPhase, 6 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef + 3 ), yPosRL ] +
					fL[ xPhase, 7 ] * rlPicSampleL[ ClipH( refOffset, refW, xRef + 4 ), yPosRL ] )
					  >>  shift1

…

If sps_ref_wraparound_enabled_flag is equal to 0, the sample value tempArray[ n ] with n = 0..3, is derived as follows:

tempArray[ n ] = ( fC[ xPhase, 0 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef − 1 ), yPosRL ] +
				fC[ xPhase, 1 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef ), yPosRL ] +
				fC[ xPhase, 2 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef + 1 ), yPosRL ] +	(H-50)
				fC[ xPhase, 3 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef + 2 ), yPosRL ] )  >>  shift1

Otherwise, the sample value tempArray[ n ] with n = 0..3, is derived as follows:

refOffset = ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) / SubWidthC

tempArray[ n ] = ( fC[ xPhase, 0 ] * rlPicSampleC[ ClipH( refOffset, refWC, xRef − 1 ), yPosRL ] +
				fC[ xPhase, 1 ] * rlPicSampleC[ ClipH( refOffset, refWC, xRef ), yPosRL ] +
				fC[ xPhase, 2 ] * rlPicSampleC[ ClipH( refOffset, refWC, xRef + 1 ), yPosRL ] +
				fC[ xPhase, 3 ] * rlPicSampleC[ ClipH( refOffset, refWC, xRef + 2 ), yPosRL ] )
				  >>  shift1

## Constraints

### 8.3.2	Decoding process for reference picture lists construction

…

It is a requirement of bitstream conformance that the following constraints apply:

- …

- When slice_temporal_mvp_enabled_flag is equal to 1, pic_width_in_luma_samples and pic_height_in_luma_samples of RefPicList[ collocated_from_l0_flag ? 0 : 1 ][ 0 ] shall be equal to pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the current picture.

- The number of distinct active reference pictures in RefPicList[ 0 ] and RefPicList[ 1 ] that have pic_width_in_luma_samples or pic_height_in_luma_samples not equal to pic_width_in_luma_samples or pic_height_in_luma_samples, respectively, of the current picture, shall be less than or equal to sps_max_rpr.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).