| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0397 |
| --- | --- |

| Title: | Non-CE3: MIP mode reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In matrix intra prediction (MIP), the block sizes are classified into three categories and each category supports 35/19/11 modes, and it requires 18/10/6 matrices, respectively.This contribution proposes to reduce the number of modes in MIP. The proposed method reduces the number of MIP modes and matrices as follows:

- Method 1: each category uses 11/11/11 modes, and it requires 6/6/6 matrices.

- Method 2: each category uses 6/7/8 modes, and it requires 3/4/4 matrices

With the method 1, the memory of MIP matrix is reduced to 75.7% with negligible BD-rate change. In the method 2, the memory of MIP matrix is reduced to 49.2%, compared to the VTM5.0.

Experimental results for Method 1 show 0.01%, -0.03%, and 0.00% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration, and Method 2 shows 0.04%, -0.03%, and -0.03% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration.

Introduction

Matrix intra prediction (MIP) was adopted in the 14th JVET meeting. In this algorithm, the block sizes are classified into three categories and under each category several MIP modes are defined. Table 1 shows the number of MIP modes and the matrix data in each category of MIP blocks. As shown in Table 1, 7920 bytes of matrix data are used for the current MIP design, and it is desirable to reduce the matrix data for the hardware implementation of VVC.

Table 1 – Number of MIP modes and the matrix data in each categories of MIP blocks

| Category(MipSizeId) | Block sizes (luma) | Number of MIP modes | Number of 10-bit parameters in MIP matrix |
| --- | --- | --- | --- |
| 0 | 4x4 | 35 | 1440 |
| 1 | 4x8, 8x4, 8x8 | 19 | 1440 |
| 2 | Other block sizes | 11 | 3456 |

Proposed method

To reduce the matrix data of MIP, the number of MIP modes is modified as follows:

- Method 1: The MIP modes are reduced 35/19/11 to 11/11/11

- Method 2: The MIP modes are reduced 35/19/11 to 6/7/8

Proposed method 1

In the proposed method 1, the MIP modes of each category are 11/11/11, and the number of matrix in MIP can be reduced to 6/6/6 as shown in Table 2.

Table 2 – Number of MIP modes and the matrix data with proposed method 1

| MipSizeId(Category) | Block sizes (luma) | Number of MIP modes | Number of 10-bit parameters in MIP matrix |
| --- | --- | --- | --- |
| 0 | 4x4 | 11 | 480 |
| 1 | 4x8, 8x4, 8x8 | 11 | 864 |
| 2 | Other block sizes | 11 | 3456 |

As shown in table 2, the number of 10-bit parameters in MIP matrix is reduced to 75.7%, compare to the VTM5. In the category 0, 11 MIP modes are generated using the selected 6 matrices ( 0th, 3rd, 5th, 9th, 15th, and 17th matrix in VTM5 ). In the category 1, 11 MIP modes are generated using the selected 6 matrices ( 0th, 1st, 3rd, 6th, 8th, and 9th matrix in VTM5 ). The reduced MIP modes are coded with the same method as VTM5.

Table 3 shows the modified MIP modes between VTM5 and method 1.

Table 3 – Modified MIP mode between VTM5 and proposed method 1

| Modified MIP modes in MipSizeId 0,1,2 | MIP modes in VTM5 |  |  |
| --- | --- | --- | --- |
|  | MipSizeId 0 | MipSizeId 1 | MipSizeId 2 |
| 0 | 0 | 0 | 0 |
| 1 | 3 | 1 | 1 |
| 2 | 5 | 3 | 2 |
| 3 | 9 | 6 | 3 |
| 4 | 15 | 8 | 4 |
| 5 | 17 | 9 | 5 |
| 6 | 20 | 10 | 6 |
| 7 | 22 | 12 | 7 |
| 8 | 26 | 15 | 8 |
| 9 | 32 | 17 | 9 |
| 10 | 34 | 18 | 10 |

In the proposed method 1, the default MIP MPM table and the mode mapping table between MIP and intra mode are also modified as shown in Table 4, 5 and 6.

Table 4 – Modified mapping between intra prediction and MIP modes for method 1

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 5 | 0 | 5 |
| 1 | 5 | 0 | 1 |
| 2, 3 | 5 | 6 | 3 |
| 4, 5 | 3 | 6 | 3 |
| 6,7 | 3 | 6 | 3 |
| 8, 9 | 3 | 6 | 3 |
| 10, 11 | 3 | 6 | 0 |
| 12, 13 | 5 | 0 | 0 |
| 14, 15 | 5 | 3 | 0 |
| 16, 17 | 5 | 0 | 4 |
| 18, 19 | 5 | 0 | 4 |
| 20, 21 | 5 | 0 | 4 |
| 22, 23 | 5 | 0 | 5 |
| 24, 25 | 5 | 0 | 1 |
| 26, 27 | 2 | 0 | 1 |
| 28, 29 | 2 | 0 | 1 |
| 30, 31 | 2 | 2 | 1 |
| 32, 33 | 2 | 2 | 1 |
| 34, 35 | 10 | 7 | 6 |
| 36, 37 | 7 | 7 | 6 |
| 38, 39 | 7 | 7 | 6 |
| 40, 41 | 7 | 7 | 6 |
| 42, 43 | 7 | 0 | 6 |
| 44, 45 | 10 | 0 | 10 |
| 46, 47 | 10 | 0 | 10 |
| 48, 49 | 10 | 0 | 9 |
| 50, 51 | 10 | 0 | 9 |
| 52, 53 | 10 | 0 | 9 |
| 54, 55 | 10 | 8 | 9 |
| 56, 57 | 10 | 0 | 9 |
| 58, 59 | 8 | 1 | 8 |
| 60, 61 | 8 | 1 | 8 |
| 62, 63 | 8 | 1 | 8 |
| 64, 65 | 8 | 1 | 8 |
| 66 | 8 | 1 | 8 |

Table 5 – Modified mapping between MIP and intra prediction modes in method 1

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 |
| 2 | 0 | 1 | 1 |
| 3 | 2 | 18 | 1 |
| 4 | 18 | 1 | 18 |
| 5 | 0 | 0 | 0 |
| 6 | 0 | 1 | 1 |
| 7 | 0 | 1 | 0 |
| 8 | 66 | 0 | 1 |
| 9 | 50 | 1 | 50 |
| 10 | 50 | 0 | 0 |

Table 6 –Modified MIP MPM candidate modes in method 1

| MipSizeId | candidate mode x |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 10 | 5 | 1 |
| 1 | 9 | 7 | 1 |
| 2 | 6 | 1 | 8 |

Proposed method 2

In the proposed method 2, the MIP modes of each categories are 6/7/8, and the number of matrix in MIP can be reduced to 3/4/4, as shown in Table 7.

Table 7 – Number of MIP modes and the matrix data with proposed method 2

| MipSizeId(Category) | Block sizes (luma) | Number of MIP modes | Number of 10-bit parameters in MIP matrix |
| --- | --- | --- | --- |
| 0 | 4x4 | 6 | 240 |
| 1 | 4x8, 8x4, 8x8 | 7 | 576 |
| 2 | Other block sizes | 8 | 2304 |

As shown in table 7, the number of 10-bit parameters in MIP matrix is reduced to 49.2%, compared to the VTM5. In the category 0, 6 MIP modes are generated using the selected 3 matrices ( 5th, 9th, and 17th matrix in VTM5 ). In the category 1, 7 MIP modes are generated using the selected 4 matrices ( 0th, 1st, 3rd, and 8th  matrix in VTM5 ). In the category 2, 8 MIP modes are generated using the selected 4 matrices ( 1st, 2nd, 3rd, and 5th matrix in VTM5 ). The reduced MIP modes are coded with the truncated binary code.

Table 8 shows the modified MIP modes between VTM5 and method 2.

Table 8 – Modified MIP mode between VTM5 and proposed method 2

| Modified MIP modes in MipSizeId 0,1,2 | MIP modes in VTM5 |  |  |
| --- | --- | --- | --- |
|  | MipSizeId 0 | MipSizeId 1 | MipSizeId 2 |
| 0 | 5 | 0 | 1 |
| 1 | 9 | 1 | 2 |
| 2 | 17 | 3 | 3 |
| 3 | 22 | 8 | 5 |
| 4 | 26 | 10 | 6 |
| 5 | 34 | 12 | 7 |
| 6 |  | 17 | 8 |
| 7 |  |  | 10 |

In the proposed method 2, the default MIP MPM table and the mode mapping table between MIP and intra mode are also modified as shown in Table 9, 10 and 11.

Table 9 – Modified mapping between intra prediction and MIP modes for method 2

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 2 | 0 | 3 |
| 1 | 2 | 0 | 0 |
| 2, 3 | 2 | 4 | 2 |
| 4, 5 | 1 | 4 | 2 |
| 6,7 | 1 | 4 | 2 |
| 8, 9 | 1 | 4 | 2 |
| 10, 11 | 1 | 4 | 4 |
| 12, 13 | 2 | 0 | 4 |
| 14, 15 | 2 | 0 | 4 |
| 16, 17 | 2 | 0 | 4 |
| 18, 19 | 2 | 0 | 4 |
| 20, 21 | 2 | 0 | 4 |
| 22, 23 | 2 | 0 | 3 |
| 24, 25 | 2 | 0 | 0 |
| 26, 27 | 0 | 0 | 0 |
| 28, 29 | 0 | 0 | 0 |
| 30, 31 | 0 | 2 | 0 |
| 32, 33 | 0 | 2 | 0 |
| 34, 35 | 5 | 5 | 4 |
| 36, 37 | 3 | 5 | 4 |
| 38, 39 | 3 | 5 | 4 |
| 40, 41 | 3 | 5 | 4 |
| 42, 43 | 3 | 0 | 4 |
| 44, 45 | 5 | 0 | 7 |
| 46, 47 | 5 | 0 | 7 |
| 48, 49 | 5 | 0 | 4 |
| 50, 51 | 5 | 0 | 4 |
| 52, 53 | 5 | 0 | 4 |
| 54, 55 | 5 | 0 | 4 |
| 56, 57 | 5 | 0 | 4 |
| 58, 59 | 4 | 1 | 6 |
| 60, 61 | 4 | 1 | 6 |
| 62, 63 | 4 | 1 | 6 |
| 64, 65 | 4 | 1 | 6 |
| 66 | 4 | 1 | 6 |

Table 10 – Modified mapping between MIP and intra prediction modes in method 2

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 0 | 0 | 1 |
| 1 | 2 | 1 | 1 |
| 2 | 0 | 1 | 1 |
| 3 | 0 | 1 | 0 |
| 4 | 66 | 1 | 1 |
| 5 | 50 | 1 | 0 |
| 6 |  | 1 | 1 |
| 7 |  |  | 0 |

Table 11 –Modified MIP MPM candidate modes in method 2

| MipSizeId | candidate mode x |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 0 | 2 | 3 |
| 1 | 0 | 5 | 2 |
| 2 | 4 | 0 | 6 |

Experimental results

The proposed method has been implemented using VTM5.0 as software base according to the common test conditions defined in JVET-N1010 [1] without any encoder optimization.

Test1 (Method 1)

In the test 1, the MIP modes of each categories are 11/11/11, and the number of matrix in MIP can be reduced to 6/6/6. Table 12 and Table 13 show the test results in AI, RA configuration.

Table 12 Experimental results of test1 for all-intra (AI) test condition; anchor is VTM5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | -0.03% | 99% | 101% |
| Class A2 | 0.00% | -0.05% | -0.06% | 100% | 101% |
| Class B | 0.01% | -0.04% | 0.01% | 99% | 101% |
| Class C | 0.04% | -0.03% | 0.05% | 99% | 100% |
| Class E | 0.01% | 0.04% | 0.00% | 98% | 100% |
| Overall | 0.01% | -0.03% | 0.00% | 99% | 101% |
| Class D | 0.07% | 0.02% | -0.14% | 99% | 100% |
| Class F | 0.00% | 0.03% | 0.04% | 99% | 99% |

Table 13 Experimental results of test1 for random-access (RA) test condition; anchor is VTM5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | 0.08% | 100% | 100% |
| Class A2 | 0.03% | -0.03% | 0.03% | 100% | 101% |
| Class B | 0.03% | 0.02% | -0.14% | 100% | 100% |
| Class C | -0.04% | -0.16% | -0.09% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.06% | -0.05% | 100% | 100% |
| Class D | 0.03% | 0.23% | -0.16% | 100% | 101% |
| Class F | -0.02% | -0.04% | 0.04% | 100% | 100% |

Test2 (Method 2)

In the test 2, the MIP modes of each categories are 6/7/8, and the number of matrix in MIP can be reduced to 3/4/4. Table 14 and Table 15 show the test results in AI, RA configuration

Table 14 Experimental results of test2 for all-intra (AI) test condition; anchor is VTM5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | -0.07% | 98% | 102% |
| Class A2 | 0.05% | -0.01% | -0.06% | 97% | 100% |
| Class B | 0.03% | -0.07% | -0.01% | 96% | 100% |
| Class C | 0.07% | -0.05% | 0.01% | 96% | 98% |
| Class E | 0.07% | 0.04% | -0.05% | 96% | 99% |
| Overall | 0.04% | -0.03% | -0.03% | 97% | 100% |
| Class D | 0.11% | 0.18% | -0.14% | 96% | 94% |
| Class F | 0.04% | 0.04% | 0.07% | 98% | 99% |

Table 15 Experimental results of test2 for random-access (RA) test condition; anchor is VTM5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.16% | 0.00% | 100% | 100% |
| Class A2 | 0.05% | 0.13% | 0.06% | 100% | 100% |
| Class B | 0.04% | -0.13% | -0.02% | 100% | 100% |
| Class C | 0.03% | -0.21% | -0.25% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.10% | -0.06% | 100% | 100% |
| Class D | 0.13% | -0.03% | -0.04% | 100% | 101% |
| Class F | 0.04% | 0.07% | 0.04% | 99% | 101% |

Conclusion

To reduce the memory of matrix data used in MIP, modified MIP modes are proposed. In the proposed methods, the memory of MIP matrix is reduced efficiently without any noticeable coding performance changes. It is recommended to adopt the proposed method to be included in the next VTM.

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th meeting, Geneva, CH, 19-27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).