| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0398 |
| --- | --- |

| Title: | Chroma block size restriction in dual tree intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to restrict chroma 2xN block in dual tree of intra coding. With this restriction, the width of chroma block becomes always bigger than or equal to 4, and thus 2xN pixel process of chroma coding is removed in the dual tree block structure. This restriction can help hardware implementation in terms of pipeline management and latency.

Experimental results show 0.02%, 0.34%, and 0.38% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration.

Introduction

In the current intra coding design of VVC, it is possible to use the different block structure between the luma and chroma coding tree when dual tree is on. This method brings good coding gain, but causes the pipeline delay and requires 2xN pixel process in the hardware architecture.

Proposed method

In this proposal, 2xN chroma block restriction is applied in the dual tree of intra coding to remove the 2xN pixel process. To restrict the 2xN chroma block, the additional block split condition is added as below:

The variable allowBtSplit is derived as follows: - If one or more of the following conditions are true, allowBtSplit is set equal to FALSE: - cbSize is less than or equal to MinBtSizeY - cbWidth is greater than maxBtSize - cbHeight is greater than maxBtSize - mttDepth is greater than or equal to maxMttDepth - treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) * ( cbHeight / SubHeightC ) is less than or equal to 16 - Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE - btSplit is equal to SPLIT_BT_VER - treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) is less than or equal to 4 - Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE - btSplit is equal to SPLIT_BT_VER - y0 + cbHeight is greater than pic_height_in_luma_samples …

The variable allowTtSplit is derived as follows: - If one or more of the following conditions are true, allowTtSplit is set equal to FALSE: - cbSize is less than or equal to 2 * MinTtSizeY - cbWidth is greater than Min( MaxTbSizeY, maxTtSize ) - cbHeight is greater than Min( MaxTbSizeY, maxTtSize ) - mttDepth is greater than or equal to maxMttDepth - x0 + cbWidth is greater than pic_width_in_luma_samples - y0 + cbHeight is greater than pic_height_in_luma_samples - treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) * ( cbHeight / SubHeightC ) is less than or equal to 32 - Otherwise, if all of the following conditions are true, allowTtSplit is set equal to FALSE - ttSplit is equal to SPLIT_TT_VER - treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) is less than or equal to 8 – Otherwise, allowTtSplit is set equal to TRUE.

Experimental results

The proposed method has been implemented using VTM5.0 as software base according to the common test conditions defined in JVET-N1010[1].

Table 1 – Table 3 show the test results in AI, RA and LD configuration.

Table 1 Experimental results of proposed method for all-intra (AI) test condition; anchor is VTM5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.23% | 0.09% | 100% | 98% |
| Class A2 | 0.07% | 0.43% | 0.44% | 100% | 96% |
| Class B | 0.00% | 0.31% | 0.52% | 100% | 96% |
| Class C | 0.03% | 0.72% | 0.87% | 99% | 91% |
| Class E | -0.01% | -0.11% | -0.28% | 101% | 96% |
| Overall | 0.02% | 0.34% | 0.38% | 100% | 95% |
| Class D | 0.07% | 1.12% | 0.99% | 99% | 93% |
| Class F | 0.07% | 0.44% | 0.55% | 100% | 95% |

Table 2 Experimental results of proposed method for random-access (RA) test condition; anchor is VTM5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.04% | 100% | 99% |
| Class A2 | 0.06% | 0.55% | 0.44% | 99% | 99% |
| Class B | 0.02% | 0.33% | 0.31% | 100% | 100% |
| Class C | 0.01% | 0.18% | 0.27% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.27% | 0.27% | 100% | 99% |
| Class D | -0.01% | 0.58% | 0.69% | 100% | 99% |
| Class F | -0.01% | 0.47% | 0.39% | 99% | 99% |

Table 3 Experimental results of proposed method for low-delay (LD) test condition; anchor is VTM5

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.18% | 0.19% | 101% | 97% |
| Class C | 0.00% | 0.36% | 0.28% | 103% | 97% |
| Class E | 0.12% | -0.16% | 0.15% | 111% | 110% |
| Overall | 0.04% | 0.15% | 0.21% | 104% | 100% |
| Class D | 0.04% | 0.00% | 1.22% | 102% | 100% |
| Class F | 0.02% | 0.12% | -0.18% | 108% | 105% |

Conclusion

To remove the 2xN pixel process in the dual tree intra coding, 2xN chroma block split is restricted. It is recommended to adopt the proposed method to be included in the next VTM.

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th meeting, Geneva, CH, 19-27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).