| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0399 |
| --- | --- |

| Title: | Non-CE3: CCLM block size restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to restrict CCLM prediction for small chroma blocks so that worst complexity issues regarding data throughput and latency of chroma intra coding are reduced. The proposed restrictions are as follows:

- Method 1: CCLM restriction if the pixel number of chroma block is less than or equal to 16.

- Method 2: CCLM restriction in 2xN and Nx2 chroma block.

Experimental results for Method 1 show 0.03%, 0.53%, and 0.60% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration, and Method 2 show 0.04%, 0.50%, and 0.52% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration.

Introduction

In the current intra coding design of VVC, separate luma chroma partitioning and CCLM prediction introduces a long latency for chroma intra prediction. In the worst case, when all the chroma block is splitted by 4x4 with the CCLM mode, cycle budget is too tight for decoding that blocks and this latency make chroma decoding process worse.

Proposed method

To reduce the data throughput and latency of chroma intra coding, CCLM is restricted in some chroma blocks as below:

- Test 1: CCLM restriction if the pixel number of chroma block is less than or equal to 16.

- Test 2: CCLM restriction in 2xN and Nx2 chroma block.

- Test 3: Test 1 + Test 2

These restriction can be applied both single and dual tree of chroma intra coding.

Experimental results

The proposed method has been implemented using VTM5.0 as software base according to the common test conditions defined in JVET-N1010 [1].

Test1 (Method 1)

In the test 2, CCLM restriction is applied if the pixel number of chroma block is less than or equal to 16.

Table 1 and Table 2 show the test results in AI, RA configuration.

Table 1 Experimental results of test1 for all-intra (AI) test condition; anchor is VTM5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.41% | 0.36% | 100% | 99% |
| Class A2 | 0.06% | 0.45% | 0.37% | 101% | 99% |
| Class B | 0.00% | 0.37% | 0.51% | 100% | 98% |
| Class C | 0.05% | 1.26% | 1.45% | 100% | 90% |
| Class E | 0.00% | 0.00% | 0.07% | 101% | 97% |
| Overall | 0.03% | 0.53% | 0.60% | 101% | 96% |
| Class D | 0.05% | 1.71% | 1.61% | 101% | 94% |
| Class F | 0.09% | 1.09% | 1.24% | 101% | 95% |

Table 2 Experimental results of test1 for random-access (RA) test condition; anchor is VTM5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 0.08% | 0.84% | 99% | 98% |
| Class A2 | 0.04% | 0.80% | 0.46% | 98% | 97% |
| Class B | 0.02% | 0.84% | 1.29% | 99% | 99% |
| Class C | 0.01% | 1.99% | 2.39% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.99% | 1.32% | 99% | 99% |
| Class D | 0.09% | 2.95% | 2.70% | 99% | 99% |
| Class F | 0.09% | 1.74% | 1.70% | 99% | 99% |

Test2 (Method 2)

In the test 2, CCLM restriction is applied in 2xN and Nx2 chroma block. Table 3 and Table 4 show the test results in AI, RA configuration

Table 3 Experimental results of test2 for all-intra (AI) test condition; anchor is VTM5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.39% | 0.32% | 100% | 99% |
| Class A2 | 0.06% | 0.52% | 0.38% | 101% | 98% |
| Class B | 0.01% | 0.41% | 0.51% | 100% | 98% |
| Class C | 0.06% | 0.99% | 1.10% | 101% | 92% |
| Class E | 0.00% | 0.11% | 0.11% | 101% | 96% |
| Overall | 0.04% | 0.50% | 0.52% | 101% | 97% |
| Class D | 0.05% | 1.24% | 1.17% | 100% | 93% |
| Class F | 0.13% | 1.14% | 1.30% | 101% | 94% |

Table 4 Experimental results of test2 for random-access (RA) test condition; anchor is VTM5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.28% | 0.64% | 99% | 98% |
| Class A2 | 0.04% | 0.74% | 0.55% | 98% | 97% |
| Class B | 0.02% | 0.73% | 0.92% | 99% | 99% |
| Class C | 0.00% | 1.58% | 1.59% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.87% | 0.97% | 99% | 99% |
| Class D | 0.06% | 2.21% | 2.32% | 99% | 98% |
| Class F | 0.14% | 1.63% | 1.53% | 99% | 98% |

Test3 (Method 1 + Method 2)

In the test 3, CCLM restriction is applied if the pixel number of chroma block is less than or equal to 16, and also applied in 2xN and Nx2 chroma block. Table 5 and Table 6 show the test results in AI, RA configuration

Table 5 Experimental results of test3 for all-intra (AI) test condition; anchor is VTM5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.55% | 0.50% | 100% | 98% |
| Class A2 | 0.08% | 0.67% | 0.57% | 100% | 95% |
| Class B | 0.01% | 0.60% | 0.81% | 100% | 96% |
| Class C | 0.07% | 1.59% | 1.82% | 100% | 92% |
| Class E | 0.00% | 0.14% | 0.11% | 101% | 96% |
| Overall | 0.05% | 0.75% | 0.83% | 100% | 95% |
| Class D | 0.09% | 1.98% | 1.91% | 100% | 94% |
| Class F | 0.16% | 1.65% | 1.96% | 100% | 94% |

Table 6 Experimental results of test3 for random-access (RA) test condition; anchor is VTM5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.25% | 0.34% | 0.90% | 99% | 98% |
| Class A2 | 0.08% | 1.10% | 0.72% | 98% | 98% |
| Class B | 0.01% | 1.29% | 1.55% | 99% | 100% |
| Class C | 0.03% | 2.54% | 2.78% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 1.39% | 1.58% | 99% | 99% |
| Class D | 0.07% | 3.07% | 2.84% | 100% | 100% |
| Class F | 0.17% | 2.01% | 2.28% | 100% | 100% |

Conclusion

To reduce the data throughput and latency of chroma intra coding, this contribution proposes to restrict CCLM prediction in small chroma block. It is recommended to adopt the proposed method to be included in the next VTM.

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th meeting, Geneva, CH, 19-27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).