| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0400 |
| --- | --- |

| Title: | CE7-related: Context-coded bin restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungah Choi Sunmi Yoo Jin Heo Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jungah.choi, sunmi.yoo, jin78.heo, jangwon84.choi, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

- Abstract

This contribution proposes to further restrict the total number of context-coded bins for coefficient coding using the intra sub-partitions (ISP) mode. The restriction for the ISP mode is set to number of coefficients in a certain unit (coefficient group (CG) or transform unit (TU)) times the limit of the context-coded bin count (1 bin/coefficient). The experimental results show X.XX% AI, X.XX% RA, X.XX% LB BD-Rate versus the VTM-5.0 anchor under common test conditions.

Introduction

VVC [1] imposes limits on the number of context-coded bins at coefficient group (CG) level with 2 bins/coefficient, i.e. 32 for a 4×4 coefficient group (CG) and 8 for a 2×2 CG. In spite of this effort, it was reported that the bin to bit ratio of VTM 4.0 is double digits percentage higher than that of HM16.19 [2]. Moreover, in the low QP case (QP = 2, 7, 12, 17), VTM4.0 consumes more context-coded bins for the same PSNR value than HM16.19. Since large percentage of context-coded bins comes from the transform coefficients, it is important to investigate an appropriate context-coded bin restriction method for transform coefficient coding.

Instead of imposing a fixed limit for each CG, transform unit (TU) level restriction methods for total number of context-coded bins [3] are proposed in the 14th JVET meeting. However, both VTM 5.0 and TU level restriction method only use number of coefficients in a certain unit and color component. There is still room to further restrict the number of context-coded bins using given information. The proposed method introduces a context-coded bin restriction on transform coefficient coding using the intra sub-partitions (ISP) mode.

Proposed method

The ISP mode divides a luma intra coding block vertically or horizontally into two or four sub-partitions depending on the coding unit (CU) size, where all sub-partitions have at least 16 samples and share one luma intra prediction mode. For each sub-partition, residual signals are generated using the prediction signals according to the luma intra prediction mode. After transform, quantization, inverse quantization, and inverse transform, the reconstructed samples of each sub-partition can be used to generate the prediction of the next sub-partition, which will repeat the process and so on.

As the distance from reference pixels gets shorter, the amount of residual signals within the sub-partition, i.e. TU of the ISP mode becomes much smaller than other TUs. Thus, current limit for context-coded bins is too large for the ISP mode. In order to find the appropriate transition point between context coding and bypass coding, the proposed method further restricts the total number of context-coded bins for the ISP mode.

For the ISP mode, we set the maximum allowed number of context-coded bin for one coefficient to 1 (1 bin/coefficient), considering the trade-off of performance and throughput improvement. The proposed method is implemented on top of the TU-level context-coded bin restriction as well as the CG-level context-coded bin restriction. In order to understand the performance of the proposed method clearly, bin to bit ratios are also checked.

Bin statistic analysis

CG-level constraints (on top of VTM 5.0)

The proposed method has been implemented on the current CG-level context coded bin restriction scheme in VTM 5.0.

TU-level constraints (on top of CE7-1.2b)

The proposed method has been implemented on top of CE7-1.2b[3].

Experimental results

CG-level constraints (on top of VTM 5.0)

The proposed method has been implemented on the current CG-level context coded bin restriction scheme in VTM 5.0.

Table 19. Performance in AI

|  | CTC QP (QP = 22, 27, 32, 37) |  |  |  |  | Low QP (QP = 2, 7, 12, 17) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.01% | 0.05% | 101% | 101% | 0.04% | 0.04% | 0.08% | 101% | 102% |
| Class A2 | 0.02% | 0.02% | 0.00% | 101% | 102% | 0.02% | 0.02% | 0.02% | 101% | 100% |
| Class B | 0.04% | -0.03% | 0.11% | 101% | 102% | 0.02% | 0.02% | 0.02% | 101% | 101% |
| Class C | 0.06% | 0.15% | 0.03% | 101% | 100% | 0.02% | 0.08% | 0.08% | 100% | 101% |
| Class E | 0.06% | 0.03% | 0.02% | 101% | 100% | 0.05% | 0.13% | 0.10% | 101% | 100% |
| Overall | 0.04% | 0.04% | 0.05% | 101% | 101% | 0.03% | 0.05% | 0.06% | 101% | 101% |
| Class D | 0.06% | 0.33% | 0.11% | 101% | 99% | 0.02% | 0.04% | 0.03% | 101% | 100% |
| Class F | 0.04% | 0.01% | 0.02% | 101% | 100% | 0.02% | 0.09% | 0.06% | 101% | 100% |

Table 20. Performance in RA

|  | CTC QP (QP = 22, 27, 32, 37) |  |  |  |  | Low QP (QP = 2, 7, 12, 17) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 21. Performance in LB

|  | CTC QP (QP = 22, 27, 32, 37) |  |  |  |  | Low QP (QP = 2, 7, 12, 17) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

TU-level constraints (on top of CE7-1.2b)

The proposed method has been implemented on top of CE7-1.2b[3].

Table 22. Performance in AI

|  | CTC QP (QP = 22, 27, 32, 37) |  |  |  |  | Low QP (QP = 2, 7, 12, 17) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | -0.06% | 101% | 100% | -0.02% | -0.11% | -0.06% | 100% | 99% |
| Class A2 | -0.01% | -0.02% | -0.01% | 100% | 99% | -0.01% | 0.00% | 0.01% | 100% | 97% |
| Class B | 0.00% | -0.03% | 0.00% | 100% | 100% | 0.00% | -0.11% | -0.09% | 100% | 99% |
| Class C | 0.03% | 0.03% | 0.15% | 100% | 99% | 0.11% | 0.06% | 0.07% | 99% | 98% |
| Class E | 0.04% | 0.00% | -0.05% | 100% | 100% | 0.05% | 0.03% | 0.07% | 100% | 98% |
| Overall | 0.02% | -0.01% | 0.01% | 100% | 100% | 0.03% | -0.03% | -0.01% | 100% | 98% |
| Class D | 0.00% | 0.28% | 0.11% | 100% | 99% | 0.12% | 0.05% | 0.10% | 100% | 97% |
| Class F | 0.03% | 0.14% | -0.10% | 100% | 98% | 0.12% | 0.06% | 0.03% | 100% | 99% |

Table 23. Performance in RA

|  | CTC QP (QP = 22, 27, 32, 37) |  |  |  |  | Low QP (QP = 2, 7, 12, 17) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 24. Performance in LB

|  | CTC QP (QP = 22, 27, 32, 37) |  |  |  |  | Low QP (QP = 2, 7, 12, 17) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Conclusion

In this contribution, it is proposed to further restrict the number of context-coded bins for the ISP block. Simulation results show that the average luma BD-rates are X.XX%, X.XX%, and X.XX% in AI, RA, and LB configurations, respectively.

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1001, 14th Meeting: Geneva, CH, 19–27 March 2019.

- M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N0049, 14th Meeting: Geneva, CH, 19–27 March 2019.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1027, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).