| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0401_r1 |
| --- | --- |

| Title: | Non-CE3: MIP mode simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, J. Gan Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@canon.com.au jonathan.gan@canon.com.au |
| Source: | Canon Australia Pty Ltd. |  |  |

_____________________________

# Abstract

Matrix intra prediction (MIP) was adopted into VTM-5.0. This contribution proposes several design changes asserted to simplify the design:

- Removal of MIP MPM scheme. MIP mode is instead signalled with a truncated binary code.

- Removal of 4x4 block support

For aspect (a) coding impact for AI is 0.01%, -0.01%, -0.02%, and for aspect (b) coding impact for AI is 0.02%, -0.02%, -0.01% for Y, CB, and Cr channels, respectively. For the combination of aspects (a) and (b) coding impact for AI is 0.02%, 0.01%, 0.02%, for RA is 0.02%, -0.07%, 0.04%, and for LB is 0.03%, 0.12%, 0.20% for Y, CB, and Cr channels, respectively.

# Introduction

Matrix intra prediction (MIP) [1] was adopted into VTM-5.0 with several aspects, certain ones of which are described below:

The MPM scheme of [1] creates a length-three list of MPMs and requires tables for mapping MIP modes to and from regular intra prediction modes (i.e. DC, planar, and the angular modes) for MPM list population.

Blocks are divided into three sets, each of which has a different number of available MIP modes:

Set A: 4x4 has 35 MIP modes.

Set B: 4x8/8x4/8x8 have 19 MIP modes.

Set C: Other sizes have 11 MIP modes.

With three MIP modes as MPMs, the remaining modes for Sets A-C are 32, 16, and 8 (coded with fixed-length codeword).

# Proposal

MIP mode MPM list removal:

- Coding of the MIP mode for a given CU is proposed to use a truncated binary codeword instead of MPM flag and MPM index / remaining mode index.

- The table to map neighbouring regular intra prediction mode to MIP mode for MPM list construction is removed.

- For regular intra predicted blocks, a neighbouring CU coded using MIP mode is considered as unavailable for the purpose of MPM list construction, so the MIP mode to regular intra prediction mode table is removed.

4x4 CU MIP mode removal:

- Coding efficiency benefit from supporting the smallest block size of 4x4 is asserted as insufficient to justify the complexity burden. Thus, it is proposed to remove support for MIP mode from this block size. As a consequence, tables associated with the 35 MIP modes (18 tables of matrix coefficients and bias values) are removed.

# Coding performance

Table 1 shows the coding performance of the MPM list removal aspect, under JVET CTC [2]. Table 2 shows results of removing 4x4 blocks and Table 3 shows the combined results.

Table 1. MPM list removal coding performance.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | 0.08% | 92% | 93% |
| Class A2 | 0.02% | -0.07% | -0.04% | 96% | 97% |
| Class B | 0.01% | -0.01% | 0.00% | 95% | 96% |
| Class C | 0.00% | -0.01% | 0.06% | 106% | 103% |
| Class E | 0.02% | 0.00% | 0.01% | 94% | 96% |
| Overall | 0.01% | -0.01% | 0.02% | 97% | 97% |
| Class D | -0.01% | 0.15% | -0.03% | 94% | 98% |
| Class F | 0.01% | 0.00% | 0.07% | 99% | 97% |

Table 2. Removal of MIP mode support for small blocks (4x4).

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.01% | 99% | 98% |
| Class A2 | 0.00% | 0.01% | -0.01% | 97% | 97% |
| Class B | 0.01% | 0.00% | 0.04% | 98% | 99% |
| Class C | 0.06% | 0.02% | -0.01% | 107% | 107% |
| Class E | 0.03% | -0.10% | -0.08% | 99% | 102% |
| Overall | 0.02% | -0.02% | -0.01% | 100% | 101% |
| Class D | 0.11% | 0.25% | 0.00% | 95% | 99% |
| Class F | 0.04% | 0.04% | 0.21% | 97% | 97% |

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0_parallel |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | -0.02% | 98% | 97% |
| Class A2 | 0.00% | 0.01% | -0.06% | 100% | 96% |
| Class B | 0.01% | -0.04% | 0.01% | 93% | 96% |
| Class C | 0.07% | 0.01% | 0.09% | 92% | 91% |
| Class E | 0.00% | 0.11% | 0.04% | 94% | 94% |
| Overall | 0.02% | 0.01% | 0.02% | 95% | 95% |
| Class D | 0.14% | 0.17% | 0.00% | 101% | 98% |
| Class F | 0.05% | 0.04% | 0.14% | 91% | 92% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.12% | 96% | 94% |
| Class A2 | 0.01% | -0.02% | -0.02% | 96% | 96% |
| Class B | 0.05% | -0.14% | -0.18% | 98% | 98% |
| Class C | 0.01% | -0.02% | 0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.07% | -0.04% | 98% | 98% |
| Class D | 0.09% | -0.17% | 0.00% | 95% | 96% |
| Class F | 0.01% | -0.08% | 0.09% | 93% | 97% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.29% | 0.00% | 94% | 97% |
| Class C | 0.04% | 0.10% | 0.04% | 97% | 98% |
| Class E | 0.01% | -0.12% | 0.74% | 97% | 97% |
| Overall | 0.03% | 0.12% | 0.20% | 96% | 97% |
| Class D | 0.06% | 0.04% | 0.43% | 101% | 100% |
| Class F | -0.04% | -0.22% | 0.54% | 97% | 97% |

Table 3. Results of aspects a and b combined.

# Conclusion

It is recommended to adopt the proposed changes into VTM-6.0 to achieve a complexity reduction.

# References

- J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, P. Helle, T. Hinz, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, Geneva, CH, March, 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March, 2019.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).