| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0404 |
| --- | --- |

| Title: | Non-CE3: Linear Model-based Intra Prediction (LMIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ramin Ghaznavi-Youvalari | Tel: Email: | ramin.ghaznavi-youvalari@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes Linear Model-based Intra Prediction (LMIP) method for improving the intra prediction in VTM-5.0. The proposed LMIP method predicts the samples inside the block based on a linear model. The parameters of the linear model is derived based on the neighboring samples and their locations by using a linear regression with mean square error method. The LMIP mode is used as a separate mode in VTM-5.0.

The LMIP tool provides on average of -0.xx%, -0.yy%, and -0.zz% BD-Rate improvements in AI, RA and LDB configurations, respectively.

# Introduction

This contribution proposes the Linear Model-based Intra Prediction (LMIP) method for improving the intra prediction in VTM-5.0. The LMIP tool models the samples behaviour inside the block based on a linear model derived from the neighboring samples.

# Proposed Method

The LMIP tool attempts in modelling the samples behaviour inside a block based on the neighboring samples. For that, a linear model is used for predicting the sample values inside the block as below:

Pred (x, y) = a * x + b* y + c                       (1)

The equation (1) predicts each pixel’s value in position (x, y) inside the block by using their locations and prediction parameters. The parameters a, b and c in the prediction model is estimated based on the neighboring samples values’ and locations. For that, the neighboring samples and their (x, y) locations are used as input parameters to a linear regression method with mean square error (MSE) minimization approach.

Figure 1 shows an example of the samples that are used for LMIP parameter derivation:

Figure 1. Neighboring samples used for LMIP parameter derivation

For the blocks where one side is in the picture boundary, only the samples from non-picture boundary side is used for deriving the parameters model parameters. In such cases, the prediction model (1) will be changed to equation (2) or (3) depending on which side of the block is in picture boundary:

Pred (x, y) = a * x + c                       (2)

Pred (x, y) = b * y + c                       (3)

Further remarks:

- The LMIP mode is disabled for the Low Frequency Non-Separable Transforms (LFNST).

- LMIP only uses MRL equal to zero cases

- A CU level flag is signalled for this mode after ISP flag.

# Simulation Results

The proposed LMIP method is implemented on top of VTM-5.0 and the simulations were performed under common test conditions as described in JVET-N1010 [1].

The tables below summarize impacts of the proposed method compared to VTM-5.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.01% | 0.00% | 106% | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.04% | -0.01% | 0.06% | 105% | #NUM! |
| Class C | -0.03% | -0.02% | -0.01% | 104% | #NUM! |
| Class E | -0.22% | -0.09% | -0.07% | 105% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | 0.25% | -0.02% | 104% | #NUM! |
| Class F | -0.08% | -0.02% | 0.08% | 104% | #NUM! |

# References

- “JVET common test conditions and software reference configurations”, JVET-N1010

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).